3GPP TSG-RAN WG3 Meeting #120	R3-232955

Incheon, Republic of Korea, 22nd – 26th May, 2023

Title: 	Further discussion on long eDRX support for RRC_INACTIVE

Source: 	Huawei

Agenda item:	21.2

Document Type:	Discussion

- Introduction

In RAN3#119bis-e, the following agreements on enhanced eDRX in RRC_INACTIVE are achieved and there are still some open issues.

NGAP agreements: Introduce a new CN-based MT communication handling IE ENUMERATED (Supported,…) in the Core Network Assistance Information for RRC INACTIVE IE in NGAP Introduce a new class 1 MT Communication Handling procedure in NGAP for RAN requesting CN to perform data buffering and for notifying of UE RRC state transition. The procedure contains the following messages: - MT COMMUNICATION HANDLING REQUEST - MT COMMUNICATION HANDLING RESPONSE - MT COMMUNICATION HANDLING FAILURE The MT COMMUNICATION HANDLING REQUEST message contains: - AMF UE NGAP ID (M) - RAN UE NGAP ID (M) - FFS on RRC state (M) as defined in 9.3.1.92 - NR Paging eDRX Cycle for RRC INACTIVE (conditionally present if RRC state is set to ‘inactive’) encoded as ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …). - FFS on any other information (pending on RAN2/SA2 progress or SDT) Introduce a new DL DATA NOTIFICATION class 2 message for AMF requesting RAN Paging Open issues: 1) The addition of the PPI/ARP/5QI/PDU Session ID in the DL MT DATA NOTIFICATION message based on CT4 LS to be discussed next meeting after SA2 spec update. 2) The discussion on support of SDT and long eDRX>10.24 seconds is postponed to next meeting pending any updates from SA2 (e.g. inclusion of MT-SDT Data Size in NG message) and RAN3 progress on the signalling to support the CN Based MT Communication handling in RAN3 specifications. 3) PTW infor over NGAP? 4) FFS how to differentiate in the MT Communication Handling procedure whether the request message is sent to indicate connection resume or to apply MT communication handling. The presence of the NR Paging eDRX Cycle for RRC INACTIVE IE is FFS 5) ASN.1 in F1AP and XnAP: FFS if the NRPaging-time-Window defined in ASN.1 can be re-used instead of a new d NRPaging-Time-Window-Inactive

In this contribution, we provide the potential impact analysis of enhanced eDRX in RRC_INACTIVE on RAN3, considering recent progress on enhanced INACTIVE eDRX in SA2/RAN2.

- Discussion

  - DL Data Notification

A new DL DATA NOTIFICATION class 2 message was introduced for AMF requesting RAN Paging. When there is DL data buffered in 5GC for the UE and the AMF considers the UE is reachable, the AMF sends the DL DATA NOTIFICATION message to the NG-RAN node.

Upon reception of the DL DATA NOTIFICATION message, the NG-RAN node shall perform RAN Paging for the UE in RRC_INACTIVE state. Note that, this is different from legacy behaviour, where in legacy RAN paging, the CN considers the UE as CM_CONNECTED and the NAS signalling or data is delivered to the NG-RAN node directly. Based on 5QI, ARP and PPI associated with an incoming DL PDU, the NG-RAN may enforce specific paging policies. Besides, the NG-RAN determines Paging Priority based on ARP associated with the QoS flow and the Core Network Assisted RAN paging information form AMF.

In a brief summary, the QFI, ARP, 5QI, PPI and PDU Session ID shall be considered in the NG-RAN node, which has been shown in the LS [1].

SA2 thanks CT4 for informing the enhancement related to paging policy differentiation. SA2 would also like to inform CT4 that SA2 agreed CR to improve the description in SA2 spec. SA2 would like to further inform CT4 that SA2 considers the QFI can also be provided by SMF to AMF, and has asked RAN2 and RAN3 to confirm the information being provided to RAN.

In our opinion, with QFI provided, the associated 5QI and ARP information is obtained via existing QoS Flow Level QoS Parameters IE. Thus only PPI/QFI/PDU Session ID are needed in the DL MT DATA NOTIFICATION message.

Proposal 1: The new DL MT DATA NOTIFICATION message includes only PPI/QFI/PDU Session ID for which DL packets are received.

  - MT Communication Handling

A new class 1 MT Communication Handling procedure in NGAP was also introduced in the last meeting. The MT COMMUNICATION HANDLING REQUEST message contains

- AMF UE NGAP ID (M)

- RAN UE NGAP ID (M)

- FFS on RRC state (M) as defined in 9.3.1.92

- NR Paging eDRX Cycle for RRC INACTIVE (conditionally present if RRC state is set to ‘inactive’) encoded as ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …).

- FFS on any other information (pending on RAN2/SA2 progress or SDT)

The FFS on RRC state has a relationship to another remaining issue, which is

FFS how to differentiate in the MT Communication Handling procedure whether the request message is sent to indicate connection resume or to apply MT communication handling. The presence of the NR Paging eDRX Cycle for RRC INACTIVE IE is FFS

According to the incoming LS [2], the MT COMMUNICATION HANDLING REQUEST message sent by NG-RAN requesting connection resume (step 4b.1 in Figure 4.8.2.2-1) has been extended to indicate that the UE is now reachable for downlink data and/or signalling, to allow for the case of small data transmission where the UE is kept in RRC inactive state

4b. [Conditional] N2 Notification, 4b.1 If the accessed NG-RAN is the same as the NG-RAN that configured RRC_INACTIVE and still has the UE context, NG-RAN sends an N2 Notification to the AMF indicating - the UE is in RRC_CONNECTED, if an AMF requested N2 Notification (see clause 4.8.3); or, - the UE is now reachable for downlink data and/or signalling if Connection Inactive procedure with CN based MT communication handling (see clause 4.8.1.1a) has been performed previously.

Thus, MT Communication Handling procedure can be initiated in the following two cases:

- Case 1: NG-RAN node requests CN to perform data buffering and MT communication handling;

- Case 2: NG-RAN node notifies UE being reachable for downlink data and/or signalling, like UE is in RRC_CONNECTED or in RRC_INACTIVE while SDT procedure is ongoing.

Moreover, we think there are other cases that may trigger the MT Communication Handling procedure. Take the RNA update as an example, if t380 expires, the UE configured with Rel-18 enhanced INACTIVE eDRX (e.g., 20.48s) shall initiate the RNA update procedure. In case of RAN update without UE context relocation, the last serving gNB may have the following cases

- Case 3a: release the UE back to RRC_INACTIVE, without any INACTIVE eDRX configuration (e.g., 0.64s), or with only Rel-17 INACTIVE eDRX configuration (e.g., 5.12s). In this case, the last serving gNB shall trigger MT Communication Handling procedure to indicate UE is now reachable for downlink data and/or signalling.

- Case 3b: release the UE back to RRC_INACTIVE, with updated Rel-18 enhanced INACTIVE eDRX configuration, for example, the INACTIVE eDRX cycle is extended from 20.48s to 40.96s. In this case, the last serving gNB shall trigger MT Communication Handling procedure to apply MT communication handling with updated Rel-18 enhanced INACTIVE eDRX configuration.

- Case 3c: move the UE to RRC_IDLE. In this case, the last serving gNB shall also trigger MT Communication Handling procedure to cancel MT communication handling. And the UE is not reachable for downlink data and/or signalling since the UE is in RRC_IDLE.

Besides the RNA update scenario without UE context relocation, modification of UE INACTIVE eDRX configuration may also happen in scenario wherein

- Case 4: RNA update with UE context relocation, wherein the UE shall be in RRC_INACTIVE or RRC_CONNECTED, or

- Case 5: UE attempts to resume a connection from RRC_INACTIVE but is rejected by the NG-RAN and keeps in RRC_INACTIVE, or

- Case 6: UE resumes from RRC_INACTIVE and is responded with Release with Redirect (with UE Context relocation) as defined in TS 38.300.

Observation 1:  Modification of UE INACTIVE eDRX configuration may trigger the MT Communication Handling procedure.

Considering the UE may be in different RRC state (in Case 2/4, the UE is in RRC_CONNECTED; In Case 1/2/3a/3b/4/5/6, the UE is in RRC_ INACTIVE; In Case 3c, the UE is in RRC_ IDLE), we think it is necessary to include the RRC state in MT COMMUNICATION HANDLING REQUEST message.

Moreover, the NR Paging eDRX Cycle for RRC INACTIVE IE is optional in the MT COMMUNICATION HANDLING REQUEST message. Specially, in case 1 and 3b, the NR Paging eDRX Cycle for RRC INACTIVE IE shall be present to apply MT communication handling; In case 2 and 3a, the NR Paging eDRX Cycle for RRC INACTIVE IE is not necessary.

Proposal 2: There is necessity to include the RRC state in MT COMMUNICATION HANDLING REQUEST message.

Proposal 3: The NR Paging eDRX Cycle for RRC INACTIVE IE is optional in the MT COMMUNICATION HANDLING REQUEST message.

  - Support of SDT and long eDRX>10.24 seconds

Enhanced INACTIVE eDRX and MT SDT:

As agreed in last RAN3 meeting, the MT-SDT Data Size IE is mandatory in XnAP and E1AP for MT-SDT. Considering support MT-SDT and enhanced eDRX in RRC_INACTIVE, the MT-SDT Data Size IE should also be included in the new DL MT DATA NOTIFICATION message.

Proposal 4: The MT-SDT Data Size IE should be included in the new DL MT DATA NOTIFICATION message.

Enhanced INACTIVE eDRX and MO SDT:

As a response to the RAN paging with MT SDT indication, or in case of MO SDT, the UE may resume and perform RA-SDT or CG-SDT. Here we take the scenario wherein enhanced INACTIVE eDRX is with RA-SDT as an example, and the scenario with CG-SDT is similar to that with RA-SDT in aspect of impacts on NG-RAN and CN interaction.

In SDT procedure over RACH, the receiving gNB would identify that the UE request is for an SDT after reception of RRCResumeRequest as well as UL SDT data and/or UL SDT signalling from UE. The receiving gNB shall indicate the UE request is for an SDT together with some SDT assistance information in the RETRIEVE UE CONTEXT REQUEST message to last serving gNB. Based on the provided information, the last serving gNB makes a decision on whether relocate the UE context or not.

If the last serving gNB decides not to relocate the full UE context, the UL SDT data is then delivered from the receiving gNB to the UPF via the last serving gNB. Subsequent UL/DL SDT data and/or signalling are transferred in the same path, i.e., UE - receiving gNB - last serving gNB – UPF/AMF.

If the last serving gNB decides to relocate UE context, the UL SDT data is delivered from the receiving gNB to the UPF. After the Path Switch procedure, subsequent UL/DL SDT data and/or signalling are transferred between UE and UPF/AMF via the receiving gNB.

However, when the UE is configured with Rel-18 enhanced INACTIVE eDRX, the AMF is requested for MT communication handling and SMF/UPF(s) performs data buffering for MT data. It implies that any DL SDT data and/or signalling shall be suspended. However, this makes no sense in case of RA-SDT where the UE is available for SDT data transmission in RRC_INACTIVE state. According to the LS [2] from SA2, for UE in RRC_INACTIVE while SDT procedure is ongoing, the NG-RAN node shall notify the CN for cancelling MT communication handling and data buffering.

SA2 thanks RAN2 for their LS (R2-2302082 / S2-2303947) on INACTIVE eDRX above 10.24sec and SDT. SA2 would like to inform RAN2, and RAN3 that they have agreed the attached CRs (23502CR3944, 23502CR4007, 23501CR4218) which include the following: - Call flows in TS 23.502 have been adjusted so that NG-RAN decides if the UE remains in RRC_INACTIVE or can be transitioned to RRC_CONNECTED during resume procedure. - The N2 message sent by the AMF requesting connection resume (step 2 in 23.502 Figure 4.8.2.2b-1) includes the QFI, 5QI, PPI and ARP of the QoS Flow that triggered the connection resume. - The N2 message sent by NG-RAN requesting connection resume (step 4b.1 in Figure 4.8.2.2-1) has been extended to include an indication that the UE is now reachable for downlink data and/or signalling, to allow for the case of small data transmission where the UE is kept in RRC inactive state. SA2 would like RAN2 and RAN3 to confirm whether the information being provided from core network is the right information i.e. whether it is all required or not.

Thus, the MT Communication Handling procedure can be initiated when the UE is in RRC_INACTIVE while SDT procedure is ongoing. RAN3 should confirm the information provided by SA2. A draft LS out in attached in the annex.

Proposal 5: RAN3 to confirm the MT Communication Handling procedure can be initiated to indicate that UE is now reachable for downlink data and/or signalling when the UE is in RRC_INACTIVE while SDT procedure is ongoing.

Proposal 6: Discuss and agree the LS out to SA2 in the annex.

  - Fallback behaviour

In RAN2#121b, there are following agreements on the fallback behaviour.

Introduce a new IE for INACTIVE eDRX to include the eDRX cycle values larger than 10.24s. UEs configured with Rel-18 enhanced INACTIVE eDRX should apply Rel-18 enhanced INACTIVE eDRX if Rel-18 enhanced INACTIVE eDRX is allowed in the serving cell, regardless of whether Rel-17 INACTIVE eDRX is allowed in the serving cell. UEs configured with Rel-18 enhanced INACTIVE eDRX should apply INACTIVE DRX if both Rel-18 enhanced INACTIVE eDRX and Rel-17 INACTIVE eDRX are not allowed in the serving cell. Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

A Rel-18 UE can be configured with both Rel-17 INACTIVE eDRX and Rel-18 enhanced INACTIVE eDRX. When performs RAN paging, the anchor gNB shall deliver both Rel-17 INACTIVE eDRX configuration and Rel-18 enhanced INACTIVE eDRX configuration to the receiving gNB.

Proposal 7: The anchor gNB delivers both Rel-17 INACTIVE eDRX configuration and Rel-18 enhanced INACTIVE eDRX configuration over XnAP when performs RAN paging for a UE configured with both Rel-17 INACTIVE eDRX and Rel-18 enhanced INACTIVE eDRX.

- Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1:  Modification of UE INACTIVE eDRX configuration may trigger the MT Communication Handling procedure.

Proposal 1: The new DL MT DATA NOTIFICATION message includes only PPI/QFI/PDU Session ID for which DL packets are received.

Proposal 2: There is necessity to include the RRC state in MT COMMUNICATION HANDLING REQUEST message.

Proposal 3: The NR Paging eDRX Cycle for RRC INACTIVE IE is optional in the MT COMMUNICATION HANDLING REQUEST message.

Proposal 4: The MT-SDT Data Size IE should be included in the new DL MT DATA NOTIFICATION message.

Proposal 5: RAN3 to confirm the MT Communication Handling procedure can be initiated to indicate that UE is now reachable for downlink data and/or signalling when the UE is in RRC_INACTIVE while SDT procedure is ongoing.

Proposal 6: Discuss and agree the LS out to SA2 in the annex.

Proposal 7: The anchor gNB delivers both Rel-17 INACTIVE eDRX configuration and Rel-18 enhanced INACTIVE eDRX configuration over XnAP when performs RAN paging for a UE configured with both Rel-17 INACTIVE eDRX and Rel-18 enhanced INACTIVE eDRX.

- Reference

[1] S2-2305617, Reply LS on Paging Policy Information for Network Triggered Connection Resume.

[2] S2-2305619, Reply LS on INACTIVE eDRX above 10.24sec and SDT

# Annex 1. Reply LS out to SA2

3GPP TSG-RAN3 Meeting #120	R3-23xxxx

# Incheon, Korea, 22nd –26th  May, 2023

Title:	[draft] Reply LS on INACTIVE eDRX above 10.24sec and SDT

Response to:	S2-2305619

Release:	Release 18

Work Item:	NR_redcap_enh-Core

Source:	RAN3

To:	SA2, RAN2

Cc:	CT4

Contact person:	Jingcong Sun

Sunjingcong@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 thanks SA2 for the LS in S2-2305619 on supporting INACTIVE eDRX above 10.24sec and SDT. RAN3 would like to provide feedback to the following question.

SA2 would like to inform RAN2, and RAN3 that they have agreed the attached CRs (23502CR3944, 23502CR4007, 23501CR4218) which include the following: - Call flows in TS 23.502 have been adjusted so that NG-RAN decides if the UE remains in RRC_INACTIVE or can be transitioned to RRC_CONNECTED during resume procedure. - The N2 message sent by the AMF requesting connection resume (step 2 in 23.502 Figure 4.8.2.2b-1) includes the QFI, 5QI, PPI and ARP of the QoS Flow that triggered the connection resume. - The N2 message sent by NG-RAN requesting connection resume (step 4b.1 in Figure 4.8.2.2-1) has been extended to include an indication that the UE is now reachable for downlink data and/or signalling, to allow for the case of small data transmission where the UE is kept in RRC inactive state. SA2 would like RAN2 and RAN3 to confirm whether the information being provided from core network is the right information i.e. whether it is all required or not.

RAN3 would like to confirm that the adjustment to call flows in TS 23.502 is useful, so UE can remain in RRC_INACTIVE for SDT transmission. For a similar reason, it is beneficial to extend the N2 message requesting connection resume to also indicate the UE is now reachable for downlink data and/or signalling.

As for the N2 message sent by the AMF requesting connection resume, RAN3 thinks it only needs to include PPI, QFI and PDU Session ID information since the ARP and 5QI can be inferred via existing QoS Flow Level QoS Parameters IE.

# 2	Actions

To SA2, RAN2:

ACTION: 	RAN3 kindly asks SA2 and RAN2 to take the above into account and provide feedback if necessary.

# 3	Dates of next RAN3 meetings

RAN3#121                                                                          2023-08-21 – 2023-08-25     Toulouse, France

RAN3#121-bis                                                                     2023-10-09 – 2023-10-13     Xiamen, China

# Annex 2. Text proposal for TS 38.413

<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| DL Data Notification | DL DATA NOTIFICATION |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.X	MT Communication Handling

#### 8.3.X.1	General

The purpose of the MT Communication Handling procedure is to request the AMF to activate or deactivate the CN based MT Communication handling for a UE in RRC_INACTIVE state with extended DRX beyond 10.24 seconds as specified in TS 23.501 [9]. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.X.2-1: MT Communication Handling procedure. Successful operation.

The NG-RAN node initiates the procedure by sending the MT COMMUNICATION HANDLING REQUEST message to the AMF.

If the NR Paging eDRX Cycle for RRC INACTIVE IE is included in the MT COMMUNICATION HANDLING REQUEST message, the AMF shall, if supported, take it into account when applying MT communication handling as specified in TS 38.304 [12] and TS 23.501 [9].

Editor’s Note: FFS on other IEs that need to be included in the MT Communication Handling procedure (e.g. RAN NR PTW).

#### 8.3.X.3	Unsuccessful Operation

Figure 8.3.X.3-1: MT Communication Handling: unsuccessful operation.

If the AMF is not able to activate CN based mobile terminating communication handling for the UE configured with eDRX cycle value longer than 10.24 seconds in RRC_INACTIVE state, it shall send a MT COMMUNICATION HANDLING FAILURE message to the NG-RAN node.

### 8.3.Y	DL Data Notification

#### 8.3.Y.1	General

This procedure is initiated by the AMF to indicate that there is DL data buffered in 5GC or DL signaling in 5GC for the UE.

The procedure uses UE associated signalling.

#### 8.3.Y.2	Successful Operation

Figure 8.5.2.2-1: DL Data Notification

The AMF initiates the DL Data Notification procedure by sending the DL DATA NOTIFICATION message to the NG-RAN node.

Upon reception of the DL DATA NOTIFICATION message, the NG-RAN node shall perform RAN Paging for the UE in RRC_INACTIVE state.

If the Paging Policy Indicator IE is included for a given QoS flow in the PDU Session Item IE within the DL DATA NOTIFICATION message, the NG-RAN node shall, if supported, use it according to TS 23.501 [9].

If the MT-SDT Information IE is included for a given QoS flow in the PDU Session Item IE within the DL DATA NOTIFICATION message, the NG-RAN node shall, if supported, use it to decide whether to initiate Mobile Terminated SDT procedures towards the UE as described in TS 38.300 [9].

Editor’s Note: FFS on the actual IEs that need to be included in the DL Data Notification procedure.

#### 8.3.Y.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.X1	MT COMMUNICATION HANDLING REQUEST

This message is sent by the NG-RAN node to the AMF to request CN based MT communication handling for UEs in RRC_INACTIVE state with long eDRX beyond 10.24 seconds as specified in TS 23.501 [9].

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NR Paging eDRX Cycle for RRC INACTIVE | O |  | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) |  | YES | reject |
| RRC State Information | M |  | 9.3.1.xx |  | YES | reject |
| FFS |  |  |  |  |  |  |

#### 9.2.2.X2	MT COMMUNICATION HANDLING RESPONSE

This message is sent by the AMF to indicate that CN based MT Communication handling was successfully applied.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| FFS |  |  |  |  |  |  |

#### 9.2.2.X3	MT COMMUNICATION HANDLING FAILURE

This message is sent by the AMF to indicate that CN based MT Communication handling was unsuccessful.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.3.X4	DL DATA NOTIFICATION

This message is sent by the AMF to indicate the there is MT data or signaling in 5GC for the UE.

Direction: AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| PDU Session List |  | 1 |  |  |  |  |
| >PDU Session Item |  | 1..<maxnoofPDUSessions> |  |  |  |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  |  |  |
| >>QoS Flow List |  | 1 |  |  |  |  |
| >>>QoS Flow Item |  | 1..<maxnoofQoSFlows> |  |  |  |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.51 |  |  |  |
| >>>>Paging Policy Indicator | O |  | 9.3.1.xxx | Paging Policy Indicator is specified in TS 23.501 [9] |  |  |
| >>>>MT-SDT Information | O |  | 9.3.1.yyy |  |  |  |
| FFS |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.15	Core Network Assistance Information for RRC INACTIVE

This IE provides assistance information for RRC_INACTIVE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| UE Specific DRX | O |  | Paging DRX 9.3.1.90 |  | - |  |
| Periodic Registration Update Timer | M |  | 9.3.3.24 |  | - |  |
| MICO Mode Indication | O |  | 9.3.1.23 |  | - |  |
| TAI List for RRC Inactive |  | 1 |  |  | - |  |
| >TAI List for RRC Inactive Item |  | 1..<maxnoofTAIforInactive> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Expected UE Behaviour | O |  | 9.3.1.93 |  | - |  |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.3.3.52 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| MICO All PLMN | O |  | 9.3.1.194 |  | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  | YES | ignore |
| Paging Cause Indication for Voice Service | O |  | ENUMERATED (supported, …) | This IE indicates whether the UE supports the feature of indication of paging cause for voice service. | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  | YES | ignore |
| CN MT communication handling | O |  | ENUMERATED (supported, …) | This IE indicates the CN support of CN based MT communication handling | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforInactive | Maximum no. of TAIs for RRC Inactive. Value is 16. |

#### 9.3.1.xx	RRC State Information

This IE indicates the RRC state information of the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| RRC state | M |  | ENUMERATED (Inactive, Connected, Idle …) | Indicates the current RRC state of the UE. |

#### 9.3.1.xxx	Paging Policy Indicator

This IE provides paging policy indicator.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Paging Policy Indicator | M |  | INTEGER (0..7) |  |

#### 9.3.1.yyy	MT-SDT Information

This IE provides MT-SDT information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Data Size | M |  | INTEGER (1…4095, …) | Indicates the data size calculated by SMF/UPF for each QoS Flow. Unit: bit. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>