3GPP TSG-RAN WG3 #120	R3-232968

# Incheon, Korea, 22nd – 26th May 2023

Agenda Item:	24.2

Source:	Ericsson

Title: 	Introduction of Network Energy Saving

Document for:	Discussion and Approval

# 1	Background

The new Work Item NES has been approved, refer to [1].

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

Objective 2 and object 6 have explicit RAN3 objectives, which we should work on the stage 3 specification.

At the last RAN3 meeting, we started the WID and had some discussion on the objectives. This paper continues the discussion.

# 2	Discussion

## 2.1	Impact on XnAP TS 38.423

The below existing procedure could be used to implement inter-node information exchange on cell DTX/DRX:

### 8.4.2 NG-RAN node Configuration Update #### 8.4.2.1 General The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface. NOTE: Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified. The procedure uses non UE-associated signalling. #### 8.4.2.2 Successful Operation Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The cell DTRX information can be carried at cell level.

Proposal 1: RAN3 to discuss and agree that NG-RAN node1 uses “NG-RAN node configuration Update” procedure to update the cell DTRX information to NG-RAN node 2 over XnAP.

In [2], we have further discussed an enhancement on the inter-node coordination, that is the Cell DTRX can be coordinated among the neighbouring nodes:

The “Cell DTRX information Report” in the above is the NG-RAN node configuration Update, and the “Cell DTRX information coordination” is the NG-RAN node configuration Update Acknowledge.

Proposal 2: RAN3 to discuss and agree that to coordinate the cell DTRX information among the NG-RAN nodes over XnAP;

Proposal 3: RAN3 to discuss and agree that NG-RAN node x uses “NG-RAN node configuration Update Acknowledge” procedure to send the coordinated value to NG-RAN node 1 over XnAP.

In some cases, the SSB beam is activated so that the UE could perform measurement. After that, the SSBs can be deactivated again.

Proposal 4: It is proposed that RAN3 to discuss and agree to include a timer in the beam activation information in the Cell Activation request over XnAP and F1AP.

## 2.2	Impact on F1AP TS 38.473

There are two existing procedures over F1AP for gNB-CU and DU to update the configuration.

### 8.2.4 gNB-DU Configuration Update #### 8.2.4.1 General The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling. NOTE: Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified. #### 8.2.4.2 Successful Operation Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

Proposal 5: RAN3 to discuss and agree to include the Cell DTRX information in F1 Setup and gNB-DU over F1AP.

# 3	Proposals

Proposal 1: RAN3 to discuss and agree that NG-RAN node1 uses “NG-RAN node configuration Update” procedure to update the cell DTRX information to NG-RAN node 2 over XnAP

Proposal 2: RAN3 to discuss and agree that to coordinate the cell DTRX information among the NG-RAN nodes over XnAP

Proposal 3: RAN3 to discuss and agree that NG-RAN node x uses “NG-RAN node configuration Update Acknowledge” procedure to send the coordinated value to NG-RAN node 1 over XnAP

Proposal 4: It is proposed that RAN3 to discuss and agree to include a timer in the beam activation information in the Cell Activation request over XnAP and F1AP.

Proposal 5: RAN3 to discuss and agree to include the Cell DTRX information in F1 Setup and gNB-DU over F1AP.

The Text Proposal is submitted in [3].

# 4	Reference

[1]	RP-221443	Study on network energy savings for NR

[2]	R3-226457	Further discussion on SI Network Energy Saving, Ericsson

[3]	R3-232969	Text Proposal on F1AP: Introduction of Network Energy Saving, Ericsson