3GPP RAN WG3 Meeting #120	R3-233063

Incheon, KR, 22nd – 26th May, 2023

Agenda Item:	21.2

Source:	Ericsson

Title:	(TP to NGAP BLCR on Rel-18 RedCap): Support of connection resume indication and introduction of paging policy parameters for RAN Paging

Document for:	Other

# TP to NGAP BL CR

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.X	MT Communication Handling

#### 8.3.X.1	General

The purpose of the MT Communication Handling procedure is to request the AMF to activate or deactivate the CN based MT Communication handling for a UE in RRC_INACTIVE state with extended DRX beyond 10.24 seconds as specified in TS 23.501 [9]. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.X.2-1: MT Communication Handling procedure. Successful operation.

The NG-RAN node initiates the procedure by sending the MT COMMUNICATION HANDLING REQUEST message to the AMF.

If the NR Paging eDRX Cycle for RRC INACTIVE IE is included in the MT COMMUNICATION HANDLING REQUEST message, the AMF shall activate MT communication handling as specified in TS 38.304 [12] and TS 23.501 [9].

If the UE Reachability Indication IE is included in the MT COMMUNICATION HANDLING REQUEST message and set to 'true', the AMF shall deactivate MT communication handling and consider that the UE is reachable for DL Data / DL Signalling delivery.

Editor’s Note: FFS on other IEs that need to be included in the Mobile Terminating Communication procedure (e.g. RAN NR PTW).

#### 8.3.X.3	Unsuccessful Operation

Figure 8.3.X.3-1: MT Communication Handling: unsuccessful operation.

If the AMF is not able to activate CN based mobile terminating communication handling for the UE configured with eDRX cycle value longer than 10.24 seconds in RRC_INACTIVE state, it shall send a MT COMMUNICATION HANDLING FAILURE message to the NG-RAN node.

### 8.3.Y	DL Data Notification

#### 8.3.Y.1	General

This procedure is initiated by the AMF to indicate that there is DL data or DL signaling buffered in 5GC for the UE.

The procedure uses UE associated signalling.

#### 8.3.Y.2	Successful Operation

Figure 8.5.2.2-1: DL Data Notification

The AMF initiates the DL Data Notification procedure by sending the DL DATA NOTIFICATION message to the NG-RAN node.

Upon reception of the DL DATA NOTIFICATION message, the NG-RAN node shall perform RAN Paging for the UE in RRC_INACTIVE state.

If the Paging Policy Differentiation IE is included in the DL DATA NOTIFICATION message, the NG-RAN shall, if supported, take them into account when performing RAN Paging for the UE in RRC_INACTIVE state.

#### 8.3.Y.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.X1	MT COMMUNICATION HANDLING REQUEST

This message is sent by the NG-RAN node to the AMF to request CN based MT communication handling for UEs in RRC_INACTIVE state with long eDRX beyond 10.24 seconds as specified in TS 23.501 [9].

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| CHOICE Communication handling type |  |  |  |  | YES | reject |
| >Connection Inactive procedure |  |  |  |  |  |  |
| >>NR Paging eDRX Cycle for RRC INACTIVE | M |  | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) |  |  |  |
| >Connection Resume procedure |  |  |  |  |  |  |
| >>UE Reachability Indication | M |  | ENUMERATED (true, …) | Indicates that the UE is reachable for downlink data and/or signalling |  |  |
| DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. |  |  |

#### 9.2.2.X2	MT COMMUNICATION HANDLING RESPONSE

This message is sent by the AMF to indicate that CN based MT Communication handling was successfully applied.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| FFS |  |  |  |  |  |  |

#### 9.2.2.X3	MT COMMUNICATION HANDLING FAILURE

This message is sent by the AMF to indicate that CN based MT Communication handling was unsuccessful.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.3.X4	DL DATA NOTIFICATION

This message is sent by the AMF to indicate the there is MT data or signaling buffered in 5GC for the UE.

Direction: AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Paging Policy Differentiation | O |  | 9.3.3.X |  | YES | reject |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.X	Paging Policy Differentiation

This IE provides paging policy differentiation information for a UE in RRC_INACTIVE state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Priority Indicator (PPI) | M |  | INTEGER (0.. 7, …) | The Paging Policy Indicator is used for paging policy differentiation (see details in TS 23.501 [9]). |
| Allocation and Retention Priority | M |  | 9.3.1.19 |  |
| 5QI | M |  | INTEGER (0..255, …) | indicate the 5QI associated with the PDU session |
| QoS Flow Identifier | M |  | 9.3.1.51 |  |
| PDU Session ID | M |  | INTEGER (0..255) | This IE identifies a PDU Session for a UE. The definition and use of the PDU Session ID is specified in TS 23.501 |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>