3GPP TSG-RAN3 Meeting #120	R3-233115

Incheon, South Korea, May 22nd – 26th 2023

Agenda Item:	12.3

Source:	Ericsson, Deutsche Telekom, InterDigital, AT&T

Title:	Continuous MDT tracing

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#119bis online meeting, MDT enhancements have been discussed which resulted in the following items being captured in the chair’s notes:

The existing MDT framework is used as baseline for data collection from the UE.

FFS on using s-based MDT, m-based MDT or both of them for data collection from the UE.

Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

FFS on determining the need for continuous MDT support for the Rel.18 use cases.

FFS on the solutions to support continuous MDT collection:

- Solution 1: existing MDT mechanism without enhancement.

- Solution 2: Introduce an indication in the logged MDT measurement configuration to indicate UE whether to collect continuous location information (e.g., historical cell information, latitude, longitude, altitude, velocity, etc.) across various RRC states (connected, inactive, idle).

- Solution 3: introduce a “Continuous MDT” flag in the management based logged MDT measurement configuration, and enable the UE to report a “Continuous MDT” flag as soon as the UE establishes an RRC connection from Idle. Introduce a continuous MDT indication in the Handover Request message to enable the target NG-RAN to configure the UE with a management-based MDT configuration

- Solution 4: down-select the solution from option 1 to 3 to configure multiple signaling based MDTs (e.g., immediate MDT and log MDT) to a UE

- Option 1: add addition trace activation IE in Xn

- Option 2: add addition trace activation IE in both NG and Xn

- Option 3: extending the CHOICE MDT Mode IE

In this contribution, we focus on further discussing continuous MDT tracing.

# 2	Discussion

During previous RAN3 meetings, we discussed MDT enhancements to support AI/ML based processes. This present contribution revisits the subject and provides additional arguments in support of the continuous MDT tracing for AI/ML related processes, taking into account the feedback received so far.

As agreed in the RAN3#119bis_e meeting:

Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

Indeed, various contributions focusing on the MDT continuity updates have been discussed during the meeting.

In particular contribution [1] explains that it is useful if the UEs selected to report AI/ML data via MDT could continue to report MDT measurements independently of whether they are in CONNECTED, INACTIVE or IDLE state. This would ensure that the time series of measurements collected via MDT comes from the same UE and it is collected throughout different connection states, hence it is consistent and does not depend on e.g. UE capabilities, UE implementation etc., which would have happened if the same time series of data was collected by multiple UEs.

This is a great advantage for AI/ML, as each UE would provide consistent and continuous measurements from which a consistent representation of the network can be achieved. Failure to reuse the same UE for MDT collection in different states leads to an inconsistent set of MDT measurements, where each set of measurements collected by a UE depends on UE capabilities and implementation. The latter causes extra work for an AI/ML Model Training function because training data collected via MDT would need to be normalized first, to remove dependencies from UE capabilities and implementation and then be fed to a model training function.

As an example, [1] describes the issue and the solution as follows:

Figure 1: Example of continuous MDT measurement collection across RRC states

Similarly, [2] states that, for model training and model inference, input data should be consecutive and a time series of information with certain granularity. It also states that when UE location information needs to be leveraged as input, model training or model inference in NG-RAN node shall collect historical UE information including UE location information in the past. Therefore, [2] proposes to introduce an indication in the logged MDT measurement configuration to indicate to the UE whether to collect consecutive historical information (coordinates, latitude, longitude, altitude, etc) no matter what the UE state is.

Finally, in [3] the following is mentioned:

“For AI/ML function such as UE trajectory prediction, the data for training or inference has the serious requirement of recording time, e.g. data in series of time. There are two types of MDT: logged MDT for RRC inactive/idle states and immediate MDT for RRC connected state. The existing mechanism is to configure for inactive/idle and connected states separately. The measurement will stop when RRC state changes. So, part of records is missing during the RRC state transition. Thus, although the record intervals for logged MDT and immediate MDT are same, the record can not be listed in stringent time series. To solve this issue, an indicator for continuous location measurement during state change is required for MDT configuration.”

From all the above, it can be appreciated that there is a benefit in enabling a continuous collection of data via MDT for one or more UEs.

Observation 1: A fragmented collection of MDT measurements for AI/ML purposes from different types of UEs increases the burden of data preparation processes, while enabling continuous MDT measurement collection ensures more controlled and consistent data sets.

Conclusion 1: it is beneficial for AI/ML to enable collection of continuous data sets via MDT, i.e. data sets that do not need to be normalised against UE capabilities and implementations.

In the framework proposed in this contribution Management Based MDT is enhanced so that MDT data are collected by the UE while the UE changes RRC state and moves across the network (in RRC_Connected, RRC_ Idle and RRC_Inactive). A continuous training dataset from the same data source allows for:

- Continuous availability in time of training information at the AI/ML training function, as opposed to training data series that are affected by time interruptions e.g. due to stoppage of data collection at one UE and later resuming of data collection by other UEs

- Availability of data that are easier to interpret because they come from the same source. Namely, the AI/ML training function can deduce a clear representation of e.g. network radio conditions, network performance conditions, etc. as the measurement come from the same source (i.e. the same UE). On the contrary, if measurements came from different UEs at different times, the AI/ML training function would have to interpret them with respect to the capabilities of the UE that generates them (two UEs under the same conditions – e.g. served by the same cell and in the same location - with different capabilities, measuring the same RS, may report different values, which creates a problem at model training level as measurements do not present a normalized representation of the network)

- The possibility to monitor the network and the UE in any RRC state. This has for example advantages to generate historical UE trajectories, which can be collected and reported by the UE across Idle, Inactive and Connected and that can help training trajectory prediction models.

At the last RAN3 meeting questions were raised related to the potential use of existing methods for addressing the issues discussed. In particular, there were comments referencing signaling based MDT, stating that it could already enable continuous MDT data collection across RRC states.

Signalling based MDT enables to target a specific UE with an MDT configuration.

However, Signalling Based MDT has the following limitations when trying to achieve continuous MDT data collection:

- Signalling Based MDT cannot provide data collection continuity when the UE moves from Idle to Connected. This is because the serving NG-RAN needs to first establish a UE context before being able to receive a Signalling Based MDT configuration for the UE. On the contrary, Management Based MDT configurations are available at the NG-RAN before the UE moves to RRC_Connected, hence a Management MDT configuration can be applied to the UE immediately upon RRC Connection Setup

- Signalling Based MDT implies that, for each UE that needs to collect MDT data, NGAP signalling procedures need to be started and a signalling based MDT context needs to be maintained at the NG-RAN. This may result in a very signalling and processing intensive procedure if it is assumed that thousands of UEs may need to be configured. On the contrary, Management Based MDT does not require any NGAP signalling, nor storage of an MDT context for the involved UEs. By means of Management Based MDT, the NG-RAN can receive one or more MDT configurations from the OAM and apply them to as many UEs without CN involvement.

Observation 2: Signalling based MDT is not optimal for Continuous MDT collection due to measurement collections gaps caused by signalling based MDT configuration signalling and due to high signalling and processing demands

Proposal 1:  It is proposed to enable collection of MDT data sets by the same UE, across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive)

Proposal 2: Continuous MDT is based on Management Based MDT configurations with different trace sessions for immediate and logged MDT. In this way, a UE can be configured with Immediate MDT measurements and with a logged MDT configuration at the same time.

As per Figure 1, to achieve the above proposal it is necessary to take adequate measures. Currently, TS32.422 [3] prohibits the configuration of immediate MDT and logged MDT within the same trace session [“Immediate MDT, Logged MDT and Logged MBSFN MDT measurements shall always be configured as separate trace sessions”]. Correspondingly, one would have to use different management based trace sessions to configure the same UE with Immediate and Logged MDT measurements. With this approach, a UE may be configured with Management Based Immediate MDT measurements and Logged MDT measurements at the same time.

The challenge is how to ensure that a UE configured with Logged MDT, which subsequently goes to RRC_Connected, can be identified as the UE that needs to be configured with immediate MDT measurements and therefore achieve continuous MDT reporting. To achieve this, the RAN should configure the UE for immediate MDT (with a configuration associated to e.g. Trace Session 1) and before sending it to IDLE it should configure it again for logged MDT (with a configuration associated to e.g. Trace Session 2). The latter logged MDT configuration however should indicate that the UE is selected for continuous MDT reporting. This indication can be flagged by the UE when the UE moves to RRC Connected again, so to give the possibility to the RAN to configure the UE with Immediate MDT.

Proposal 3: Continuous MDT is based on signalling to the UE, as part of the Logged MDT configuration, that the UE is selected for Continuous MDT. This enables the UE to report a “Continuous MDT” flag as soon as the UE establishes an RRC connection from Idle. Hence the UE can be configured with Immediate MDT measurements

A summary of the procedure can be provided by the following figure and steps

Figure 2: Example of “continuous MDT” signalling support

- The RAN receives a Management Based MDT configuration made of at least two configurations: an Immediate MDT configuration and a Logged MDT configuration. The two configurations have different Trace Session identifiers. This configuration contains a “Continuous MDT” flag. With this information, the RAN knows that it has to configure selected UEs with MDT measurements in CONNECTED and in IDLE/INACTIVE.

- The RAN may configure the UE with Immediate MDT and logged MDT as per the configuration details above (i.e. including a “continuous MDT” flag in the Logged MDT configuration pushed to the UE).

- The UE reports Immediate MDT measurements collected while in RRC_CONNECTED and according to the Immediate MDT configuration received.

- The UE moves to IDLE/INACTIVE. The UE will collect logged MDT measurements and store them as per received Logged MDT configuration.

- When the UE moves from IDLE/INACTIVE to CONNECTED, the UE indicates to the RAN, as part of the RRC Setup procedure, that it is configured with Continuous MDT. This allows the RAN to immediately configure the UE with Immediate MDT measurements.

- The RAN configures the UE with a Logged MDT configuration including a “Continuous MDT” flag

- The UE reports Logged MDT results

- To guarantee for continuous MDT measurement collection across connected mode mobility, an indication of “Continuous MDT” is included in the Handover Preparation signalling towards the target RAN. The target RAN is therefore able to appropriately configure MDT at the UE.

It should be noted that there is no real need to identify the UE during such continuous MDT process, but rather to identify the trace session activity. For that the OAM can assign Trace References that allow to identify such continuous MDT measurement collection at the Trace Collection Entity (TCE) and therefore the TCE can deduce that measurements associated with such Trace Reference come from a specific set of UEs providing continuous measurements.

As an additional note we would like to clarify that the intention of these enhancements would not be that of providing real time data, but rather to enable the collection of a set of data from the same UE or group of UEs, so that:

- The Model Training function can receive a set of data corresponding to network measurements from the same one or more UEs. This makes the set of data consistent across time and less dependent on UE implementation and capabilities.

- The Model Training function can be fed with data continuously from the first MDT configuration set at the UE, which allows the Training function to carry out training within a given time frame (it should not be assumed that training is a process that can run forever, every AI/ML deployment will rely on training to be completed within a given time window).

Regarding the demands on UEs, the solution implies that the UE is enhanced to be able to receive a “continuous MDT” flag, as part of the Logged MDT configuration, and that the UE reports this flag at RRC Setup.

Proposal 4: Continuous MDT is based on signalling from the UE, as part of e.g. the RRC Setup procedure, that the UE is selected for Continuous MDT. This enables the NG-RAN to immediately configure the UE with Immediate MDT measurements

On the basis of the above solution description, we conclude that the solution we would like to propose for agreement in RAN3 is what has been captured as Solution 3 during RAN3-119bis, namely:

Solution 3: introduce a “Continuous MDT” flag in the management based logged MDT measurement configuration, and enable the UE to report a “Continuous MDT” flag as soon as the UE establishes an RRC connection from Idle. Introduce a continuous MDT indication in the Handover Request message to enable the target NG-RAN to configure the UE with a management-based MDT configuration

Based on all the above we would like to propose:

Proposal 5: The following solution is selected as way forward for MDT solution discussions:

Solution 3: introduce a “Continuous MDT” flag in the management based logged MDT measurement configuration, and enable the UE to report a “Continuous MDT” flag as soon as the UE establishes an RRC connection from Idle. Introduce a continuous MDT indication in the Handover Request message to enable the target NG-RAN to configure the UE with a management-based MDT configuration

Given that a solution for Continuous MDT needs to be validated by SA5, the following is proposed:

Proposal 6: Send the LS in [7] to SA5 to check for feasibility of the updates to the MDT Configuration concerning continuous MDT tracing.

to progress work in RAN3, the following is proposed:

Proposal 7: It is proposed to agree on the TP for AI/ML BLCR to TS38.413 [5].

Proposal 8: It is proposed to agree on the TP for AI/ML BLCR to TS38.423 [6].

# 3	Conclusion

In this contribution possible enhancements to the MDT framework that would enable MDT to be used for data collection for AI/ML have been discussed.

The following conclusions and proposals were derived:

Observation 1: A fragmented collection of MDT measurements for AI/ML purposes from different types of UEs increases the burden of data preparation processes, while enabling continuous MDT measurement collection ensures more controlled and consistent data sets.

Conclusion 1: it is beneficial for AI/ML to enable collection of continuous data sets via MDT, i.e. data sets that do not need to be normalised against UE capabilities and implementations.

Observation 2: Signalling based MDT is not optimal for Continuous MDT collection due to measurement collections gaps caused by signalling based MDT configuration signalling and due to high signalling and processing demands

Proposal 1:  It is proposed to enable collection of MDT data sets by the same UE, across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive)

Proposal 2: Continuous MDT is based on Management Based MDT configurations with different trace sessions for immediate and logged MDT. In this way, a UE can be configured with Immediate MDT measurements and with a logged MDT configuration at the same time.

Proposal 3: Continuous MDT is based on signalling to the UE, as part of the Logged MDT configuration, that the UE is selected for Continuous MDT. This enables the UE to report a “Continuous MDT” flag as soon as the UE establishes an RRC connection from Idle. Hence the UE can be configured with Immediate MDT measurements

Proposal 4: Continuous MDT is based on signalling from the UE, as part of e.g. the RRC Setup procedure, that the UE is selected for Continuous MDT. This enables the NG-RAN to immediately configure the UE with Immediate MDT measurements

Proposal 5: The following solution is selected as way forward for MDT solution discussions:

Solution 3: introduce a “Continuous MDT” flag in the management based logged MDT measurement configuration, and enable the UE to report a “Continuous MDT” flag as soon as the UE establishes an RRC connection from Idle. Introduce a continuous MDT indication in the Handover Request message to enable the target NG-RAN to configure the UE with a management-based MDT configuration

Proposal 6: Send the LS in [7] to SA5 to check for feasibility of the updates to the MDT Configuration concerning continuous MDT tracing.

Proposal 7: It is proposed to agree on the TP for AI/ML BLCR to TS38.413 [5].

Proposal 8: It is proposed to agree on the TP for AI/ML BLCR to TS38.423 [6].

# References

- R3-231604, MDT Enhancements for AI/ML, Ericsson, Deutsche Telekom, InterDigital

- R3-231842, Discussion on MDT enhancement for continuous AI-ML related information, Lenovo, Samsung

- R3-231208, Discussion on MDT enhancement for AI/ML for NG-RAN, Samsung

- 3GPP TS 32.422: "Subscriber and equipment trace; Trace control and configuration management".

- R3-233117, “(TP for AI/ML BLCR to TS38.413) MDT Enhancements for AI/ML”, Ericsson, Deutsche Telekom, InterDigital, AT&T

- R3-233116, “(TP for AI/ML BLCR to TS38.423) MDT Enhancements for AI/ML”, Ericsson, Deutsche Telekom, InterDigital, AT&T

- R3-233118, “LS on Continuous MDT”, Ericsson, Deutsche Telekom, InterDigital, AT&T