3GPP TSG-RAN WG3 Meeting #120	R3-233135

# Incheon, South Korea, May 22nd – 26th 2023

Agenda Item:	8.3

Source:	Ericsson

Title:	UE identifiers for management-based trace data

Document for:	Discussion

# 1	Introduction

RAN3 received an LS from SA5 in [1], about UE identifiers for the support of trace for dual-connectivity and split RAN architecture.

SA5 wants to be able to correlate traces, for the same UE, from different RAN entities e.g. MN, SN, CU-CP, CU-UP, DU.

SA5 asks RAN3 to provide feedback on the following aspects:

1.	Availability of UE identifier(s) suitable for trace data correlation for Dual Connectivity and split deployments

2.	Any relevant details and/or limitations in their usage, including persistency and conditionality

3.	Details of any ongoing work which could affect any details provided

This contribution aims at discussing the 3 aspects above, and at establishing a group position in order to draft an answer to SA5.

# 2	Discussion

## 2.1 Availability of UE identifier(s)

The first question is about the availability of UE identifier(s) for trace correlation within the RAN. RAN3 has many UE identifiers used over different interfaces, but wat SA5 is looking for is a common identifier, shared by different entities, so that trace data from the same UE can be identified in DC and split RAN architecture. A similar discussion happened in rel-15 (see [2]), and led to the introduction of the RAN UE ID in F1AP and E1AP. The main goal of the identifier was to correlate traces or logs from different RAN entities (here CU-UP and CU-CP), for the same UE.

Observation 1: The RAN UE ID has been introduced in rel-15 for a purpose similar to what SA5 is now asking

Therefore, it is proposed to inform SA5 of the existence of the RAN UE ID in RAN.

Proposal 1: For the 1st question, answer to SA5 that the RAN UE ID is available in RAN, for a purpose similar to trace data correlation

During RAN2#119bis-e, some companies mentioned the TR/TRSR. However, this ID is out of scope of this LS Response and shall not be mentioned to SA5, for the following reasons:

- SA5 is asking for the most appropriate “UE Identifier” which meets the criteria in the LS. TR/TRSR is not a UE ID;

- The mechanism described by some companies to send an associated UE ID via the CN (and cell traffic trace mechanism) is not under RAN3 scope, and therefore shall not be mentioned;

- The enhancements of TR/TRSR introduced in TS 38.401 to cover the split architecture concerns MDT Traces and not Cell Trace, which is the topic of this LS. For example, initial access is not covered in MDT, because the TR/TRSR is allocated only when the UE is connected to the CN (MDT can’t start until the context is being set up as it depends on getting consent before the measurements are started).

Proposal 2: TR/TRSR is not in the scope of this LS response

## 2.2 Details and limitations of RAN UE ID

The 2nd question asks more detail about the possible UE identifiers, and how to use it in the context of trace data correlation for DC and split RAN architecture.

It can be seen that the RAN UE ID is only present over F1AP and E1AP, and therefore is only suitable for the split RAN architecture use-case, and not the DC use-case.

Observation 2: RAN UE ID cannot be used for the DC use-case

Also, this IE is optional, and there are no conditions mentioned in the procedural text for this IE.

Observation 3: No conditions are defined for the presence of the RAN UE ID in F1AP and E1AP messages

SA5 also asks about the persistency of the UE identifier(s). Regarding the RAN UE ID, it can be seen that it will not survive inter-node HO, UE going to Idle or Inactive.

Observation 4: RAN UE ID does not survive inter-node HO, UE going to Idle or Inactive

Proposal 3: For the 2nd question, answer to SA5 that the RAN UE ID is limited to the split RAN architecture, that there are no conditions defining the presence of this identifier into corresponding F1AP and E1AP messages, and that the RAN UE ID does not survive HO or UE going to Idle or Inactive

## 2.3 Ongoing work in RAN3

The 3rd question would be easier to answer, as there is no rel-18 WID mentioning any similar objective, or which will impact the current discussion.

Proposal 4: For the 3rd question, answer to SA5 that RAN3 has not identified any ongoing work that could affect the provided feedback

# 3	Conclusion

The LS from SA5 on UE identifiers for management-based trace data has been discussed and the following observations and proposals have been made:

- Observation 1: The RAN UE ID has been introduced in rel-15 for a purpose similar to what SA5 is now asking

- Proposal 1: For the 1st question, answer to SA5 that the RAN UE ID is available in RAN, for a purpose similar to trace data correlation

Proposal 2: TR/TRSR is not in the scope of this LS response

Observation 2: RAN UE ID cannot be used for the DC use-case

Observation 3: No conditions are defined for the presence of the RAN UE ID in F1AP and E1AP messages

Observation 4: RAN UE ID does not survive inter-node HO, UE going to Idle or Inactive

Proposal 3: For the 2nd question, answer to SA5 that the RAN UE ID is limited to the split RAN architecture, that there are no conditions defining the presence of this identifier into corresponding F1AP and E1AP messages, and that the RAN UE ID does not survive HO or UE going to Idle or Inactive

Proposal 4: For the 3rd question, answer to SA5 that RAN3 has not identified any ongoing work that could affect the provided feedback

# 4	References

[1] R3-231121 - LS on RAN UE Id optionality (SA5)

[2] R3-191720 - RAN UE ID (Ericsson)