3GPP TSG-RAN WG3 Meeting #120	R3-233299

# Incheon, South Korea, May 22nd – 26th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0294 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

x

| Title: | Clarification of the RAN UE ID usage in stage-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Orange, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The RAN UE ID has been introduced in E1AP and F1AP specifications during rel-15, but its usage has never been specified in stage-2. How to allocate and transfer the RAN UE ID is therefore unclear and can lead to misinterpretation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the RAN UE ID among the other RAN identifiers. Impact assessment towards the previous version of the specification (same release): This CR has no impact on the protocol. This CR has no functionality impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.x (new), 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of the 1st Set of Changes >>>>>>>>>>>>>>>>>>>>

## 6.2	NG-RAN identifiers

-- TEXT OMITTED –

### 6.2.x	RAN UE ID

The RAN UE ID is allocated to a UE by the gNB-DU at UE Initial Access for troubleshooting and observability purposes. The RAN UE ID is a unique identifier that needs to be transferred between RAN nodes in order to correlate logs for a given UE between different RAN interfaces.

<<<<<<<<<<<<<<<<<<<< End of the 1st Set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of the 2nd set of Changes >>>>>>>>>>>>>>>>>>>>

## 8.1	UE Initial Access

The signalling flow for UE Initial access is shown in Figure 8.1-1.

Figure 8.1-1: UE Initial Access procedure

1.	The UE sends an RRCSetupRequest message to the gNB-DU.

2.	The gNB-DU includes the RRC message and, if the UE is admitted, the corresponding low layer configuration for the UE in the  INITIAL UL RRC MESSAGE TRANSFER message and transfers to the gNB-CU. The INITIAL UL RRC MESSAGE TRANSFER message includes the C-RNTI and the RAN UE ID allocated by the gNB-DU.

3.	The gNB-CU allocates a gNB-CU UE F1AP ID for the UE and generates a  RRCSetup message towards UE. The RRC message is encapsulated in -the DL RRC MESSAGE TRANSFER message.

4.	The gNB-DU sends the RRCSetup message to the UE.

5.	The UE sends the RRC CONNECTION SETUP COMPLETE message to the gNB-DU.

6.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

7.	The gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

8.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

9.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the UE context in the gNB-DU. In this message, it may also encapsulate the SecurityModeCommand message. In case the RAN UE ID provided in step 2 needs to be updated, the gNB-CU includes the new RAN UE ID in the UE CONTEXT SETUP REQUEST message..In case of NG-RAN sharing, the gNB-CU includes the serving PLMN ID (for SNPNs the serving SNPN ID).

10.	The gNB-DU sends the SecurityModeCommand message to the UE.

11.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message to the gNB-CU.

12.	The UE responds with the SecurityModeComplete message

13.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

14.	The gNB-CU generates the RRCReconfiguration message and encapsulates it in the DL RRC MESSAGE TRANSFER message

15.	The gNB-DU sends RRCReconfiguration message to the UE.

16.	The UE sends RRCReconfigurationComplete message to the gNB-DU.

17.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

18.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>