3GPP TSG-RAN WG3 Meeting #121bis	R3-235147

Xiamen, China, 09 - 13 October 2023

Agenda item:	26.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.413 BL CR) Resolution of open issues for timing resiliency

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we evaluate and resolve open issues for timing resiliency according to the following WID objectives [1]:

1. 5GS network timing synchronization status and reporting [RAN3, RAN2]: a. AMF providing clock quality reporting control information per-UE to the gNB. [RAN3] b. gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3] Note 1: Details of the 5G clock quality information will be decided by RAN3. c. UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2] d. gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3]

# 2	Discussion

## 2.1	Encoding of the Clock Quality Acceptance Criteria IE

SA2 refers to TS 23.501 Table 5.27.1.12-1 to list the attributes that may be contained in RAN Timing Synchronization Status (TSS) reporting.

Table 5.27.1.12-1: Information elements that gNB or UPF/NW-TT timing synchronization status information may contain (all optional)

| Information Name | Description |
| --- | --- |
| Synchronization state | Indicates the state of the node synchronization, represented by the values "Locked", "Holdover", or "Freerun" (NOTE 1). |
| Clock quality |  |
| >> Traceable to GNSS | Indicates whether the current time source is traceable to the GNSS and represented by values “Yes” or “No”. |
| >> Traceable to UTC | Indicates whether the current time source is traceable to the UTC and represented by values “Yes” or “No”. |
| >> Frequency stability | Describes the estimate of the variation of the local clock when it is not synchronized to another source (NOTE 2). |
| >> Clock Accuracy | Describes the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock (NOTE 3). |
| Parent time source | Describes the primary source the node is currently using, represented by the values "SyncE", "PTP", "GNSS", "atomic clock", "terrestrial radio", "serial time code", "NTP", "hand_set", "other". |
| NOTE 1: Clock is in the "Locked", "Holdover", or "Freerun" mode, as defined in ITU-T G.810 [164]. NOTE 2: Frequency stability is estimated in a similar manner as for offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588 [126]. NOTE 3: Clock accuracy measurement considers accuracy up to gNB antenna and RAN internal process. |  |

For clock quality reporting towards the UE, the gNB receives clock quality reporting control information from the AMF including the Clock Quality Detail Level IE which can take one of the following values: “clock quality metrics” or “acceptable/not acceptable indication”.

When the clock quality detail level equals “acceptable/not acceptable indication”, the gNB also receives the Clock Quality Acceptance Criteria IE from the AMF. The acceptance criteria are associated with a UE and used to decide the service status after a primary time source event (e.g., degradation, failure, recovery). According to TS 23.501, the clock quality acceptance criteria for the UE may be defined based on one or more attributes listed in Table 5.27.1.12-1. That is, the same table in SA2 can be used for defining both the clock quality acceptance criteria and for RAN TSS reporting when there is only a single acceptable value/threshold for the attribute, thus, the same encoding could be reused in RAN3 specifications.

Observation-1:	The Clock Quality Acceptance Criteria IE could reuse the same encoding as the RAN Timing Synchronisation Status IE if there is only a single acceptable value/threshold for the attribute.

However, there is an additional consideration for how to encode the Clock Quality Acceptance Criteria IE. While it is clear that a RAN TSS report has a single value per supported attribute of Table 5.27.1.12-1, the Clock Quality Acceptance Criteria IE could potentially have multiple acceptable values for two of the attributes in Table 5.27.1.12-1 (i.e., synchronization state and parent time source). For example, a time synchronization service for a UE could be “acceptable” as long as the synchronization state is not “Freerun”. This means two different values of synchronization state are tolerated (i.e., “Locked", "Holdover”). A similar case can be seen for parent time source, where a service could be considered “acceptable” as long as it is within the subset of a few sources (e.g., all parent time sources except “GNSS”). From RAN3 perspective, different alternatives can be considered to address this area:

Option A) Multiple values of clock quality acceptance criteria are allowed for the same attribute. This is only applicable for synchronization state and parent time source attributes. Different encoding options can be decided:

A.1) The RAN Timing Synchronisation Status Information IE and Clock Quality Acceptance Criteria IE use the same encoding, and the clock quality reporting control information allows to have a list of Clock Quality Acceptance Criteria IE (instead of one IE). A similar approach is already specified in TS 29.503 for the clock quality acceptance criteria encoding at the UDM. In TS 29.503, the TimeSyncData type defines clockQualityAcceptanceCriteria as 1…N ClockQualityAcceptanceCriterion. This array of ClockQualityAcceptanceCriterion can have one element of synchronization state, clock quality, or parent time source. If an array of criterion is configured, multiple instances of the synchronization state or parent time source can be stored.

A.2) The RAN Timing Synchronisation Status Information IE and Clock Quality Acceptance Criteria IE do not use the same encoding. For synchronization state and parent time source attributes, the RAN Timing Synchronisation Status Information IE uses ENUMERATED type while the Clock Quality Acceptance Criteria IE uses BIT STRING type where each bit corresponds to a value of the attribute and “1” indicates “acceptable”.

Option B)	Multiple values of clock quality acceptance criteria are not allowed for the same attribute. The RAN Timing Synchronisation Status Information IE and Clock Quality Acceptance Criteria IE use the same encoding.

From the above alternatives, option A.2 seems suitable for the synchronisation state and parent time source attributes.

Observation-2:	There can be multiple acceptable values for synchronisation state and parent time source. The clock quality acceptance criteria for these attributes can be encoded using BIT STRING where each bit represents an attribute value.

The rest of the clock quality acceptance criteria attributes only need one configuration to define the acceptable threshold for the UE’s service. That is, for the attributes Traceable to UTC, Traceable to GNSS, Clock Frequency Stability, and Clock Accuracy, there is only one value/threshold allowed per UE. Even if Clock Accuracy were to have multiple choices for encoding of the IE, it is sufficient to have a single encoding for the acceptance criteria, i.e., INTEGER with 25ns granularity (pending RAN3 conclusion on the encoding of Clock Accuracy IE, see section 2.2) since the acceptance criteria is just a threshold for the service, it is not related to gNB implementation.

Observation-3:	Only a single value of clock quality acceptance criteria is allowed for the remaining attributes (i.e., Traceable to UTC, Traceable to GNSS, Clock Frequency Stability, Clock Accuracy). The clock quality acceptance criteria for these attributes can be encoded the same as in the RAN Timing Synchronisation Status Information IE. For Clock Accuracy, a single choice with the smallest allowed granularity (e.g. 25ns) is sufficient.

The resulting encoding for the Clock Quality Acceptance Criteria IE is as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | BIT STRING { locked (0), holdover (1), freeRun (2) } (SIZE (8, …)) | Each position in the bitmap represents a synchronisation state. If a bit is set to “1”, the respective synchronisation state is acceptable. If a bit is set to “0”, the respective synchronisation state is not acceptable. Bits 3-7 reserved for future use. |
| Traceable to UTC | O |  | ENUMERATED (true, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | INTEGER (1..400000000, …) | Clock accuracy expressed in units of 25 ns. |
| Parent Time Source | O |  | BIT STRING { syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4), serialTimeCode (5), nTP (6), handset (7), other (8) } (SIZE (16, …)) | Each position in the bitmap represents a parent time source. If a bit is set to “1”, the respective parent time source is acceptable. If a bit is set to “0”, the respective parent time source is not acceptable. Bits 9-15 reserved for future use. |

Table 1: Clock Quality Acceptance Criteria

Proposal 1:	Encode the Clock Quality Acceptance Criteria IE as shown in Table 1 above.

## 2.2	Encoding of the Clock Accuracy IE

During RAN3#120 different encodings of the Clock Accuracy IE were discussed. The description of clock accuracy in SA2 states that it is “the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock”. Clock accuracy claims an accuracy the clock has, that is, a worst-case description. When a clock indicates an accuracy value, for example, 2μs clock accuracy, it is expected the attribute has been conservatively estimated (to a measurement precision consistent with the type of time source the clock has, elapsed time since the last synchronization, holdover specifications, etc.). Then, the expected accuracy the clock will have will be within the 2μs indicated. The encoding of clock accuracy as an integer allows the gNB to provide a single value (the worst case) of the expected accuracy of its clock. A maximum of 1s seems enough to cover a vast range of use cases and the granularity of the attribute can be set to 25ns (e.g., aligned with uncertainty IE granularity). To clarify the interpretation of clock accuracy, the following semantics description can be added “Indicates the time is accurate to within the clock accuracy value”.

Proposal 2:	The encoding of the Clock Accuracy IE is a single INTEGER type with 25ns granularity and range up to 1sec. The semantics description clarifies that time is accurate to within the clock accuracy value.

If proposal 2 is agreeable, then it completes the encoding of all attributes in the Timing Synchronisation Status Attributes IE. Since these attributes can be provided to the UE via dedicated RRC signalling when a UE’s clock quality detail level equals “clock quality metrics”, RAN3 should inform RAN2 (and SA2/CT4) of the encoding as well as any other relevant RAN3 agreements.

Proposal 3:	Send an LS to RAN2/SA2/CT4, informing them of the encoding of the Timing Synchronisation Status Attributes IE and any other relevant RAN3 agreements.

## 2.3	RAN TSS reporting control over F1AP

SA2 describes the use of pre-configured thresholds as follows in TS 23.501 clause 5.27.1.12:

[…] gNBs may be pre-configured with thresholds for each timing synchronization status attribute, if supported, that is described in Table 5.27.1.12-1. When the network timing synchronization status exceeds the thresholds (i.e. status degradation), or the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB notifies the TSCTSF (either using N2 node level signalling via AMF, or via OAM) with the scope of the timing synchronization status (i.e. gNB ID or a list of Cell IDs within a single gNB) and the corresponding network timing synchronization status (TSS) attributes as described in this clause. The gNB indicates the status change to the UEs via SIB information: […] NOTE 1: NG-RAN is assumed not to provide clock quality metrics better than the pre-configured threshold, i.e. if a clock quality metric is better than the corresponding threshold, the NG-RAN reports the threshold value to the UE in an RRC message instead. NOTE 2: It is assumed the pre-configured thresholds in the gNB(s) are sufficient to meet UE time sync performance requirement which are configured by the operator.

The pre-configured thresholds are node-level attributes of the gNB-DU, not associated with a specific UE. As described in NOTE 1 above and taking clock accuracy as an example, if the pre-configured threshold for the clock accuracy attribute is 400ns, then the gNB never reports a clock accuracy value less than 400ns; if the actual accuracy is e.g. 350ns, the gNB only reports the 400ns value of the preconfigured threshold. The pre-configured thresholds are assumed to be strict enough to meet the requirements of all UEs that may consume time synchronization services (as indicated in NOTE 2 above), e.g., in the example no UEs have a clock accuracy requirement stricter than 400ns.

Observation-4:	There is only a single pre-configured threshold for each RAN TSS attribute, and it is assumed to meet the time sync performance requirement of all UEs.

In addition to the pre-configured thresholds, the gNB may receive UE-specific clock quality acceptance criteria from the CN when the clock quality detail level for the UE is “acceptable/not acceptable”. When evaluating the clock quality acceptance criteria, the NG-RAN split architecture brings additional considerations which were not discussed in SA2 since CU-DU split is transparent to WGs outside of RAN3. The baseline for the gNB-CU is to know if the pre-configured thresholds (not associated to any UE) at the gNB-DU are no longer met or are met again. Taking clock accuracy as an example, while the preconfigured threshold is met, then the clock accuracy is acceptable for all UEs. However, while the preconfigured threshold is no longer met:

-	If the acceptance criterion for clock accuracy is the same as the pre-configured threshold for all UEs, then the gNB-CU knows that the current clock accuracy is not acceptable for all UEs;

-	If the acceptance criterion for clock accuracy is more relaxed than the pre-configured threshold for at least some UEs, then the gNB-CU does not know whether the current clock accuracy is acceptable or not for those UEs since it only knows that the clock accuracy is worse than the pre-configured threshold, but does not know how much worse. The gNB-DU will not generate any additional RAN TSS reports until the pre-configured threshold is met again.

For the gNB-CU to know the current RAN TSS status of the gNB-DU, gNB-CU needs the non-UE associated RAN TSS reports from the gNB-DU. Non-UE associated signaling is preferred between the gNB-CU and gNB-DU because the status information at the gNB-DU applies to all the UEs served by it. Then, the gNB-CU correlates the current RAN TSS (non-UE associated) and UE’s clock quality acceptance criteria (UE associated) to determine the “acceptable/not acceptable” result for a specific UE. Based on the result, a dedicated RRC message is generated at the gNB-CU and sent to the UE.

The goal is to keep the gNB-CU updated with the current status of the synchronization plane of the gNB-DU to evaluate the service status (acceptable/not acceptable) provided to each UE. The example in Figure 1 illustrates the desirability for the gNB-DU to trigger a RAN TSS report not only when the pre-configured threshold is crossed, but also when other thresholds are crossed that correspond to the acceptance criteria of UE1 and UE2.

Figure 1: Example of RAN TSS reporting towards UE1 and UE2 based on UE-specific clock accuracy acceptance criteria

To our understanding, following NOTE 2 in TS 23.501 clause 5.27.1.12, the pre-configured thresholds configured at the gNB will be the strictest ones sufficient to meet all UE’s time synchronization criteria. However, for more relaxed acceptance criteria configured for specific UEs, the pre-configured thresholds would not be enough to ensure the gNB-CU gets enough updates to evaluate the acceptance criteria.

Observation-5:	RAN TSS reporting triggered only by pre-configured thresholds is insufficient to support clock quality detail level “acceptable / not acceptable”.

To ensure the acceptance criteria can be evaluated at the gNB-CU, there are several solution options as shown in Table 2. The clock quality is considered to be in “degraded state” when a pre-configured threshold is no longer met.

| Option | F1AP impact | gNB-DU behavior while clock quality is in “degraded state” |
| --- | --- | --- |
| A) | gNB-CU signals a list of clock accuracy thresholds in the TSS REQUEST message, corresponding to the acceptance criteria of UEs served by the gNB-CU. | gNB-DU triggers a RAN TSS report whenever any of the signalled thresholds are crossed. |
| B) | Procedural impact only. | gNB-DU triggers a RAN TSS report whenever the clock accuracy value changes (e.g., by at least 25ns, which is the clock accuracy granularity of Proposal 2). |
| C) | gNB-CU signals a relative threshold (an integer in units of 25ns) in the TSS REQUEST message. | gNB-DU triggers a RAN TSS report whenever the clock accuracy value changes by the signalled relative threshold. |

Table 2: Solution options for RAN TSS reporting over F1AP while clock is in “degraded state”

Taking the example in Figure 1 to illustrate each option:

-	Option A: The gNB-CU signals clock accuracy thresholds of 400ns, 500ns, and 550ns to the gNB-DU. Then, while the clock quality is in “degraded state”, the gNB-DU triggers a RAN TSS report whenever any of the signalled thresholds are crossed. Therefore, additional RAN TSS reports may be triggered at 500ns and 550ns.

-	Option B: While the clock quality is in “degraded state”, the gNB-DU triggers a RAN TSS report whenever the clock accuracy value changes by e.g., 25ns (if Proposal 2 is agreed). Therefore, additional RAN TSS reports may be triggered at 425ns, 450ns, 475ns, 500ns, etc.

-	Option C: The gNB-CU signals a relative threshold value of 2 to the gNB-DU. Then, while the clock quality is in “degraded state”, the gNB-DU triggers a RAN TSS report whenever the clock accuracy value changes by 50ns. Therefore, additional RAN TSS reports may be triggered at 450ns, 500ns, etc.

All three options seem feasible. Option A appears to be aligned with SA2’s intention and matches the behavior of pre-configured thresholds, i.e., a RAN TSS report is triggered when a threshold is no longer met or is met again. Therefore, Option A seems preferable.

Proposal 4:	Discuss additional triggers for gNB-DU to report RAN TSS while the clock quality is in degraded state, to enable the gNB-CU to evaluate clock quality acceptance criteria.

Proposal 5:	Add an optional Clock Accuracy Threshold List IE to the F1AP: TIMING SYNCHRONISATION STATUS REQUEST message, that includes a list of clock accuracy thresholds corresponding to the acceptance criteria of UEs served by the gNB-CU.

# 3	Conclusions

In this paper, we evaluated open issues for timing resiliency and made the following proposals:

Proposal 1:	Encode the Clock Quality Acceptance Criteria IE as shown in Table 1 above.

Proposal 2:	The encoding of the Clock Accuracy IE is a single INTEGER type with 25ns granularity and range up to 1sec. The semantics description clarifies that time is accurate to within the clock accuracy value.

Proposal 3:	Send an LS to RAN2/SA2, informing them of the encoding of the Timing Synchronisation Status Attributes IE and any other relevant RAN3 agreements.

Proposal 4:	Discuss additional triggers for gNB-DU to report RAN TSS while the clock quality is in degraded state, to enable the gNB-CU to evaluate clock quality acceptance criteria.

Proposal 5:	Add an optional Clock Accuracy Threshold List IE to the F1AP: TIMING SYNCHRONISATION STATUS REQUEST message, that includes a list of clock accuracy thresholds corresponding to the acceptance criteria of UEs served by the gNB-CU.

A draft LS is provided in Annex A, an NGAP TP is provided in Annex B, and an F1AP TP is provided in Annex C.

# References

- RP-231106 Revised WID on NR Timing Resiliency and URLLC enhancements, Nokia, Nokia Shanghai Bell

# Annex	A:	Draft LS

3GPP TSG-RAN WG3 Meeting #121bis	R3-23xxxx

Xiamen, China, 9 – 13 October 2023

Title:	LS on timing resiliency

Release:	Release 18

Work Item:	TRS_URLLC-NR

Source:	RAN3

To:	RAN2, SA2, CT4

Cc:	-

Contact Person:

#### Name:	Sean Kelley

E-mail Address:	sean.kelley@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 is currently discussing 5GS network timing synchronization status and reporting. RAN3 has reached agreement regarding encoding details of the RAN timing synchronization status attributes as shown below.

| IE Name | Encoding |
| --- | --- |
| Synchronisation State | ENUMERATED (locked, holdover, freeRun, …) |
| Traceable to UTC | ENUMERATED (true, false, …) |
| Traceable to GNSS | ENUMERATED (true, false, …) |
| Clock Frequency Stability | BIT STRING (SIZE (16)) |
| Clock Accuracy | INTEGER (1..40000000, …) |
| Parent Time Source | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |

2. Actions:

To RAN2, SA2, CT4.

ACTION: 	Please take into account RAN3 agreements on the encoding details of the RAN timing synchronization status attributes.

3. Date of Next TSG-RAN WG3 Meeting:

RAN3#122	from 2023-11-13	to 2023-11-17		Chicago

# Annex	B: Text Proposal for TS 38.413

first modification

#### 9.3.1.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | BIT STRING { locked (0), holdover (1), freeRun (2) } (SIZE (8, …)) | Each position in the bitmap represents a synchronisation state. If a bit is set to “1”, the respective synchronisation state is acceptable. If a bit is set to “0”, the respective synchronisation state is not acceptable. Bits 3-7 reserved for future use. |
| Traceable to UTC | O |  | ENUMERATED (true, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | INTEGER (1..400000000, …) | Clock accuracy expressed in units of 25 ns. |
| Parent Time Source | O |  | BIT STRING { syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4), serialTimeCode (5), nTP (6), handset (7), other (8) } (SIZE (16, …)) | Each position in the bitmap represents a parent time source. If a bit is set to “1”, the respective parent time source is acceptable. If a bit is set to “0”, the respective parent time source is not acceptable. Bits 9-15 reserved for future use. |

#### 9.3.1.x3	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided towards the AMF as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | INTEGER (1..40000000, …) |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

next modification

### 9.4.5	Information Element Definitions

** unchanged text skipped **

** unchanged text skipped **

ClockQualityAcceptanceCriteria ::= SEQUENCE {

synchronisationState			BIT STRING (SIZE(8))				OPTIONAL,

traceabletoUTC					ENUMERATED {true, ...}				OPTIONAL,

traceabletoGNSS					ENUMERATED {true, ...}				OPTIONAL,

clockFrequencyStability			BIT STRING (SIZE(16))				OPTIONAL,

clockAccuracy					INTEGER (1..40000000, ...)			OPTIONAL,

parentTImeSource				BIT STRING (SIZE(16))				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ClockQualityAcceptanceCriteria-ExtIEs} }	OPTIONAL,

...

}

ClockQualityAcceptanceCriteria-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

** unchanged text skipped **

RANTimingSynchronisationStatusInfo	::= SEQUENCE {

synchronisationState			ENUMERATED {locked, holdover, freerun, ...}						OPTIONAL,

traceabletoUTC					ENUMERATED {true, false, ...}									OPTIONAL,

traceabletoGNSS					ENUMERATED {true, false, ...}									OPTIONAL,

clockFrequencyStability			BIT STRING (SIZE(16))											OPTIONAL,

clockAccuracy					INTEGER (1..40000000, ...)													OPTIONAL,

parentTImeSource				ENUMERATED {synce, ptp, gnss, atomicclock, terrestrialradio, serialtimecode, ntp, handset, other, ...}	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {RANTimingSynchronisationStatusInfo-ExtIEs} }	OPTIONAL,

...

}

RANTimingSynchronisationStatusInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

# Annex	C: Text Proposal for TS 38.473

first modification

#### 9.2.yy.1	TIMING SYNCHRONISATION STATUS REQUEST

This message is sent by the gNB-CU to request the gNB-DU to start or stop reporting of RAN timing synchronization status information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN TSS Request Type | M |  | ENUMERATED (start, stop, …) |  | YES | reject |
| Clock Accuracy Threshold List | O |  | 9.3.1.y1 |  | YES | ignore |

last modification

#### 9.3.1.x1	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [21]. |
| Clock Accuracy | O |  | INTEGER (1..40000000, …) |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

#### 9.3.1.y1	Clock Accuracy Threshold List

This IE indicates the list of clock accuracy thresholds relevant while the clock quality is degraded.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Clock Accuracy Threshold Item |  | 1..<maxnoofClockAccuracyThreshold> |  |  |
| >Clock Accuracy Threshold | M |  | INTEGER (1..400000000, …) | Clock accuracy threshold expressed in units of 25 ns. |

| Range bound | Explanation |
| --- | --- |
| maxnoofClockAccuracyThreshold | Maximum no. of clock accuracy thresholds. Value is 8 [FFS]. |