3GPP TSG-RAN WG3 Meeting #121bis	R3-235397

Xiamen, 9 Oct – 13 Oct 2023

Title: 	Discussion on QoE measurement in NR-DC

Source: 	China Unicom

Agenda item:	11.3

Document Type:	Discussion

1	Introduction

This paper provides the text proposal for the BL CR of 38.423, based on the discussion in R3-234811.

2	TP to BL CR of 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

9.2.3.x1	QMC Initial Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Configuration List |  |  |  |  |
| >QMC Configuration List Item |  |  | 1..<maxnoofUEAppLayerMeas> |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE could only be sent by the MN. |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Configuration Sending Request | O |  | BOOLEAN | This IE could only be sent by the SN. |
| >>QoE Reporting Option Preference | O |  | ENUMERATED (SRB4, SRB5[FFS], ...) |  |
| >>RVQoE Reporting Option Request | O |  | ENUMERATED (SRB4,…) |  |
| >>Available RAN Visible QoE Metrics | O |  |  | This IE can be used in the SN change procedure. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

Editor’s Note: Whether to also include QoE configuration container in the IE is FFS.

-------------------------------------------Next change-------------------------------------------

9.2.3.x2	QMC Initial Coordination Response

Editor’s Note: The messages used to convey this IE are FFS.

This IE contains the information that the M-NG-RAN node needs to provide in response to the S-NG-RAN node or the information that the S-NG-RAN node needs to provide in response to the M-NG-RAN node, for the  QMC initial coordination.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- | --- |
| QMC Response List |  |  |  |  |  |
| >QMC Response List Item |  |  |  | 1..<maxnoofUEAppLayerMeas> |  |
| >>QoE Reference | M |  |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  |  | INTEGER (0..15, ...) |  |
| >>QoE Configuration Sending Option Response |  | O |  | ENUMERATED (MN, SN, …). | This IE could only be sent by the MN. |
| >>QoE Reporting Option |  | O |  | ENUMERATED (SRB4, SRB5, …) |  |
| >>RVQoE Reporting Option Response |  | O |  | ENUMERATED (SRB4, …) | This IE could only be sent by the MN. |
| >>RAN Visible QoE Configuration Preference |  | O |  | 9.2.3.x5 | This IE can be used in the SN change procedure to provide the RAN Visible QoE Configuration Preference of the new SN. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

-------------------------------------------Next change-------------------------------------------

9.2.3.x3	QMC Modification Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for requesting modification of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Modification Request List |  |  |  |  |
| >QMC Modification Request Item |  |  | 1..<maxnoofUEAppLayerMeas> |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE could only be sent by the MN. |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 |  |
| >>QoE Reporting Modification Request | O |  | BOOLEAN | This IE indicates the request to switch the SRB used for receiving of QoE reports. |
| >> Application Session Status | O |  | ENUMERATED (MN, SN, MN and SN) | This IE indicates the bearer(s) for the application session is in the MN, SN or bother MN and SN. |
| >>RAN Visible QoE Reporting Modification Request | O |  | ENUMERATED (true, ...) |  |
| >>Available RAN Visible QoE Metrics | O |  | 9.2.3.158 |  |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

9.2.3.x4	QMC Modification Response

This IE contains the information that the S-NG-RAN node or the information that the M-NG-RAN node needs to provide in the response to the QMC modification request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Modification Response List |  |  |  |  |
| >QMC Modification Response Item |  |  | 1..<maxnoofUEAppLayerMeas> |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>QoE Reporting Modification response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>RVQoE Reporting Modification response | O |  | ENUMERATED (accepted, rejected, release) | FFS whether the code points should be refined, to reflect the agreement about the response from non-RVQoE configuring node. |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>