3GPP TSG-RAN WG3 Meeting #121bis	R3-235548

Xiamen, China, 09 - 13 October 2023

Agenda item:	23.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.455 BL CR) Resolution of open issues for UL CPP

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we resolve several open issues related to positioning accuracy enhancements, in particular UL CPP.

# 2	Discussion

## 2.1	Uplink carrier phase positioning (UL CPP)

UL CPP was discussed at the previous RAN3 meeting, and some agreements reached with FFSes. Meanwhile, RAN1 has made significant progress and reached the following agreements at RAN#114 related to configuration of UL SRS transmission within indicated time window(s):

Agreement Confirm the following working assumption with modification made in RAN1#113: Working assumption To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows

Agreement When a LMF requests the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), - the duration of a time window can be configured by one of the following values: - {1, 2, 4, 8, 12} OFDM symbols - {1, 2, 4, 6, 8, 12, 16} slots - FFS: additional values - the number of the time windows can be configured as: - {1, 2, …, 16}

The NRPPa Positioning Information Exchange procedure already enables the LMF to request the serving gNB to configure SRS transmissions for the UE. This is achieved by the LMF sending the POSITIONING INFORMATION REQUEST message including the Requested SRS Transmission Characteristics IE.  However, it is currently not possible for the LMF to indicate time window(s) for the SRS transmissions.  Therefore, the following needs to be added to the POSITIONING INFORMATION REQUEST message:

-	Start of time window (mandatory): Subframe number, slot offset, and symbol index with respect to the SFN initialization time.

-	Duration of time window (mandatory): CHOICE of either a number of consecutive slots or a number of consecutive symbols

-	Periodicity of time window (optional): Defined similar to the PeriodicitySRS IE in the Requested SRS Transmission Characteristics IE.

Proposal 1:	Introduce a Time Window Information for SRS IE in the POSITIONING INFORMATION REQUEST message to indicate the start of the time window, the duration of the time window, and (optionally) the periodicity of the time window for UL SRS transmission.

If proposal 1 is agreeable, then the Time Window Information for SRS IE is proposed to be encoded as follows:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time Window Start |  | 1 |  |  |
| >System Frame Number | M |  | INTEGER(0..1023) |  |
| >Slot Number | M |  | INTEGER(0..79) |  |
| >Symbol Index | M |  | INTEGER(0..12) |  |
| CHOICE Time Window Duration | M |  |  |  |
| >Symbols |  |  |  |  |
| >>Duration in Symbols | M |  | ENUMERATED (1, 2, 4, 8, 12, …) |  |
| >Slots |  |  |  |  |
| >>Duration in Slots | M |  | ENUMERATED (1, 2, 4, 6, 8, 12, 16, …) |  |
| Time Window Type | M |  | ENUMERATED (single, periodic, …) |  |
| Time Window Periodicity | C-ifTimeWindowTypePeriodic |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) [FFS] | Unit: Milli-seconds |
| Number of Periodic Time Windows | C-ifTimeWindowTypePeriodic |  | INTEGER (1..16, …) |  |

Table 1: Encoding of Time Window Information for SRS IE

Proposal 2:	Encode the Time Window Information for SRS IE as shown in Table 1.

Regarding measurement of UL SRS within indicated time window(s), RAN1 has agreed on the following:

Agreement To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows

Agreement When a LMF requests the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s): - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2, …, 16} Additional note: The introduced window is not only for CP positioning from RAN1 understanding.

The NRPPa Measurement procedure already enables the LMF to request the serving/neighboring gNB(s) to measure the UL SRS from the UE. However, it is currently not possible for the LMF to indicate time window(s) for measuring.  According to the RAN1 agreements, the time window(s) for measurement are defined by the following parameters:

-	Start of time window (mandatory): Subframe number and slot offset with respect to the SFN initialization time. We note that the MEASUREMENT REQUEST message already includes these two parameters, so nothing more is needed.

-	Duration of time window (mandatory): Number of consecutive slots.

-	Periodicity of time window (optional): Defined similar to the Measurement Periodicity IE in the MEASUREMENT REQUEST message.

Therefore, the following needs to be added to the MEASUREMENT REQUEST message:

-	Duration of time window (mandatory): number of consecutive slots/symbols

-	Periodicity of time window (optional): defined similar to the Measurement Periodicity IE in the MEASUREMENT REQUEST message.

Proposal 3:	Introduce a Time Window Information for Measurement IE in the MEASUREMENT REQUEST message to indicate the duration of the time window, and (optionally) the periodicity of the time window to measure UL SRS from the UE.

If proposal 3 is agreeable, then the Time Window Information for Measurement IE is proposed to be encoded as follows:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Time Window Duration | M |  |  | Duration of time window with start time given by the System Frame Number IE and Slot Number IE |
| >Slots |  |  |  |  |
| >>Duration in Slots | M |  | ENUMERATED (1, 2, 4, 6, 8, 12, 16, …) |  |
| Time Window Type | M |  | ENUMERATED (single, periodic, …) |  |
| Time Window Periodicity | C-ifTimeWindowTypePeriodic |  | ENUMERATED (160, 320, 640, 1280, 2560, 5120, 10240, …) [FFS] | Unit: Milli-seconds |
| Number of Periodic Time Windows | C-ifTimeWindowTypePeriodic |  | INTEGER (1..16, …) |  |

Table 2: Encoding of Time Window Information for Measurement IE

Proposal 4:	Encode the Time Window Information for Measurement IE as shown in Table 2.

It is observed that the periodicity values of the Measurement Periodicity IE in the MEASUREMENT REQUEST message (for measuring) are not the same as the periodicity values in the PeriodicitySRS IE in the Requested SRS Transmission Characteristics IE (for UL SRS transmission) as shown below:

-	PeriodicitySRS IE: INTEGER (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …)

-	Measurement Periodicity IE: INTEGER (160, 320, 640, 1280, 2560, 5120, 10240, 20480, 40960, 61440, 81920, 368640, 737280, 1843200)

RAN3 can tentatively use the overlapping values (i.e., 160ms to 10240ms) with FFS pending RAN1.

# 3	Conclusions

In this paper, we evaluated the RAN3 specification impacts of Rel-18 positioning accuracy enhancements and proposed the following:

Related to uplink carrier phase positioning:

Proposal 1:	Introduce a Time Window Information for SRS IE in the POSITIONING INFORMATION REQUEST message to indicate the start of the time window, the duration of the time window, and (optionally) the periodicity of the time window for UL SRS transmission.

Proposal 2:	Encode the Time Window Information for SRS IE as shown in Table 1.

Proposal 3:	Introduce a Time Window Information for Measurement IE in the MEASUREMENT REQUEST message to indicate the start of the time window, the duration of the time window, and (optionally) the periodicity of the time window to measure UL SRS from the UE.

Proposal 4:	Encode the Time Window Information for Measurement IE as shown in Table 2.

# References

- RP-231460 Revised WID on Expanded and Improved NR Positioning, Intel Corporation

# Annex	A:	Text Proposal for TS 38.455

Start of modifications

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |
| Time Window Information for SRS | O |  | 9.2.x1 |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifUeTegInfoReqPeriodic | This IE shall be present if the UE TEG Information Request IE is set to the value "periodic". |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >Timing Reporting Granularity Factor Extended | O |  | FFS | FFS |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information for Measurement | O |  | 9.2.x2 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.x1	Time Window Information of SRS

This IE contains the time window(s) when UL SRS transmission is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time Window Start |  | 1 |  |  |
| >System Frame Number | M |  | INTEGER(0..1023) |  |
| >Slot Number | M |  | INTEGER(0..79) |  |
| >Symbol Index | M |  | INTEGER(0..12) |  |
| CHOICE Time Window Duration | M |  |  |  |
| >Symbols |  |  |  |  |
| >>Duration in Symbols | M |  | ENUMERATED (1, 2, 4, 8, 12, …) |  |
| >Slots |  |  |  |  |
| >>Duration in Slots | M |  | ENUMERATED (1, 2, 4, 6, 8, 12, 16, …) |  |
| Time Window Type | M |  | ENUMERATED (single, periodic, …) |  |
| Time Window Periodicity | C-ifTimeWindowTypePeriodic |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) [FFS] | Unit: Milli-seconds |
| Number of Periodic Time Windows | C-ifTimeWindowTypePeriodic |  | INTEGER (1..16, …) |  |

| Condition | Explanation |
| --- | --- |
| ifTimeWindowTypePeriodic | This IE shall be present if the Time Window Type IE is set to the value “periodic”. |

### 9.2.x2	Time Window Information of Measurement

This IE contains the time window(s) when UL SRS measurement is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Time Window Duration | M |  |  | Duration of time window with start time given by the System Frame Number IE and Slot Number IE |
| >Slots |  |  |  |  |
| >>Duration in Slots | M |  | ENUMERATED (1, 2, 4, 6, 8, 12, 16, …) |  |
| Time Window Type | M |  | ENUMERATED (single, periodic, …) |  |
| Time Window Periodicity | C-ifTimeWindowTypePeriodic |  | ENUMERATED (160, 320, 640, 1280, 2560, 5120, 10240, …) [FFS] | Unit: Milli-seconds |
| Number of Periodic Time Windows | C-ifTimeWindowTypePeriodic |  | INTEGER (1..16, …) |  |

| Condition | Explanation |
| --- | --- |
| ifTimeWindowTypePeriodic | This IE shall be present if the Time Window Type IE is set to the value “periodic”. |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<< unchanged texts omitted >>>>>>>>>>>

TimeStamp ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotIndex				TimeStampSlotIndex,

measurementTime			RelativeTime1900	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeStamp-ExtIEs} }	OPTIONAL,

...

}

TimeStamp-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimeStampSlotIndex ::= CHOICE {

sCS-15			INTEGER(0..9),

sCS-30			INTEGER(0..19),

sCS-60			INTEGER(0..39),

sCS-120			INTEGER(0..79),

choice-extension		ProtocolIE-Single-Container { { TimeStampSlotIndex-ExtIEs} }

}

TimeStampSlotIndex-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TimingReportingGranularityFactorExtended ::=INTEGER(0..1)

TimeWindowInformation-Measurement ::= SEQUENCE {

timeWindowDurationMeasurement		TimeWindowDurationMeasurement,

timeWindowType						ENUMERATED {single, periodic, ...},

timeWindowPeriodicityMeasurement	TimeWindowPeriodicityMeasurement		OPTIONAL,

numberOfPeriodicTimeWindows			INTEGER (1..16, ...)					OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeWindowInformation-Measurement-ExtIEs} }	OPTIONAL,

...

}

TimeWindowInformation-Measurement-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimeWindowInformation-SRS ::= SEQUENCE {

timeWindowStartSRS					TimeWindowStartSRS,

timeWindowDurationSRS				TimeWindowDurationSRS,

timeWindowType						ENUMERATED {single, periodic, ...},

timeWindowPeriodicitySRS			TimeWindowPeriodicitySRS				OPTIONAL,

numberOfPeriodicTimeWindows			INTEGER (1..16, ...)					OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeWindowInformation-SRS-ExtIEs} }	OPTIONAL,

...

}

TimeWindowInformation-SRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimeWindowDurationMeasurement ::= CHOICE {

durationSlots		ENUMERATED {1, 2, 4, 6, 8, 12, 16, ...},

choice-extension		ProtocolIE-Single-Container { { TimeWindowDurationMeasurement-ExtIEs} }

}

TimeWindowDurationMeasurement-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TimeWindowDurationSRS ::= CHOICE {

durationSymbols		ENUMERATED {1, 2, 4, 8, 12, ...},

durationSlots		ENUMERATED {1, 2, 4, 6, 8, 12, 16, ...},

choice-extension		ProtocolIE-Single-Container { { TimeWindowDurationSRS-ExtIEs} }

}

TimeWindowDurationSRS-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TimeWindowPeriodicityMeasurement ::= ENUMERATED {ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240, ...}

TimeWindowPeriodicitySRS ::= ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot625, ms1, ms1dot25, ms2, ms2dot5, ms4, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240, ...}

TimeWindowStartSRS ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotNumber				SlotNumber,

symbolIndex				INTEGER (0..12),

iE-Extension			ProtocolExtensionContainer { { TimeWindowStartSRS-ExtIEs} }	OPTIONAL,

...

}

TimeWindowStartSRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

End of modifications