3GPP TSG-RAN WG3 Meeting #121bis	R3-235645

Xiamen, CN, 9 Oct - 13 Oct 2023

Agenda item:	26.3

Source:	CMCC

Title:	Discussion on network slice service continuity

Document for:	Discussion

Introduction

During previous meeting, there is no conclusion achieved on network slice service continuity scenario, but some remaining issues are left as follows.

Stage 2 update can be wait for SA2’s response.

For NGAP, whether to signal both original slice and alternative slice (replaced slice) is still open.

To be continued...

For NGAP, whether to signal both original slice and alternative slice (replaced slice) is still open.

This contribution will continue to discuss the network slice service continuity based on the left issues.

Discussion

Network Slice Replacement

Aiming to support the service continuity scenario, SA2 gives their principles related to network slice replacement.

0) The trigger in the AMF to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM) or based on AM PCF/NSSF notification. The UE capability to support this feature is a prerequisite for the execution of the optimizations in this conclusions.

During the SA2 WI discussion, as the TS 23.501 specified, support of network slice replacement is described as follows:

- 5.15.19 Support of Network Slice Replacement The Network Slice Replacement feature is used to replace an S-NSSAI with an Alternative S-NSSAI when an S-NSSAI becomes unavailable or congested. The Network Slice Replacement may be triggered in the following cases: - If the NSSF detects that an S-NSSAI becomes unavailable or congested (e.g., based on OAM or NWDAF analytics output), it sends network slice availability notification for the S-NSSAI to the AMF. The notification may include an Alternative S-NSSAI which can be used by the AMF to replace the S-NSSAI. The NSSF notifies the AMF when the S-NSSAI is available again. - If the PCF detects that an S-NSSAI becomes unavailable or congested for a UE (e.g., based on OAM or NWDAF analytics output), it sends access and mobility related policy notification to the AMF. The notification may include an Alternative S-NSSAI which can be used by the AMF to replace the S-NSSAI. The PCF notifies the AMF when the S-NSSAI is available again for the UE. - The OAM sends notification to AMF when an S-NSSAI becomes unavailable or congested (and also when this S-NSSAI becomes available again) and provides the Alternative S-NSSAI to AMF. Based on the notification above from NSSF or PCF or OAM, the AMF may determine that an S-NSSAI is to be replaced with Alternative S-NSSAI.

From the SA2 specification, it is found that the Network Slice Replacement feature is used to replace an S-NSSAI with an Alternative S-NSSAI when an S-NSSAI becomes unavailable or congested based on the notification originated from NSSF, PCF and OAM. After the AMF determine that an S-NSSAI is to be replaced with alternative S-NSSAI, PDU session could be established towards the alternative S-NSSAI or retained by using existing PDU session associated with alternative S-NSSAI. Anyway, the SMF shall sends the alternative S-NSSAI to NG-RAN in N2 SM information as specified in SA2 CR.

During a new PDU Session establishment procedure towards an S-NSSAI, - if the UE is provided with the mapping of the S-NSSAI to an Alternative S-NSSAI, the UE provides both the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message. When the AMF receives the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message, or when the AMF receives only the S-NSSAI in PDU Session Establishment message the AMF determines that the S-NSSAI is to be replaced with the Alternative S-NSSAI, the AMF includes both the Alternative S-NSSAI and the S-NSSAI to the SMF. - if the UE is not provided with the mapping of the S-NSSAI to the Alternative S-NSSAI, the UE provides the S-NSSAI in the PDU Session Establishment message. When the AMF determines that the requested S-NSSAI is to be replaced with the Alternative S-NSSAI and if the UE supports Network Slice Replacement, the AMF performs UE Configuration Update procedure to reconfigure the UE with the Alternative S-NSSAI. The AMF continues the PDU Session establishment procedure with the Alternative S-NSSAI and provides both the Alternative S-NSSAI and the S-NSSAI to the SMF. The SMF proceeds with the PDU Session establishment using the Alternative S-NSSAI. The SMF sends the Alternative S-NSSAI to NG-RAN in N2 SM information and to UE in PDU Session Establishment Accept message. For existing PDU Session associated with an S-NSSAI that is replaced with the Alternative S-NSSAI, after the AMF sends mapping of the S-NSSAI to the Alternative S-NSSAI to the supporting UE in UE Configuration Update message, the AMF sends updates to the SMF of the PDU Session, e.g. triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be transferred to Alternative S-NSSAI and includes the Alternative S-NSSAI as follows (see details in clause 4.3.3 of TS 23.502 [3]): - If the SMF determines that the PDU Session needs to be retained (e.g. if the anchor UPF can be reused with the alternative S-NSSAI and SSC mode 1), the SMF sends the Alternative S-NSSAI to the UPF in the N4 message, to the NG-RAN in N2 message and to the supporting UE in PDU Session Modification Command message. - If the SMF determines that the PDU Session needs to be re-established, the SMF sends the Alternative S-NSSAI to the supporting UE either in PDU Session Modification Command if the PDU Session is of SSC mode 3, or in PDU Session Release if the PDU Session is of SSC mode 2 or SSC mode 1, to trigger the re-establishment of the PDU Session. The UE includes both, the S-NSSAI and the Alternative S-NSSAI in the PDU Session Establishment message.

From the highlights described, there will be impact on NGAP associating with PDU session establishment procedure or PDU session modification procedure.

For PDU session establishment procedure, a S-NSSAI IE is already mandatory carried in PDU SESSION RESOURCE SETUP REQUEST message. When SMF sends the alternative S-NSSAI IE to NG-RAN node, we prefer introducing a new S-NSSAI IE to represent the alternative slice. The reason is that we desire to build a future-proof solution. Since CN does not aware the slice resource is unavailable or congested in the NG-RAN node, there is a possibility that NG-RAN node may not support the alternative slice due to temporary overloaded or unavailable. Assumed that the resource for the original slice in CN network is relieved or available again, CN may hope NG-RAN node continue to use original slice, but the original slice is overload in NG-RAN node. If reusing the existing S-NSSAI to indicate orignal slice in PDU Session Resource Setup Request message, the NG-RAN node will fail to establish the PDU session with original slice. If signalling both original slice and alternative slice, NG-RAN node can decide to continue using the alternative slice. In spite of SA2 gives its preference that they do not want to consider the scenaro that slice is unavailable or congested in the NG-RAN node, introducing a new separate IE for alternative S-NSSAI provides a feasible and future-oriented solution to solve the case mentioned above.

Observation 1: Since CN does not aware the slice resource is unavailable or congested in the NG-RAN node, there is a possibility that NG-RAN node may not support the alternative slice due to temporary overloaded or unavailable.

Observation 2: Introducing a new separate IE for alternative S-NSSAI provides a feasible and future-oriented solution to solve the issue that original slice in CN network is relieved or available again while the original slice is overload in NG-RAN node.

For PDU session modification procedure, a new separate IE should also be introduced in PDU SESSION RESOURCE MODIFY REQUEST message to solve the aforementioned issue. The NG-RAN node shall replace the previously S-NSSAI by the received S-NSSAI for the concerned PDU session if only using the S-NSSAI representing orignal slice. There will be a failure for PDU session modification with status quo that the slice resource is unavailable or congested in the NG-RAN node.

Proposal 1: It is proposed to introduce a new slice IE representing the alternative slice in PDU SESSION RESOURCE SETUP REQUEST message and PDU SESSION RESOURCE MODIFY REQUEST message.

Network Slice Replacement during HO

From SA2’s conclusion, it is specified as follows:

During a handover procedure, if an S-NSSAI has to be replaced with an Alternative S-NSSAI, the handover procedure (including any PDU session associated with the S-NSSAI to be replaced) shall continue unaffected by the Network Slice Replacement. Any Network Slice Replacement for the S-NSSAI shall not take place during the handover.

SA2 already agrees that any Network Slice Replacement for the S-NSSAI shall not take place during the handover. From our prespective, RAN3 should follow SA2’s conclusion and endeavour to perform network slice replacement before handover instead of during handover. Before handover, CN can trigger the network slice replacement by establishing new PDU session or modifying the existing PDU session, ensuring the service continuity for the potential upcoming handover.

Observation 3: RAN3 should follow SA2’s conclusion and endeavour to perform network slice replacement before handover instead of during handover.

Assumed that source gNB supports slice1 and slice2 and a candidate gNB supports slice1, a UE with the PDU session under source gNB originally supports slice1. Due to the slice1 in CN is congested or unavailable, CN triggers the network slice replacement for UE1 by transferring associated slice from slice1 to slice2. If UE1 desires handover to the candidate gNB which only supports the original slice1, the handover fails since it does not support the alternative slice. Actually, candidate gNB should have been a potential target for the UE, since UE also supports original slice with a better SLA compared to alternative slice. Thus, in order to avoid the case mentioned above, we think it is required to signal the original slice IE and alternative slice IE respectively in the Handover Request message, helping target NG-RAN node identify the original slice for admission control.

Proposal 2: It is proposed to signal the original slice IE and alternative slice IE respectively in the Handover Request message, helping target NG-RAN node identify the original slice for admission control.

Conclusions

In this paper, we provide our insights on network slice service continuity. The following observations and proposals are listed below:

Observation 1: Since CN does not aware the slice resource is unavailable or congested in the NG-RAN node, there is a possibility that NG-RAN node may not support the alternative slice due to temporary overloaded or unavailable.

Observation 2: Introducing a new separate IE for alternative S-NSSAI provides a feasible and future-oriented solution to solve the issue that original slice in CN network is relieved or available again while the original slice is overload in NG-RAN node.

Observation 3: RAN3 should follow SA2’s conclusion and endeavour to perform network slice replacement before handover instead of during handover.

Proposal 1: It is proposed to introduce a new slice IE representing the alternative slice in PDU SESSION RESOURCE SETUP REQUEST message and PDU SESSION RESOURCE MODIFY REQUEST message.

Proposal 2: It is proposed to signal the original slice IE and alternative slice IE respectively in the Handover Request message, helping target NG-RAN node identify the original slice for admission control.

Reference

- TR 23.700-41, Study on enhancement of network slicing;

- TS 23.501, System architecture for the 5G System (5GS);