3GPP TSG-RAN WG3 #121							R3-23xxxx

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	14.2

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion: CB: # MobilityEhn_L1L2Mobility

Document for:	Discussion

# Introduction

This is the offline summary for the following comeback:

CB: # MobilityEhn_L1L2Mobility

- discuss on the principles for the above issues, and capture the agreements

- Update the corresponding TPs.

- list all the open issues.

(moderator - HW)

Summary of offline discussion in R3-234574

# For the Chairman’s Notes

Issue 1:  L1 measurement configuration and reporting

Proposals for agreement:

Proposal 1: For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU.

Proposal 2: Agree to add the RS configuration in UE Context Setup/Modification in stage2 and stage 3 with FFS.

Proposal 3: The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations.

Open issues:

FFS whether CU needs to request the DU to provide the RS configuration.

Issue 2: TCI state configuration

Open issues:

FFS on the need of an explicit request from CU

Issue 3: Selected beam transfer

Open issues:

FFS on the procedure from CU to target DU to inform the selected beam information and target cell ID.

Option 1: new class 2 procedure with different name.

Option 2: same class 2 procedure

Option 3: class 1.

Down selection among those 3 options  at next meeting.

Issue 4: TA acquisition

Proposals for agreement:

Proposal 1:In inter-DU LTM, the gNB-CU provides the CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure.

Proposal 2:Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2.

Proposal 3:WA: Indicate the source DU ID when request the CFRA resources to the candidate DU.

Proposal 4:WA: to introduce (2)new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

Proposal 5: how the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU).

Proposal 6:Source DU is in charge of checking the TA validity.

Open issues:

FFS: Whether to study the fallback case pending to RAN2/RAN1.

FFS on the following issues:

- Candidate DU sends the source DU ID to CU when providing the TA value

- Any UE Identification information needed during TA info transfer C-DU?

- TA timer associated with the TA value,

- Detected RAP ID,

- Serving gNB-DU ID,

- Candidate cell ID.

- RO information

- SSB information

- Carrier information

- S-DU indicates the valid TA values to target -DU after LTM for subsequent LTM

Issue 5: 	E1 aspects

Option 2: One more option that can minimize the impact on CU-UP when performing LTM is that the CU-UP provides only one UL TNL address which will only be used by the target cell after successful execution of LTM cell switch.

Open issues:

FFS: continue checking on option 2

Issue 6: LTM and L3 handover collision

Proposals for agreement:

Proposal 1:In case that L3 handover is triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority.

Proposal 2:In case that LTM is triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority,

Proposal 3:WA: In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority.

Issue 7: Reference configuration

Open issues:

Prioritize the reference configuration on F1 at next meeting.

List all options and down selection.

TPs assignment to capture the agreements above and the ones in online discussion:

Draft R3-23xxxx was R3-234460 TP for 38401 BLCR, intra-DU LTM procedure update, ZTE.

Draft R3-23xxxx was R3-234074 TP for 38401 BLCR, inter-DU LTM procedure update, Huawei.

Draft R3-23xxxx was R3-233870 TP for F1AP, co-existence between LTM and L3 mobility, NEC.

Draft R3-23xxxx was R3-233906 TP for F1AP, capture all agreements apart from the LTM and L3 handover collision ones, Ericsson

# Discussion

## Issue 1:  L1 measurement configuration and reporting

The progress at online session:

RS configuration sends from C-DU to S-DU via CU or CU request. Need to discuss detail.

Proposed rewording:

For inter-gNB-DU LTM, the RS configuration needs to be sent from C-DU to S-DU via CU.

Open issues:

- F1 signalling for the CU to obtain the RS configuration of candidate DUs:

UE associated signalling (UE Context Setup/Modification) or non-UE associated signalling (F1 SETUP)?

Nokia: Capture in the stage 2proceudre FFS on RS configuration

E: also in stage 3.

Agree to add FFS on stage2 and stage 3 about the RS configuration.

FFS whether CU needs to request the DU to provide.

- F1 signalling to transfer the RS configuration to other candidate DUs and source DU:

Proposal: The gNB-CU uses the UE Context Modification Request procedure to transfer the RS configuration of candidate cells to the source gNB-DU and/or other candidate gNB-DUs to generates the L1 configurations.

## Issue 2: TCI state configuration

The agreements made online:

The gNB-CU uses the UE Context Setup procedure to collect the TCI state configurations of candidate cells in LTM configuration phase for inter-DU LTM.

After LTM configuration, the gNB-CU indicates the collected TCI state configurations of candidate cells to the source gNB-DU and all candidate DUs via UE Context Modification procedure for the UE.

QC: is there a explicit request from CU?

FFS on an explicit request from CU

## Issue 3: Selected beam transfer

After the source DU gets the TCI state information, the next step for the source DU is to choose the proper TCI state index and indicate it to the UE in LTM cell switch command and to the target DU based on the incoming LS from RAN2.

- Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

The progress online:

The LTM Cell Change Notification procedure is reused to transfer the selected beam from the source gNB-DU to the target gNB-DU via gNB-CU. FFS for the detail of selected beam information.

For the part of CU to target DU is FFS of which procedure can be used.

Option 1: introduce the LTM Cell Change Notification procedure from CU to target DU.

Option 2: reuse UE context modification

Liwei: no response is needed from cand

FFS on the procedure from CU to target DU to inform the selected beam information and target cell ID.

Option 1: new class 2 procedure with different name.

Option 2: same class 2 procedure

Option 3: class 1.

Down selection among those 3 options  at next meeting.

## TA acquisition

Progress online:

RAN3 to prioritize specifying support for early TA acquisition “without RAR” and mark the method “with RAR” as FFS.

CU shall send the TA value to the source DU.

CFRA resources configuration to the S-DU:

RAN2 assumes that the RRC RACH configuration and CFRA resource are provided separately to the UE by the source DU, i.e. the RRC RACH configuration is provided in the RRCReconfiguration message and the CFRA resource is provided by the PDCCH order.

In inter-DU LTM, the gNB-CU provides the CFRA resource received from the candidate gNB-DU to the source gNB-DU via UE Context Modification procedure.

RACH resource sharing:

Working assumption: the CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU. need align with RAN2.

WA: Indicate the source DU ID when request the CFRA resources to the candidate DU.

TA information transfer from C-DU to S-DU:

Option 1: To introduce new class 2 non-UE associated F1 signaling to deliver the TA information rom the candidate gNB-DU to the source gNB-DU via gNB-CU.

Option 2: C-DU informs S-DU by reusing UE Context Modification procedure

Option 3: S-DU requests the T-DU to provide the TA info by reusing UE Context Modification procedure

Comments:

WA: to introduce (2) new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU.

TA acquisition failure and fallback solution

Option 1: how the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU).

Option 2: Candidate DU informs source DU that the preamble is not detected

how the source DU detect failure of TA info acquisition is up to implementation (eg., based on timer in source DU).

FFS: Whether to study the fallback case pending to RAN2/RAN1.

Open issues:

- Candidate DU sends the source DU ID to CU when providing the TA value

- Any UE Identification information needed during TA info transfer C-DU?

- TA timer associated with the TA value,

- Detected RAP ID,

- Serving gNB-DU ID,

- Candidate cell ID.

- RO information

- SSB information

- Carrier information

- TA validity checking and maintenance

Source DU is in charge of checking the TA validity.

  - S-DU indicates the valid TA values to target -DU after LTM for subsequent LTM

## E1 aspects

Option 1: One option that can minimize the impact on CU-UP when performing LTM is that the steps 2 and 3 are executed together with steps 12 and 13.

Option 2: One more option that can minimize the impact on CU-UP when performing LTM is that the CU-UP provides only one UL TNL address which will only be used by the target cell after successful execution of LTM cell switch.

continue checking on option 2

## LTM and L3 handover collision

Latest status:

- Option 1: OAM configured priority.

- Option 2: Network decides the priority based on scenario (intra-gNB-CU or inter-gNB-CU) and some assistance information (the measurement results, candidate target cells).

- Option 3: Flexible priority. The handover triggered first take the high priority.

For option 3, The detailed description would be:

- Case 1, L3 handover triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority.

- Case 2, LTM triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority,

- WA: Case 3, LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority. This is to avoid to cancel the LTM command already sent to the UE which seems complex from moderator point of view.

NEC: case 3 needs to be considered. Specify in stage 3.

e///: cause value is needed?

New proposal :

RAN3 considers the following three cases for HO collision discussion, and discusses possible solutions to avoid sending LTM and L3 HO command to UE simultaneously:

- Case 1: The DU receives the L3 HO command while the DU is about to trigger LTM switching the UE to a different target cell.

- Case 2: The CU receives the LTM notification from the DU after the CU has started preparing (inter-CU) L3 HO but has not triggered L3 HO.

- Case 3: The DU receives the L3 HO command while the DU has already triggered the LTM. In this case, the UE may have already switched to another cell, and the L3 HO is no longer applicable.

## Reference configuration

Open issue\: prioritize the reference configuration on F1 at next meeting.

List all options and down selection

Some company proposals from R3-233886:

Proposal 1: A source gNB-DU may generate the lower layer reference configuration for candidate the cells in the candidate DU for initial LTM configuration.

Pending to RAN2?

What is the reference configuration generate by the  source DU?

Proposal 2: The gNB-DU takes the reference configuration into account when generating the lower layer RRC configurations.

Proposal 3: The gNB-CU may provide a reference configuration to the candidate gNB-DU in the UE Context Setup Request message to the gNB-DU.

Proposal 4: The candidate gNB-DU may generate the reference configuration for LTM configuration if not received from the gNB-CU.

Proposal 5: The candidate gNB-DU is not allowed to update the reference configuration if received from the gNB-CU.