3GPP TSG-RAN WG3 Meeting #121	R3-233992

Toulouse, France, 21 – 25 Aug, 2023

Title: 	(TP to Netw_Energy_NR BLCR for TS 38.473, 38.413, 38.470 and 38.300) Network energy saving techniques

Source: 	Huawei

Agenda item:	24.2

Document Type:	Other

# 1. Introduction

In last RAN3 #119b meeting, the following agreements on Network Energy Saving (NES) were reached [1]:

Inter-node beam activation: For inter-node beam activation, the XnAP CELL ACTIVATION procedure, and the F1AP GNB-CU CONFIGURATION UPDATE procedure are reused. - Over Xn interface, the CELL ACTIVATION REQUEST message may include the SSB beam list that is requested to be activated, and the CELL ACTIVATION RESPONSE message may include SSB beam list that are activated. When the receiving NG-RAN node cannot activate any of the SSB beams, it should respond with the CELL ACTIVATION FAILURE message with an appropriate cause value. - FFS if a new cause is needed. The detailed IE name/encoding can be further refined. - Over F1 interface, the GNB-CU CONFIGURATION UPDATE message may include the SSB beam list that is requested to be activated, and the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE may include SSB beam list that are activated. In case the gNB-DU cannot activate any of the requested SSB beams, it should respond with the GNB-CU CONFIGURATION UPDATE FAILURE message with an appropriate cause value. - FFS if a new cause is needed. The detailed IE name/encoding can be further refined. To be continued at the next meeting (by contribution driven): - Whether the DU sends it preferred beam activation/decision of its own beams to the CU? - Whether the new beams deactivation cause (‘energy saving’) is needed? - The timer indicated how long the SSBs are activated? Enhancements on restricting paging in a limited area: It’s up to gNB’s implementation to decide to which UEs should apply the paging enhancement technique. The paging enhancement technique is applicable for UEs in RRC inactive state. Introduce the recommended SSB beam list in the F1AP paging message. Introduce a list of last few served SSB beam s/recommended SSB beam list as paging assistance information to the gNB-CU in UE CONTEXT RELEASE COMPLETE message over F1AP. Send a LS to RAN2 asking feedback containing the agreements for RRC inactive UE, and to SA2 asking feedback for RRC idle UE. The above agreements can be revisited based on reply from RAN2.

In last RAN3 #120 meeting, the following agreements and remaining issues on NES were captured in [2]:

Cell DTX/DRX: WA: Support the exchange of the Cell DTX/DRX configuration over Xn. The detail infor of the Cell DTX/DRX configuration is pending to RAN1 and RAN2. Introduce the new cause value “SSB not Available”? Resolve the FFS(es) in the BLCR including the criticality of the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message, the maximum number of the SSB areas etc.

In this paper, we continue to discuss the remaining issues for NES.

# 2. Discussion

## 2.1 Inter-node beam activation

- FFS(es) in the F1AP BLCR

- In the BLCR, the SSBs within the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message is set to have criticality “reject”. And there is an editor’s note as follows.

- Editor’s Note: The SSBs within the cell to be Activated List IE may be further refined (e.g., the assigned criticality).

- At first glance, it seems that there is no need to fail the whole procedure due to not support the beam activation feature. But there were some discussions at the previous meeting. An example for Rel-18 gNB-CU and legacy gNB-DU over F1 interface is given as follows.

- The new gNB-CU requests to request SSB activation.

- The legacy gNB-DU will ignore the SSBs within the cell to be Activated List, and activate the whole cell and beams instead.

- The new gNB-CU consider that no beams are activated, since it does not receive the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, and notify the neighbour NG-RAN node.

- To resolve the different version issue, a new cell failed activated list IE can be introduced in the response message. It seems simple to keep the criticality of the SSBs within the cell to be Activated List IE as “reject”.

- By contrast, for XnAP, a new dedicated NR Cells and SSBs List IE is introduced in the Cell Activation Request message with criticality “ignore”. Then the legacy peer node can ignore it, and not feedback the SSBs activated list. Hence no change is needed.

For F1AP, remove the editor’s note and keep the criticality of the SSBs within the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message to “reject”.

- There are also FFS regarding the maximum value of SSB areas as follows. Given the maximum SSB values can be different over different frequency band. The simple way is to remove the FFS and keep the value equal to 64.

| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. FFS |
| --- | --- |

Remove the FFS on the maximum number of SSB beam in the BLCR for F1AP.

- Cause value for SSB activation failure

In last RAN3#120 meeting, the new case value for SSB beam activation failure was discussed. Some companies thought a new cause value “SSB not Available” could be introduced similar to the existing cause value “Cell not Available” while others thought the case for this value should be clarified.

We envision that a new cause value is beneficial, as it could be used to distinguish the failure due to “SSB not available” from other failures. The peer node may take proper actions. For example, the peer node may request the activation of other SSB beams in the cell if the cause is “SSB not available” instead when necessary.

Introduce a new cause value “SSB not Available” or more specifically “SSB activation not available” in XnAP and F1AP to indicate that none of the requested SSB beam(s) can be successfully activated.

- Beams deactivation: Whether to introduce a cause for beams deactivation (e.g. ‘energy saving’)

In current F1AP specification, the DU will send the SSB Coverage State to CU (through Coverage Modification Notification IE in GNB-DU CONFIGURATION UPDATE message) when modifying the coverage of an SSB beam. Similarly, over Xn, a NG-RAN node will send the SSB Coverage State to the peer node (through Coverage Modification List IE in NG-RAN NODE CONFIGURATION UPDATE message) when modifying the coverage of an SSB beam. In both of the above cases, if the SSB beam is deactivated, the corresponding SSB Coverage State is set to “0”.

The modification of SSB beam coverage could be due to e.g. CCO or NES. It would be beneficial to indicate the detailed cause over the network interfaces. For example, if the SSB beam deactivation (i.e. SSB Coverage State = “0”) is due to NES, the peer node may request the activation of this SSB beam when the its load becomes high; while if the SSB beam deactivation is due to CCO (high interference), the peer node will not request the activation of this SSB beam, as this will increase the interference and reduce the overall capacity. Hence, a cause value e.g. “network energy saving” is beneficial.

To indicate the “network energy saving” cause, there are two signalling options in F1AP.

- Option 1: updating the Coverage Modification Notification to include the new cause.  This cause could only be used for the deactivation case (i.e. SSB Coverage State = “0”), but not for other cases (i.e. SSB Coverage State = 1, 2,…).

- Option 2: introducing a new IE containing the new cause value and a list SSBs suggested to be deactivated.

Option 1 is preferred since it is simple to update the exiting IE, and notify the cause in one unified IE.

While for XnAP, the existing Coverage Modification Cause can be enhanced to indicate “network energy saving” for SSB beam deactivation.

Introduce a new cause in the Coverage Modification Notification for F1AP and in the Coverage Modification List for XnAP to indicate “network energy saving” for SSB beam deactivation.

- Timer for SSB activation

There were some discussions whether to introduce a timer to indicate the minimum activation time that an SSB beam is requested to be activated. In the legacy specification, it has been specified that for the inter-system energy saving case the “minimum activation time” is used and the configuration of the “minimum activation time” of a cell may be done by O&M or be left to node implementation, as described in TS 38.300 as follows.

#### 15.4.2.2 Inter-system energy saving The solution builds upon the possibility for the NG-RAN node owning a capacity booster cell to autonomously decide to switch-off such cell to dormant state. The decision is typically based on cell load information, consistently with configured information. The switch-off decision may also be taken by O&M. The NG-RAN node indicates the switch-off action to the eNB over NG interface and S1 interface. The NG-RAN node could also indicate the switch-on action to the eNB over NG interface and S1 interface. The eNB providing basic coverage may request a NG-RAN node's cell re-activation based on its own cell load information or neighbour cell load information, the switch-on decision may also be taken by O&M. The eNB requests a NG-RAN node's cell re-activation and receives the NG-RAN node's cell re-activation reply from the NG-RAN node over the S1 interface and NG interface. Upon reception of the re-activation request, the NG-RAN node's cell should remain switched on at least until expiration of the minimum activation time. The minimum activation time may be configured by O&M or be left to the NG-RAN node's implementation.

Following the same logic, the “minimum activation time” for SSB beams, if needed, can be configured by O&M or be left to the implementation, and there is no need to introduce a timer in the request message.

No need to signal the timer indicating the minimum activation time of SSB beams.

- Beam activation: Whether the DU sends its own preferred beam activation to the CU

The following beam activation procedure was proposed and discussed in last meetings:

  - Step 1: the DU informs the CU of its preferred SSB beam activation decision;

  - Step 2: the CU makes beam activation decision.

Such procedure allows the CU to decide which SSB beam(s) to be activated. In current specifications, it is the DU that is responsible of deciding the SSB configuration and informing the CU of the configurations, e.g. SSB Positions In Burst and SSB Coverage Modification List. Following the same logic, it is proper to let the DU, but not the CU, to decide which SSB beam(s) to be activated.

The indication of preferred beam activation from DU to CU is not supported.

## 2.2 Paging enhancements

SA2 replied the LS on the enhancements to restricting paging in a limited area in S2-2307984/R3-233729, as follows:

SA2 thanks RAN3 for the LS on the enhancements to restricting paging in a limited area. Q2: RAN3 would respectfully like to ask SA2 if there are any issues from CN perspective with this mechanism. Answer: SA2 does not see any showstoppers from CN perspective for the enhancements to restrict paging in a limited area for UEs in RRC IDLE. SA2 requests RAN3 to inform SA2 when the related CRs are agreed in order to proceed with alignment CRs in specifications under SA2 control. SA2 would also like to validate the following assumption: If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how paging escalation happen will be controlled by the AMF based on existing procedures. and ask the following questions: Q1: SA2 understands that the beam-based paging is most efficient towards low mobility and static UEs. How is the gNB expected to determine whether the UE is static or not across multiple RRC connections from the same UE considering TS 33.501 contains requirements on 5G-S-TMSI reallocation? Q2: SA2 would like to ask about the validity condition of the potential “List of recommended beams” container that is stored in the AMF. What is the AMF condition to delete the stored "List of recommended beams"?

According to SA2’s reply, there is no issues from CN perspective to restrict paging in a limited area for UEs in RRC idle. We understand the CN paging enhancements for idle UEs is also beneficial for NES. The following provides our views to the questions to SA2.

Reply to SA2

About the assumption validation, we expect the AMF can do the same paging escalation based on the existing mechanisms. That is, when the AMF does not receive UE service request, it can decide to escalate and send paging messages to more TAs based on own its implementation. No new AMF mechanism is expected.

About the Q1, RAN3 has already made the following agreement.

- It’s up to gNB’s implementation to decide to which UEs should apply the paging enhancement technique.

There are many ways for the gNB to determine the UE mobility state. For example, according to TS 38.413, the Expected UE Mobility IE included in the CN Assisted RAN Parameters Tuning and/or the Core Network Assistance Information for RRC INACTIVE is provided by CN to indicate whether the UE is expected to be stationary or mobile. Another example is that the NG-RAN make such decision based on the UE cell/beam measurement results etc. If the UE endures multiple RRC connections, the NG-RAN again, can decide the UE mobility state based on its implementation.

About Q2, we think that the AMF can decide the validity condition of the potential “List of recommended beams”, e.g., the same as the handling of the existing Recommended Cells for Paging, Last Visited Cell Information in TS 38.413. One possible way is that when the UE moves to the connected state, the AMF can delete the stored "List of recommended beams" or under other conditions. And this is left to the AMF implementation.

Agree the draft reply LS to SA2 in annex 5. Specifically,

- About the assumption validation, RAN3 confirms and expects the AMF can do the paging escalation based on the existing mechanisms without new techniques. It is up to the AMF decision to escalate and send paging messages over more TAs when its attempts fail.

- for Q1, RAN3 has made the agreement that “It’s up to gNB’s implementation to decide to which UEs should apply the paging enhancement technique”. This means, the gNB can make this decision based on the Expected UE Mobility IE or UE measurement results. RAN3 will not specify it in RAN specification.

- for Q2, RAN3 considers that the AMF itself can decide the validity condition of the potential “List of recommended beams”, e.g., the same as the handling of the existing Recommended Cells for Paging, Last Visited Cell Information in TS 38.413. It is up to the AMF implementation to determine to delete it when the UE moves to the connected state, or under other conditions.

CN paging

In the legacy UE Context Release/Suspend/Resume procedures, the NG-RAN node can inform the AMF of a list of Recommended Cells for Paging, including the following information: 1) a list of visited and non-visited cells, and 2) the time a UE stayed in a visited cell. When a CN paging is initiated, the AMF sends the list of Recommended Cells for Paging to the NG-RAN node, and the NG-RAN node could page UE in the recommended SSBs only.

In order to support the SSB level paging enhancement, i.e., allow the exchange of Recommended SSBs for Paging over NG. Specifically, Information on Recommended Cells and RAN Nodes for Paging and Assistance Data for Paging could include the info of Recommended SSBs, including: 1) a list of visited and non-visited SSBs, and 2) the time a UE stayed in a visited SSB.

In the disaggregated gNB case, upon the reception of the recommended SSBs for Paging from the AMF, the gNB-CU could inform the gNB-DU of the paging SSBs in PAGING, which includes: a list of paging SSBs.

To support CN paging enhancement, the information of recommended SSBs for Paging could be transferred over NG. Specifically,

- Over NG, Information on Recommended Cells and RAN Nodes for Paging and Assistance Data for Paging to include 1) a list of visited and non-visited SSBs, and 2) the time a UE stayed in a visited SSB.

To enable the NG-RAN node to get knowledge of the information of the visited SSBs, the UE History Information transferred over NG and Xn could be extended to include the Last Visited SSB Information, which is similar with the legacy Last Visited Cell Information.

RAN3 to discuss whether the last visited SSB information could be transferred over NG and Xn. Specifically, the stage 3 NG and Xn impacts include:

- UE History Info to include 1) a list of visited SSBs, and 2) the time UE stayed in a visited SSB.

## 2.3 Cell DTX/DRX

RAN2 has agreed that Cell DTX/DRX could be periodic, and the Cell DTX/DRX configuration could contain the periodicity, start slot/offset, on duration. The specific values of cell DTX/DRX configuration are under discussion in RAN2. Given that this Cell DTX/DRX is lower layer per cell information, it can be the DU to decide this parameter.

In the CU/DU split architecture, the Cell DTX/DRX parameters including the periodicity, start slot/offset, on duration are decided by the gNB-DU.

In addition, the working assumption was made at the previous meeting.

- WA: Support the exchange of the Cell DTX/DRX configuration over Xn

In RAN1#113 meeting, the group common L1 signalling using PDCCH for cell DTX/DRX activation and deactivation was agreed in R1-2306246. We think the necessity and feasibility of the exchange of such info over network interfaces highly depend on the time granularity of cell DTX/DRX parameters (periodicity and on duration) and how dynamically the cell DTX/DRX configuration could be activated/deactivated. If the activation/deactivation is frequent, it would be problematic to timely notify the activation/ deactivation info to peer node. If the group common L1 signalling is used to lower signalling overhead, and is not dynamic, e.g., in terms of hundreds of microseconds, then the exchange is needed;

Hence, we think it will be finally up to the operator policy and detailed RRM algorithm. RAN3 can be pending on the detailed Cell DTX/DRX parameters specified by other groups.

Turn the WA to agreement to exchange the Cell DTX/DRX configuration per cell over Xn and F1 interface. How to use it can be left to the operator policy.

## 2.4 NES mode

For cell selection/re-selection and CHO mechanisms discussed in RAN2, a cell may enter the “NES mode”. In RAN2#121b and RAN2#122 meetings, the following agreements were made:

RAN2#121b

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

RAN2#122

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

Agreements

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

When a cell enters or leaves the “NES mode”, it may need to indicate or update its current NES mode to neighbour nodes, which would be helpful for target cell selection during HO. RAN3 can further address it when the exact NES mode is specified by other groups.

The NES mode exchange over network interfaces are pending to other groups.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- For F1AP, remove the editor’s note and keep the criticality of the SSBs within the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message to “reject”.

- Remove the FFS on the maximum number of SSB beam in the BLCR for F1AP.

- Introduce a new cause value “SSB not Available” or more specifically “SSB activation not available” in XnAP and F1AP to indicate that none of the requested SSB beam(s) can be successfully activated.

- Introduce a new cause in the Coverage Modification Notification for F1AP and in the Coverage Modification List for XnAP to indicate “network energy saving” for SSB beam deactivation.

- No need to signal the timer indicating the minimum activation time of SSB beams.

- The indication of preferred beam activation from DU to CU is not supported.

Agree the draft reply LS to SA2 in annex 5. Specifically,

- About the assumption validation, RAN3 confirms and expects the AMF can do the paging escalation based on the existing mechanisms without new techniques. It is up to the AMF decision to escalate and send paging messages over more TAs when its attempts fail.

- for Q1, RAN3 has made the agreement that “It’s up to gNB’s implementation to decide to which UEs should apply the paging enhancement technique”. This means, the gNB can make this decision based on the Expected UE Mobility IE or UE measurement results. RAN3 will not specify it in RAN specification.

- for Q2, RAN3 considers that the AMF itself can decide the validity condition of the potential “List of recommended beams”, e.g., the same as the handling of the existing Recommended Cells for Paging, Last Visited Cell Information in TS 38.413. It is up to the AMF implementation to determine to delete it when the UE moves to the connected state, or under other conditions.

To support CN paging enhancement, the information of recommended SSBs for Paging could be transferred over NG. Specifically,

- Over NG, Information on Recommended Cells and RAN Nodes for Paging and Assistance Data for Paging to include 1) a list of visited and non-visited SSBs, and 2) the time a UE stayed in a visited SSB.

- RAN3 to discuss whether the last visited SSB information could be transferred over NG and Xn. Specifically, the stage 3 NG and Xn impacts include:

- UE History Info to include 1) a list of visited SSBs, and 2) the time UE stayed in a visited SSB.

In the CU/DU split architecture, the Cell DTX/DRX parameters including the periodicity, start slot/offset, on duration are decided by the gNB-DU.

Turn the WA to agreement to exchange the Cell DTX/DRX configuration per cell over Xn and F1 interface. How to use it can be left to the operator policy.

The NES mode exchange over network interfaces are pending to other groups.

- The corresponding TP to TS 38.300, TS 38.470, TS 38.473 and TS 38.413 and the draft LS are provided in Annexes.

# 4. Reference

- 3GPP RAN3#119b Chairman’s notes (RAN3_119bis-e_agenda_20230426_EOM.doc)

- 3GPP RAN3#120 Chairman’s notes ( RAN3_120_agenda_20230530_EOM1.doc)

# Annex 1: TP to BLCR for 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 15.4	Support for Energy Saving

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 15.4.2	Solution description

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 15.4.2.a		Inter-node Beam Deactivation and Activation

This mechanism allows an NG-RAN node to request a neighbouring NG-RAN node to switch on certain SSB beams which have been deactivated e.g., in the capacity layer. With this mechanism, the NG-RAN node (e.g., in the coverage-layer) can request the re-activation of SSB beams that are deactivated before.

### 15.4.2.b	Restricting Paging in a limited area

This mechanism allows an NG-RAN node to page e.g. stationary UEs in certain SSB(s) instead of all the SSBs within a cell. This mechanism is applicable for UEs in both RRC inactive and idle state. It’s up to gNB’s implementation to decide to which UEs should apply the paging enhancement mechanism.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 2: TP to BLCR for 38.470

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 5.1	General

The following clauses describe the functions supported over F1-C and F1-U.

## 5.2	F1-C functions

### 5.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and the gNB-CU.

The F1 setup function allows to exchange application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface, and exchange the intended TDD DL-UL configuration originating from the gNB-DU or destined to the gNB-DU. The F1 setup is initiated by the gNB-DU.

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, may activate or deactivate cells, or activate SSBs of cells. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. With the gNB-CU Configuration Update function, energy saving with cell activation/deactivation can be supported as defined in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 5.2.5	Paging function

The gNB-CU is responsible for filtering target cells for paging based on the UE Radio Capability for Paging. The gNB-CU may filter SSBs of the target cells for paging for network power saving.

The gNB-DU is responsible for transmitting the paging information according to the scheduling parameters provided. The gNB-DU also takes the UE paging capability into account for paging when provided by the gNB-CU.

The gNB-CU provides paging information to enable the gNB-DU to calculate the exact PH, if the eDRX is configured, PO and PF. The gNB-CU determines the PA. The gNB-DU consolidates all the paging records for a particular PH, PO, PF and PA, and encodes the final RRC message and broadcasts the paging message on the respective PH, PO, PF in the PA.

The paging function also supports CN controlled subgrouping paging for UE Power Saving.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 3: TP to 38.473 (based on BLCR R3-233804)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the NR PRACH Configuration List IE is included in the Served Cell Information IE contained in the F1 SETUP REQUEST message, the gNB-CU may store the information, and forward it to other RAN nodes for RACH optimisation. If the L139 Info IE included in the NR PRACH Configuration List IE is present, it shall contain the Root Sequence Index IE.

If the RedCap Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the Cell DTX/DRX configuration IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may take this IE into account for network energy saving.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization or Network Energy Saving.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the Cell DTX/DRX configuration IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may take this IE into account for network energy saving.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [4].

If the Neighbour Cell Information List IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall use the received information for Cross Link Interference management and/or NR-DC power coordination and/or network energy saving. The gNB-DU shall consider the received Neighbour Cell Information List IE content valid until reception of an update of the IE for the same cell(s). If the Intended TDD DL-UL Configuration NR IE and Cell DTX/DRX configuration IE are absent from the Neighbour Cell Information List IE, whereas the corresponding NR CGI IE is present, the receiving gNB-DU shall remove the previously stored Neighbour Cell Information IE corresponding to the NR CGI. If the Cell DTX/DRX configuration IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall, if supported, use the received information to determine its cell DTX/DRX configuration for network energy saving.

If the GNB-CU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List |  | 0 .. < maxnoofSSBAreas> |  | List of SSB beams within the cell requested to be activated | EACH | reject |
| >>>SSB Index | M |  | INTEGER (0..63) | Identifier of SSB beam requested to be activated | - |  |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >> E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>> E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| >>Cell DTX/DRX configuration | O |  | 9.3.1.aaa |  | YES | ignore |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | Ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the F1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, RL failure-RLC, Unknown or already allocated gNB-CU UE F1AP ID, Unknown or already allocated gNB-DU UE F1AP ID, Unknown or inconsistent pair of UE F1AP ID, Interaction with other procedure, Not supported QCI Value, Action Desirable for Radio Reasons, No Radio Resources Available, Procedure cancelled, Normal Release, ..., Cell not available, RL failure-others, UE rejection, Resources not available for the slice(s), AMF initiated abnormal release, Release due to Pre-Emption, PLMN not served by the gNB-CU, Multiple DRB ID Instances, Unknown DRB ID, Multiple BH RLC CH ID Instances, Unknown BH RLC CH ID, CHO-CPC resources to be changed, NPN not supported, NPN access denied, gNB-CU Cell Capacity Exceeded, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, Unknown BAP address, Unknown BAP routing ID, Insufficient UE Capabilities, SCG activation deactivation failure, SCG deactivation failure due to data transmission, Requested Item not Supported on Time, Unknown or already allocated gNB-CU MBS F1AP ID, Unknown or already allocated gNB-DU MBS F1AP ID, Unknown or inconsistent pair of MBS F1AP ID, Unknown or inconsistent MRB ID, TAT-SDT expiry, SSB not Available) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, Transport Resource Unavailable, ... , Unknown TNL address for IAB, Unknown UP TNL information for IAB) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| RL Failure-RLC | The action is due to an RL failure caused by exceeding the maximum number of ARQ retransmissions. |
| Unknown or already allocated gNB-CU UE F1AP ID | The action failed because the gNB-CU UE F1AP ID is either unknown, or (for a first message received at the gNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated gNB-DU UE F1AP ID | The action failed because the gNB-DU UE F1AP ID is either unknown, or (for a first message received at the gNB-DU) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE F1AP ID | The action failed because both UE F1AP IDs are unknown, or are known but do not define a single UE context. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| Not supported QCI Value | The action failed because the requested QCI is not supported. |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. |
| Normal Release | The action is due to a normal release of the UE (e.g. because of mobility) and does not indicate an error. |
| Cell Not Available | The action failed due to no cell available in the requested node. |
| RL Failure-others | The action is due to an RL failure caused by other radio link failures than exceeding the maximum number of ARQ retransmissions. |
| UE rejection | The action is due to gNB-CU’s rejection of a UE access request. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| AMF initiated abnormal release | The release is triggered by an error in the AMF or in the NAS layer. |
| Release due to Pre-Emption | Release is initiated due to pre-emption. |
| PLMN not served by the gNB-CU | The PLMN indicated by the UE is not served by the gNB-CU. |
| Multiple DRB ID Instances | The action failed because multiple instances of the same DRB had been provided. |
| Unknown DRB ID | The action failed because the DRB ID is unknow. |
| Multiple BH RLC CH ID Instances | The action failed because multiple instances of the same BH RLC CH ID had been provided. This cause value is only applicable to IAB. |
| Unknown BH RLC CH ID | The action failed because the BH RLC CH ID is unknown. This cause value is only applicable to IAB. |
| CHO-CPC resources to be changed | The gNB-DU requires gNB-CU to replace, i.e. overwrite the configuration of indicated candidate target cell. |
| NPN not supported | The action fails because the indicated SNPN is not supported in the node. |
| NPN access denied | The action is due to rejection of a UE access request for NPN. |
| gNB-CU Cell Capacity Exceeded | The number of cells requested to be added was exceeding maximum cell capacity in the gNB-CU. |
| Report Characteristics Empty | The action failed because there is no measurement object in the report characteristics. |
| Existing Measurement ID | The action failed because the measurement ID is already used. |
| Measurement Temporarily not Available | The gNB-DU can temporarily not provide the requested measurement object. |
| Measurement not Supported For The Object | At least one of the concerned object(s) does not support the requested measurement. |
| Unknown BAP address | The action failed because the BAP address is unknown. This cause value is only applicable to IAB. |
| Unknown BAP routing ID | The action failed because the BAP routing ID is unknown. This cause value is only applicable to IAB. |
| Insufficient UE Capabilities | The setup can’t proceed due to insufficient UE capabilities. |
| SCG activation deactivation failure | The action failed due to rejection of the SCG activation deactivation request. |
| SCG deactivation failure due to data transmission | The SCG deactivation failure due to ongoing or arriving data transmission. |
| Requested Item not Supported on Time | The gNB-DU is unable to provide the measurement results on time. |
| Unknown or already allocated gNB-CU MBS F1AP ID | The action failed because the gNB-CU MBS F1AP ID is either unknown, or (for a first message received at the gNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated gNB-DU MBS F1AP ID | The action failed because the gNB-DU MBS F1AP ID is either unknown, or (for a first message received at the gNB-DU) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of MBS F1AP ID | The action failed because both MBS F1AP IDs are unknown, or are known but do not define a single MBS context. |
| Unknown or inconsistent MRB ID | The action failed because the MRB ID is unknown or inconsistent. |
| TAT-SDT expiry | The UE context release is requested from the gNB-DU due to the expiry of the Timing Alignment timer for CG-SDT. |
| SSB not Available | The action failed due to no SSB available in the requested node. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| CHOICE NR-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  | - |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. | - |  |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. | - |  |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. | - |  |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. | - |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>>NR FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | YES | ignore |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the ServingCellConfigCommon IE as defined in TS 38.331 [8] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >NR-U |  |  |  |  | YES | ignore |
| >>NR-U Channel Info List |  | 1..< maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>NR-U Channel Info Item |  |  |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Index to uniquely identify the part of the NR-U Channel Bandwidth consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum. Value 1 represents the first part of the NR-U Channel Bandwidth on which a channel access procedure is performed. Value 2 represents the second part of the NR-U Channel Bandwidth on which a channel access procedure is performed, and so on. | - |  |
| >>>>NR-U ARFCN | M |  | INTEGER (0.. maxNRARFCN) | It represents the centre frequency of the NR-U Channel Bandwidth for NR bands restricted to operation with shared spectrum channel access, as defined in TS 37.213 [46]. Allowed values are specified in TS 38.101-1 [26] in Table 5.4.2.3-2, Table 5.4.2.3-3 and Table 5.4.2.3-4. | - |  |
| >>>>NR-U Channel Bandwidth | M |  | ENUMERATED (10MHz, 20MHz, 40MHz, 60 MHz, 80 MHz,. …) |  | - |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Includes the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. | - |  |
| RANAC | O |  | RAN Area Code 9.3.1.57 |  | YES | ignore |
| Extended Served PLMNs List |  | 0..1 |  | This is included if more than 6 Served PLMNs is to be signalled. | YES | ignore |
| >Extended Served PLMNs Item |  | 1 ..<maxnoofExtendedBPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >>TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| Cell Direction | O |  | 9.3.1.78 |  | YES | ignore |
| Broadcast PLMN Identity Info List |  | 0..<maxnoofBPLMNsNR> |  | This IE corresponds to the PLMN-IdentityInfoList IE and the NPN-IdentityInfoList IE (if available) in SIB1 as specified in TS 38.331 [8]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE and NPN identities and associated information contained in the NPN-IdentityInfoList IE (if available) are included and provided in the same order as broadcast in SIB1. NOTE: In case of NPN-only cell, the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are not included. | YES | ignore |
| Cell Type | O |  | 9.3.1.87 |  | YES | ignore |
| >PLMN Identity List | M |  | Available PLMN List 9.3.1.65 | Broadcast PLMN IDs in SIB1 associated to the NR Cell Identity IE | - |  |
| >Extended PLMN Identity List | O |  | Extended Available PLMN List 9.3.1.76 |  | - |  |
| >5GS-TAC | O |  | OCTET STRING (3) |  | - |  |
| >NR Cell Identity | M |  | BIT STRING (36) |  | - |  |
| >RANAC | O |  | RAN Area Code 9.3.1.57 |  | - |  |
| >Configured TAC Indication | O |  | 9.3.1.87a | NOTE: This IE is associated with the 5GS TAC in the Broadcast PLMN Identity Info List IE | YES | ignore |
| >NPN Broadcast Information | O |  | 9.3.1.157 | If this IE is included the content of the PLMN Identity List IE and Extended PLMN Identity List IE if present in the Broadcast PLMN Identity Info List IE is ignored. | YES | reject |
| Configured TAC Indication | O |  | 9.3.1.87a | NOTE: This IE is associated with the 5GS TAC on top-level of the Served Cell Information IE | YES | ignore |
| Aggressor gNB Set ID | O |  | 9.3.1.93 | This IE indicates the associated aggressor gNB Set ID of the cell | YES | ignore |
| Victim gNB Set ID | O |  | 9.3.1.93 | This IE indicates the associated Victim gNB Set ID of the cell | YES | ignore |
| IAB Info IAB-DU | O |  | 9.3.1.106 |  | YES | ignore |
| SSB Positions In Burst | O |  | 9.3.1.138 |  | YES | ignore |
| NR PRACH Configuration | O |  | 9.3.1.139 |  | YES | ignore |
| SFN Offset | O |  | 9.3.1.208 |  | YES | ignore |
| NPN Broadcast Information | O |  | 9.3.1.157 |  | YES | reject |
| Supported MBS FSA ID List |  | 0..<maxnoofMBSFSAs> |  | Shall contain all MBS Frequency Selection Area Identities associated with the NR CGI. | YES | ignore |
| >MBS Frequency Selection Area Identity | M |  | OCTET STRING(3) |  | – |  |
| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [8]. First bit = 1Rx, second bit = 2Rx, third bit = halfDuplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |
| Cell DTX/DRX configuration | O |  | 9.3.1.aaa |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  | - |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | - |  |
| >>SSB Coverage Modification List |  | 0..1 |  |  | - |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>SSB Inactivation Cause | O |  | ENUMERATED (network energy saving, …) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.aaa	Cell DTX/DRX configuration

This IE contains the Cell DTX/DRX configuration information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell DTX configuration [FFS] |  | 0..1 |  |  |
| >Periodicity | M |  |  |  |
| >Start offset | M |  |  |  |
| >On duration | M |  |  |  |
| Cell DRX configuration [FFS] |  | 0..1 |  |  |
| >Periodicity | M |  |  |  |
| >Start offset | M |  |  |  |
| >On duration | M |  |  |  |

Editor’s Note: Encoding of IEs is to be formulated, e.g., the presence, the formats etc.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

CellDTXDRXConfiguration,

SSBInactivationCause

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-CellDTXDRXConfiguration,

id-SSBInactivationCause,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

CauseRadioNetwork ::= ENUMERATED {

unspecified,

rl-failure-rlc,

unknown-or-already-allocated-gnb-cu-ue-f1ap-id,

unknown-or-already-allocated-gnb-du-ue-f1ap-id,

unknown-or-inconsistent-pair-of-ue-f1ap-id,

interaction-with-other-procedure,

not-supported-qci-Value,

action-desirable-for-radio-reasons,

no-radio-resources-available,

procedure-cancelled,

normal-release,

...,

cell-not-available,

rl-failure-others,

ue-rejection,

resources-not-available-for-the-slice,

amf-initiated-abnormal-release,

release-due-to-pre-emption,

plmn-not-served-by-the-gNB-CU,

multiple-drb-id-instances,

unknown-drb-id,

multiple-bh-rlc-ch-id-instances,

unknown-bh-rlc-ch-id,

cho-cpc-resources-tobechanged,

nPN-not-supported,

nPN-access-denied,

gNB-CU-Cell-Capacity-Exceeded,

report-characteristics-empty,

existing-measurement-ID,

measurement-temporarily-not-available,

measurement-not-supported-for-the-object,

unknown-bh-address,

unknown-bap-routing-id,

insufficient-ue-capabilities,

scg-activation-deactivation-failure,

scg-deactivation-failure-due-to-data-transmission,

requested-item-not-supported-on-time,

unknown-or-already-allocated-gNB-CU-MBS-F1AP-ID,

unknown-or-already-allocated-gNB-DU-MBS-F1AP-ID,

unknown-or-inconsistent-pair-of-MBS-F1AP-ID,

unknown-or-inconsistent-MRB-ID,

tat-sdt-expiry,

ssb-not-available

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

CellGroupConfig ::= OCTET STRING

CellCapacityClassValue ::= INTEGER (1..100,...)

CellDTXDRXConfiguration	::= (FFS)

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Cells-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-gNB-CUSystemInformation			CRITICALITY reject	EXTENSION GNB-CUSystemInformation				PRESENCE optional }|

{ ID id-AvailablePLMNList				CRITICALITY ignore	EXTENSION AvailablePLMNList						PRESENCE optional }|

{ ID id-ExtendedAvailablePLMN-List		CRITICALITY ignore	EXTENSION ExtendedAvailablePLMN-List			PRESENCE optional }|

{ ID id-IAB-Info-IAB-donor-CU			CRITICALITY ignore	EXTENSION IAB-Info-IAB-donor-CU					PRESENCE optional}|

{ ID id-AvailableSNPN-ID-List			CRITICALITY ignore	EXTENSION AvailableSNPN-ID-List					PRESENCE optional }|

{ ID id-MBS-Broadcast-NeighbourCellList	CRITICALITY ignore	EXTENSION MBS-Broadcast-NeighbourCellList	PRESENCE optional }|

{ ID id-SSBs-toBeActivated-List			CRITICALITY reject	EXTENSION SSBs-toBeActivated-List				PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

cellCoverageState					CellCoverageState,

sSBCoverageModificationList			SSBCoverageModification-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

sSBInactivationCause			SSBInactivationCause 	OPTIONAL,

...

}

CellCoverageState ::= INTEGER (0..63, ...)

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Served-Cell-Information ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

fiveGS-TAC						FiveGS-TAC			OPTIONAL,

configured-EPS-TAC				Configured-EPS-TAC 		OPTIONAL,

servedPLMNs						ServedPLMNs-List,

nR-Mode-Info					NR-Mode-Info,

measurementTimingConfiguration	OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RANAC							CRITICALITY ignore	EXTENSION RANAC							PRESENCE optional }|

{	ID id-ExtendedServedPLMNs-List		CRITICALITY ignore	EXTENSION ExtendedServedPLMNs-List	PRESENCE optional }|

{	ID id-Cell-Direction				CRITICALITY ignore	EXTENSION Cell-Direction				PRESENCE optional }|

{	ID id-BPLMN-ID-Info-List			CRITICALITY ignore	EXTENSION BPLMN-ID-Info-List			PRESENCE optional }|

{	ID id-Cell-Type						CRITICALITY ignore	EXTENSION CellType						PRESENCE optional}|

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-AggressorgNBSetID				CRITICALITY ignore	EXTENSION AggressorgNBSetID				PRESENCE optional}|

{	ID id-VictimgNBSetID				CRITICALITY ignore	EXTENSION VictimgNBSetID				PRESENCE optional}|

{	ID id-IAB-Info-IAB-DU				CRITICALITY ignore	EXTENSION IAB-Info-IAB-DU				PRESENCE optional}|

{	ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst			PRESENCE optional }|

{	ID id-NRPRACHConfig					CRITICALITY ignore	EXTENSION NRPRACHConfig					PRESENCE optional }|

{	ID id-SFN-Offset					CRITICALITY ignore	EXTENSION SFN-Offset					PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject 	EXTENSION NPNBroadcastInformation		PRESENCE optional }|

{	ID id-Supported-MBS-FSA-ID-List				CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List						PRESENCE optional }|

{	ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information	PRESENCE optional }|

{	ID id-CellDTXDRXConfiguration		CRITICALITY ignore	EXTENSION CellDTXDRXConfiguration 	PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SSBAreaRadioResourceStatusItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBInactivationCause  ::= ENUMERATED { network-energy-saving,...}

SSBInformation ::= SEQUENCE {

sSBInformationList	SSBInformationList,

iE-Extensions		ProtocolExtensionContainer { { SSBInformation-ExtIEs } }	OPTIONAL

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-CellDTXDRXConfiguration 							ProtocolIE-ID ::= xxx

id-SSBInactivationCause								ProtocolIE-ID ::= yyy

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

maxnoofNR-UChannelIDs						INTEGER ::= 16

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofCHOexecutioncond						INTEGER ::= 2

maxnoofServedCellsIAB						INTEGER ::= 512

maxnoofSSBarea											INTEGER ::= 64

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 4. TP to TS 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.5.1	Paging

#### 8.5.1.1	General

The purpose of the Paging procedure is to enable the AMF to page a UE in the specific NG-RAN node.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Paging

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

If the Paging Cause IE is included in the PAGING message, the NG-RAN node shall, if supported, transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the PEIPS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [8].

If the Assistance Data for Recommended SSBs IE is included in the Assistance Data for Paging IE, the NG-RAN node may take it into account when determining the cells and SSBs where paging will be performed.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.69	Assistance Data for Paging

This IE provides assistance information for paging optimisation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Assistance Data for Recommended Cells | O |  | 9.3.1.70 |  | - |  |
| Paging Attempt Information | O |  | 9.3.1.72 |  | - |  |
| NPN Paging Assistance Information | O |  | 9.3.1.183 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | ignore |
| Assistance Data for Recommended SSBs | O |  | 9.3.1.aaa |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| Last Visited SSB List |  | 0..< maxnoofSSBsinUEHistoryInfo > |  | List of visited SSBs. Most recent SSB related information is added to the top of the list. | YES | ignore |
| >Last Visited SSB Information | M |  | 9.3.1.ccc | The SSB related information | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8. |
| maxnoofSSBsinUEHistoryInfo | Maximum number of last visited SSB information records that can be reported in the IE. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.100	Information on Recommended Cells and RAN Nodes for Paging

This IE provides information on recommended cells and NG-RAN nodes for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Cells for Paging | M |  | 9.3.1.71 |  |
| Recommended RAN Nodes for Paging | M |  | 9.3.1.101 |  |
| Recommended SSBs for Paging | O |  | 9.3.1.bbb |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.aaa	Assistance Data for Recommended SSBs

This IE provides assistance information for paging in recommended SSBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended SSBs for Paging | M |  | 9.3.1.bbb | Includes visited and non-visited SSBs, where visited SBBs are listed in the order the UE visited them with the most recent SSB being the first in the list. Non-visited SSBs are included after the visited SSB they are associated with. |

#### 9.3.1.bbb	Recommended SSBs for Paging

This IE contains recommended SSBs for paging.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended SSB List |  | 1 |  |  |
| >Recommended SSB Item |  | 1..<maxnoofRecommendedSSBs> |  | Includes visited and non-visited SSBs, where visited SBBs are listed in the order the UE visited them with the most recent SSB being the first in the list. Non-visited SSBs are included after the visited SSB they are associated with. |
| >>NG-RAN CGI | M |  | 9.3.1.73 |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |
| >>Time Stayed in SSB | O |  | INTEGER (0..4095) | This is included for visited SSBs and indicates the time a UE stayed in an SSB in seconds. If the UE stays in an SSB more than 4095 seconds, this IE is set to 4095. (Value FFS) |

| Range bound | Explanation |
| --- | --- |
| maxnoofRecommendedSSBs | Maximum no. of recommended SSBs. Value is 64. |

#### 9.3.1.ccc	Last Visited SSB Information

The Last Visited SSB Information may contain SSB specific information of a cell.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SSB Index | M |  | INTEGER (0..63) |  |
| Time Stay | O |  | INTEGER (0..4095) | This is included for visited SSBs and indicates the time a UE stayed in an SSB in seconds. If the UE stays in an SSB more than 4095 seconds, this IE is set to 4095 (Value FFS). |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 5. Draft LS

# Title:	[Draft] Reply LS on the enhancements to restricting paging in a limited area

# Response to:	S2-2307984/R3-233729

# Release:	Rel-18

# Work Item:	Netw_Energy_NR-Core

Source:	Huawei [will be RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Feng Han

E-mail Address:	hanfeng3@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:        -

1. Overall Description:

RAN3 would like to thank SA2 for their reply LS on the enhancements to restricting paging in a limited area.

RAN3 discussed the questions and would like to provide the following answers:

Assumption validation: SA2 would also like to validate the following assumption: If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how paging escalation happen will be controlled by the AMF based on existing procedures.

Answer: RAN3 confirms that the assumption is validated. RAN3 expects the AMF can do the paging escalation based on the existing mechanisms without new techniques.

Q1: SA2 understands that the beam-based paging is most efficient towards low mobility and static UEs. How is the gNB expected to determine whether the UE is static or not across multiple RRC connections from the same UE considering TS 33.501 contains requirements on 5G-S-TMSI reallocation?

Answer: RAN3 has made the agreement that “It’s up to gNB’s implementation to decide which UEs should apply the paging enhancement technique”. The gNB can make this decision e.g., based on existing Expected UE Mobility IE or relying on the UE measurement results. RAN3 will not specify it in RAN specification.

Q2: SA2 would like to ask about the validity condition of the potential “List of recommended beams” container that is stored in the AMF. What is the AMF condition to delete the stored "List of recommended beams"?

Answer: RAN3 consdiers that the AMF itself can decide the validity condition of the potential “List of recommended beams”, e.g., the same as the handling of the existing Recommended Cells for Paging, Last Visited Cell Information in TS 38.413. It is up to the AMF implemention to determine to delete it when the UE moves to the connected state, or under other conditions.

2. Actions:

To SA2

ACTION: 	RAN3 kindly asks SA2 to take above information into account.

3. Date of Next TSG RAN3 Meetings:

3GPP TSG RAN WG3#121-bis	09 - 13 October, 2023		Xiamen, China

3GPP TSG RAN WG3#122	13 – 17 November, 2023		Chicago, United States