3GPP TSG-RAN WG3 #121								R3-233997

Toulouse, France,21th – 25th Aug 2023

Agenda Item:	8.1

Source:	China Telecom, Huawei, ZTE

Title: 	Discussion on the attribute "isENDCAllowed"

Document for:	Discussion

Introduction

At RAN3 #121 meeting, a reply LS from SA5 on introduction of a new attribute “Only Resource Coordination” to support source coordination between LTE and NR SA was received [1]. The following request from SA5 are as follow:

SA5 understands if "isENDCAllowed" is set to false, eNB and en-gNB are still able to exchange the information, e.g., resource coordination via X2, but are not allowed to perform EN-DC.

Therefore SA5 kindly asks RAN3 to evaluate whether the existing attribute “isENDCAllowed” can be reused in the use case proposed by RAN3.

SA5 believes the existing attribute “isENDCAllowed” can be also used to support source coordination between LTE and NR SA. This paper aims to discuss the proposed attribute by SA5.

Discussion

Q1: whether the existing attribute “isENDCAllowed” can be applied in this use case?

In Reply LS from SA5 [1], the following understanding on "isENDCAllowed" from SA5 are listed as follow:

Already now the network resource model offers the attribute "isENDCAllowed" to control whether the cells shall be allowed to perform EN-DC.

SA5 understands if "isENDCAllowed" is set to false, eNB and en-gNB are still able to exchange the information, e.g., resource coordination via X2, but are not allowed to perform EN-DC.

As above, SA5 understands the attribute “isENDCAllowed” aims to control enable or disable EN-DC function, and resource coordination is not one of EN-DC functions. Whether “isENDCAllowed” in or “No EN-DC” in TS36.300 can be applied this scenario was discussed in RAN3#116 meeting. During offline discussion, companies believed “No EN-DC” attribute does not ensure that the EN-DC X2 is used only for resource coordination. For example, the “No EN-DC” attribute may be set but the X2 could be used to run the UE Radio Capability ID Mapping procedure. A more precise attribute to ensure that the X2 is used only for Resource Coordination should be added, where it is clearly stated that the X2 connection is used for Resource Coordination Only. Furthermore, Per TS36.423, the LTE-NR resource coordination is closely linked to EN-DC.

- 9.1.4.24	E-UTRA – NR CELL RESOURCE COORDINATION REQUEST

Direction: eNB  en-gNB, en-gNB  eNB.

This message is sent by a neighbouring eNB to a peer en-gNB or by a neighbouring en-gNB to a peer eNB, both nodes able to interact for EN-DC, to express the desired resource allocation for data traffic, for the sake of E-UTRA - NR Cell Resource Coordination.

Whether E-UTRA-NR Cell Resource Coordination can be enabled independently is up to implementation. Based on this, RAN3 had the agreement “to introduce a new attribute “Only Resource Coordination” in Neighbour Cell Relation to support resource coordination between LTE and NR SA sites”.

Therefore, it is proposed to reply to SA5 with an LS containing the following answers:

RAN3 had evaluated the attribute “isENDCAllowed” and understand this attribute does not prevent non-resource coordination functions supported in this scenario. Furthermore, LTE-NR resource coordination is closely linked to EN-DC, and EN-DC functions and E-UTRA-NR Cell Resource Coordination may be not enabled independently in current commercial product. Based on this, RAN3 understands to introduce a new attribute “Only Resource Coordination” in Neighbour Cell Relation can ensure that the X2 is used only for Resource Coordination.

Proposal 1: it is proposed to reply to SA5 with an LS containing the following answers:

RAN3 understands this attribute the attribute “isENDCAllowed”does not prevent non-resource coordination functions supported in this scenario. Furthermore, LTE-NR resource coordination is closely link to EN-DC, and EN-DC functions and E-UTRA-NR Cell Resource Coordination may be not enabled independently in current commercial product. Based on this, RAN3 understands to introduce a new attribute “Only Resource Coordination” in Neighbour Cell Relation can ensure that the X2 is used only for Resource Coordination.

Q2: the overall use case and sequence of messages

In the Reply LS, SA5 asks:

In case our understanding is not correct we would appreciate if RAN3 would be able to elaborate more on the overall use case and sequence of messages. E.g. in which node the attribute has to be set, and to which node the corresponding information will be broadcasted via X2, such that SA5 is able to take this into account.

Based on the motivation of this use case provided in Reply LS to SA5[], we provide a more detailed overall use case as SA5 requests:

the attribute “Resource Coordination Only” is motivated by the need to coordinate resources between co-channel sharing E-UTRA and NR cells, where E-UTRA cells are served by an E-UTRAN eNB in one site and NR cells are served by an NG-RAN gNB and an E-UTRAN en-gNB collocated at a different site. Given that an SA gNB can also support an EN-DC X2 connection, namely an SA gNB can also act as an en-gNB, the solution proposes to reuse the E-UTRA – NR Cell Resource Coordination function available over the EN-DC X2 interface to enable the coordination of resources between co-channel sharing LTE and NR cells. Under this deployment scenario, the assumption made by RAN3 is that the X2: E-UTRA – NR Cell Resource Coordination procedure can lead to resource coordination between E-UTRAN cells of the eNB and NR cells of the en-gNB and that, by means of implementation, the gNB co-located with the en-gNB would also be able to coordinate its resources with the E-UTRAN cells.

RAN3 endorsed a CR to TS36.300, see R3-225158, where the new “Resource Coordination Only” attribute was added to the Neighbour Cell Relation and defined as follows:

Resource Coordination Only: If checked, the Neighbour Cell Relation shall use the X2 interface instance only to coordinate resources between the E-UTRA cell and the NR cell.

This attribute is set at eNB side and not be delivered to en-gNB via X2 interface.

We also provide a draft reply LS to SA5[3] and would like to ask RAN3 to approve this LS.

Proposal 2: to approve the draft Reply LS to SA5[3]

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: it is proposed to reply to SA5 with an LS containing the following answers:

- RAN3 had evaluated the attribute “isENDCAllowed” and understand this attribute does not prevent non-resource coordination functions supported in between LTE and NR SA scenario. Furthermore, LTE-NR resource coordination is closely link to EN-DC, and EN-DC functions and E-UTRA-NR Cell Resource Coordination may be not enabled independently in current commercial product. Based on this, RAN3 understands to introduce a new attribute “Only Resource Coordination” in Neighbour Cell Relation can ensure that the X2 is used only for Resource Coordination

- the attribute “Resource Coordination Only” is motivated by the need to coordinate resources between co-channel sharing E-UTRA and NR cells, where E-UTRA cells are served by an E-UTRAN eNB in one site and NR cells are served by an NG-RAN gNB and an E-UTRAN en-gNB collocated at a different site. Given that an SA gNB can also support an EN-DC X2 connection, namely an SA gNB can also act as an en-gNB, the solution proposes to reuse the E-UTRA – NR Cell Resource Coordination function available over the EN-DC X2 interface to enable the coordination of resources between co-channel sharing LTE and NR cells. Under this deployment scenario, the assumption made by RAN3 is that the X2: E-UTRA – NR Cell Resource Coordination procedure can lead to resource coordination between E-UTRAN cells of the eNB and NR cells of the en-gNB and that, by means of implementation, the gNB co-located with the en-gNB would also be able to coordinate its resources with the E-UTRAN cells.

- RAN3 endorsed a CR to TS36.300, see R3-225158, where the new “Resource Coordination Only” attribute was added to the Neighbour Cell Relation and defined as follows:

- Resource Coordination Only: If checked, the Neighbour Cell Relation shall use the X2 interface instance only to coordinate resources between the E-UTRA cell and the NR cell.

- This attribute is set at eNB side and not be delivered to en-gNB via X2 interface.

Proposal 2: to approve the draft Reply LS to SA5[3]

Reference

- R3-233738/ S5-234839, Reply LS on introduction of a new attribute “Only Resource Coordination” to support source coordination between LTE and NR SA

- R3-232051, Reply LS on introduction of a new attribute “Only Resource Coordination” to support source coordination between LTE and NR SA

- R3-233996, [Draft] Reply LS on introduction of a new attribute “Only Resource Coordination” to support source coordination between LTE and NR SA, Source: China Telecom, To SA5