3GPP TSG-RAN WG3 #121																		R3-234015

21st – 25th Aug 2023

Toulouse, France

Agenda item:	17.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS38.300 BL CR) Discussion on Support Mobility and Service Continuity Enhancements

Document for:	Discussion & decision

# 1 Introduction

This contribution discusses the potential required RAN3 enhancements to support Mobility and Service Continuity Enhancements.

# 2 LS from RAN2

RAN3 received a RAN2 LS ([1]), which states following:

For mobility enhancement in Rel-18 NR NTN, RAN2 has discussed the solution for source cell to broadcast in advance servingCellConfigCommon from the target cell (as common (C)HO signalling) in order to reduce signalling overhead of handover command which is sent using dedicated (per UE) signalling. In this way, the target´s cell servingCellConfigCommon does not need to be included in each RRCReconfiguration message. During RAN2’s discussion, it was indicated this might have RAN3 impact, e.g., for the inter-gNB handover case, the target cell’s servingCellConfigCommon might need to be transferred to the source cell using different means than what’s available in the legacy specification. To facilitate RAN2’s future work, RAN2 would like to ask RAN3 to answer the following question.

In case target cell’s servingCellConfigCommon needs to be broadcasted in the source cell, whether it is feasible that the target cell’s servingCellConfigCommon is provided independently to the source cell for the purpose of inter-gNB handover in Rel-18 with or without specification changes?

ACTION:	RAN2 respectfully asks RAN3 to provide answer to the above question.

The RAN2 LS is for quasi-earth fixed cell case with a single target cell. It proposes a common HO signalling for NTN, wherein the common part of the target cell configuration is broadcast in system information of the source cell, while the remainder of the configuration is to be still provided in HO command and then delivered to the UE via dedicated message (RRC Reconfiguration). This requires the target gNB to divide the configuration of HOCommand into two parts – the common part to be used by the source gNB sent to UE via SIB, another part to be delivered to UE via in RRC Reconfiguration.

The handover to target gNB can be initiated for 2 cases:

- Case 1: due to a EFC Satellite beam switch to next geo-area. This case causes all UEs to be handover to target gNB.     Case 1 is the one considered by the RAN2 LS.

- Case 2: due to the movement of the UE which may be in the cell edge. This case only affects an individual UE.

The current handover signaling is performed per UE. When target gNB receives a XnAP HANDOVER REQUEST message (or a NGAP HO signaling), target gNB cannot know whether the HO is for case 1 thus the target gNB provides HOCommand without the common part. Without knowing whether the handover is for Case 1 or Case 2, the target gNB can only perform as normal Xn (or NG) handover, which target gNB generates a HOCommand including all necessary configuration. It is not possible for target gNB to skip the servingCellConfigCommon in HOCommand for Case 1, but keep it in HOCommand for case 2.

Observation 1-1: when target gNB receives a XnAP/NGAP HO signaling, target gNB cannot know whether this HO is due to EFC satellite beam switch which target gNB shall not include the common part, or only for individual UE which target gNB shall include all configuration part.

RAN3 already agreed the Xn/NG enhancements to support the time-based trigger condition in Xn/NG-HO. The Xn/NG-HO with time based trigger condition allows to preconfigure the UEs with the upcoming cells individually and well ahead  before the handover is executed. The UE can then execute the handover as soon as the conditions are met without further HO command. In addition, by using delta configuration the size of the CHO command can be further reduced. Therefore, the gain of common HO signaling compared to the existing solutions is minimal.

In RAN3 tradition (since LTE), the source gNB can only know the target gNB’s application level configuration data via XnAP Setup procedure or NG-RAN node Configuration Update procedure when there is Xn available between source gNB and target gNB. When there is no Xn between source gNB and target gNB, OAM configuration is used. Both does not work for servingCellConfigCommon. In certain NTN deployment, there is no Xn between source gNB and target gNB. This is why RAN3 spent big effort to develop the enhancements to support the NG-HO with time-based trigger condition.

In the current handover, the RRC configuration is transferred from target gNB to source gNB via a transparent container in NGAP signaling for NG-based handover, and XnAP signaling for Xn-based handover. Current XnAP/NGAP signaling does not support transfer the servingCellConfigCommon separately.

Observation 1-2: there is no existing signaling can be used to transfer the servingCellConfigCommon.

It is not possible for source gNB to know the servingCellConfigCommon in advance via OAM configuration. One may argue to introduce the enhancements to support target gNB transfer servingCellConfigCommon to source gNB via NGAP/XnAP signaling. This is complex

- This requires source gNB initiate a XnAP/NGAP procedure before it initiates the Xn/NG handover preparation procedure for a UE.

- When the source gNB initiate the XnAP/NGAP HO for an affected UE, target gNB does not include the common part in the HOCommand.

However, it is possible that the source gNB may initiate the HO for different UEs in above Case 1 and Case 2 at the same time, e.g. a UE at the cell edge of another cell belonging to same source gNB is moving to same target cell.

In addition, this does not align with RAN3 principle that only application level configuration data is transferred over XnAP Setup procedure, and NG-RAN node Configuration Update procedure. For NG-interface, this will be more complex to introduce the transfer of servingCellConfigCommon. RAN3 never support this type of configuration exchange over Xn and NG.

Due to the complexity and uncertain benefit, it is proposed to not introduce any Xn/NG enhancement in Rel-18.

Proposal 1: RAN3 reply RAN2 that RAN3 will not introduce any Xn/NG signaling enhancement to support the servingCellConfigCommon in Rel-18.

# 3 TAC in Xn interface

Last RAN3 meeting agreed following:

WA: Do not exchange multiple TACs over Xn for NTN.

How to describe which TAC should be used in semantic description?

The Xn SETUP REQUEST/RESPONSE message includes

- TAI Support List, which includes all TAIs supported by the NG-RAN node.

- For each serving cell, it includes a TAC.

RAN3 agreed “WA: Do not exchange multiple TACs over Xn for NTN.” We think this is related to the TAC in the Served Cell Information NR IE. In this IE, the TAC IE is mandatory.

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |

One option is the NTN gNB does not exchange the Served Cell Information NR IE, or exchange the Served Cell Information NR IE but not include a cell which has multiple TACs. When need to handover a UE to a NTN cell, it is very likely that only one candidate cell can be selected as target cell.  Without knowing the multiple TACs of target cell, a UE may be incorrectly handover to a target cell, which the associated TACs all belong to the UE’s forbidden area. The AMF will then de-register the UE.  Since the most scenario only have 1 candidate cell to be selected as target cell, there is no other candidate cells. So this may be not an issue that a UE is HO to a NTN cell which all TACs belong to the UE’s forbidden area, then the AMF will know via the ULI and take action (e.g. de-register the UE)

So there maybe no need to clarify the TAC in Stage-3. If needed, Stage-2 text may be clarified, for example, TS38.300 can be updated as below:

When an NR NTN cell broadcast multiple Tracking Area Codes (TACs) per PLMN, the NTN NG-RAN node does not exchange the information for this NR NTN cell during the Xn Setup procedure or Xn NG-RAN node Configuration Update procedure.

Proposal 2: During the Xn Setup procedure or Xn NG-RAN node Configuration Update procedure, the NTN NG-RAN node does not exchange information for a served cell which have multiple TACs.

TP for TS38.300 BL CR can be found in Annex - Text Proposal for TS38.300 BL CR

If the proposal is accepted, similar TP is needed for TS36.300 BL CR.

# 4 Conclusions

In this contribution, we analyzed the support the remaining issues to support service continuity in NR NTN. Our proposals are:

Proposal 1: RAN3 reply RAN2 that RAN3 will not introduce any signaling enhancement to support the servingCellConfigCommon.

Proposal 2: During the Xn Setup procedure or Xn NG-RAN node Configuration Update procedure, the NTN NG-RAN node does not exchange information for a served cell which have multiple TACs.

TP for TS38.300 BL CR can be found in Annex - Text Proposal for TS38.300 BL CR

If the proposal is accepted, similar TP is needed for TS36.300 BL CR.

# References

- R3-233721, LS on common signalling in (C)HO	RAN2

- R2-2306645, Summary of [AT122][105][NR-NTN Enh] Common signaling in (C)HO (OPPO)

## Annex - Text Proposal for TS38.300 BL CR

### 16.14.3	Mobility and State transition

#### 16.14.3.1	Mobility in RRC_IDLE and RRC_INACTIVE

The same principles as described in 9.2.1 apply to mobility in RRC_IDLE for NTN and the same principles as described in 9.2.2 apply to mobility in RRC_INACTIVE for NTN unless hereunder specified.

The network may broadcast multiple Tracking Area Codes (TACs) per PLMN in an NR NTN cell. A TAC change in the System Information is under network control, i.e. it may not be exactly synchronised with real-time illumination of beams on ground.

The UE can determine the network type (terrestrial or non-terrestrial) implicitly by the existence of cellBarredNTN in SIB1.

The NTN ephemeris is provided in SIB19. It includes serving cell's NTN payload ephemeris and optionally neighbouring cell's NTN payload ephemeris.

##  Unaffected part is omitted ##

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

When an NR NTN cell broadcast multiple Tracking Area Codes (TACs) per PLMN, the NTN NG-RAN node does not exchange the information for this NR NTN cell during the Xn Setup procedure or Xn NG-RAN node Configuration Update procedure.