3GPP TSG-RAN WG3 Meeting #121	R3-234046

Toulouse, France, August 21st – 25th 2023

Agenda Item:	11.4

Source:	Ericsson

Title:	(TP for QoE BL CR for TS 38.473): Enhancements of Rel-17 QoE and RVQoE Features

Document for:	Agreement

# Introduction

In this paper we discuss the following enhancements of Rel-17 QoE and RVQoE features:

- DU participation in RVQoE measurements and reporting.

- Trigger-based RVQoE reporting.

- QMC continuity upon intra-5GC inter-RAT mobility.

A TP for QoE BL CR for TS 38.473 is provided in the Annex.

# DU participation in RVQoE measurements and reporting

We discuss the following issues:

- Deactivation of RVQoE reporting towards the DU.

- DU participation in generating the RVQoE configuration.

## Deactivation of RVQoE reporting to the DU

With respect to the procedure for deactivation of RVQoE reporting via F1AP, the following was agreed in previous meetings:

Introduce the deactivation of RAN visible QoE information transfer via F1. No need to introduce pause/resume mechanism in Release 18.

A class-2 procedure is used for DU to deactivate the RVQoE reporting over F1AP.

Further discuss the details of the procedure used for RVQoE deactivation over F1, e.g., legacy or new procedure, UE associated or non-UE associated signaling.

With respect to the above, we propose that the F1AP procedure for deactivation of RVQoE reporting is:

- A newly defined procedure: Except for the DU-CU Radio Information Transfer, the class-2 procedure names in TS 38.473 are quite feature-specific. Hence, it seems reasonable to define a new QMC-specific procedure.

- A UE-associated procedure: It makes no sense to define a non-UE-associated procedure because, in that case, the deactivation message sent by the DU would mean that the DU is not interested in any RVQoE reports from any UE served by this DU. This is not the intention of the agreement.

Proposal 1: Introduce a new class-2 F1AP UE-associated procedure to control the transfer of RVQoE information from the CU to the DU.

An example of the new UE-associated procedure QoE Information Transfer Control is shown below and proposed in a TP for QoE BL CR for TS 38.473, presented in the Annex.

The RAN3#119-bis-e meeting notes also state the following:

RAN3 should discuss whether the deactivation of RVQoE reporting over F1 is performed per RVQoE configuration or not.

We think that the deactivation of RVQoE reporting should be per RVQoE configuration. Otherwise, the indication would mean that the DU does not want any RVQoE reports for any service type from this UE. Note that a UE may be configured with more than one RVQoE configuration at a time. Then, if the deactivation is not associated to any QoE reference, then the deactivation would apply to both the measurements in which the DU is interested and to the measurements in which the DU is not interested. This does not make any sense whatsoever.

Proposal 2: The deactivation of RVQoE reporting over F1AP is per RVQoE configuration.

In fact, we notice that the F1AP QOE INFORMATION TRANSFER message defined in Rel-17 does not contain any QoE reference. This may lead to a paradoxical situation that the DU has no idea for which service type and QoE reference it is receiving an RVQoE report. Note that the RVQoE metrics defined in Rel-17 may pertain to both the VR and DASH service types. Given that a UE may be simultaneously configured for both VR-related and DASH-related RVQoE measurements, the above paradoxical scenario is possible. Hence, it is also needed to introduce the QoE reference into the existing F1AP QOE INFORMATION TRANSFER message.

The RAN3#119-bis-e meeting notes also capture the following TBC:

Clarify whether the DU triggered deactivation of RVQoE reporting over F1 pertains only to the present application session

The deactivation request should apply only to the present session. The DU could be uninterested in RVQoE reports for pertaining to a QoE reference for various reasons. One of the reasons could be that, after receiving several RVQoE reports, the DU concludes that the “situation” with the present application session is as expected, so the DU may want to allocate its resources to monitoring the sessions that are in a more challenging situation. On the other hand, just because the RVQoE in the present session is as expected, it by no means implies that the RVQoE in subsequent sessions pertaining to that same QoE reference will also be good. So, at the beginning of every application session subject to RVQoE measurements, the CU should consider whether it should send the RVQoE measurement results to the DU. If the DU is not interested in receiving the RVQoE reports, it can always deactivate the reporting. This is fully in line with the legacy principles.

Proposal 3: The-DU triggered deactivation of RVQoE reporting over F1 pertains only to the present application session.

## DU participation in generating the RVQoE configuration

The following is to be further discussed in RAN3:

DU participation in assembling the RVQoE configuration.

Scheduling optimization at the DU was one of the main reasons for introducing, not only the F1AP QoE Information Transfer procedure, but the entire RVQoE concept in Rel-17. In fact, depending on the use case, it might be so that the DU is the only consumer (or the main consumer) of RVQoE reports, meaning that it is appropriate to let the DU have a say in assembling the RVQoE configuration.

Nevertheless, the Rel-17 specifications do not allow the DU to participate in generating the RVQoE measurement configuration. This contradiction should be addressed in Rel-18.

As of today, for RVQoE measurements, the CU receives a list of available RVQoE metrics (from the CN in case of s-based QoE, or from the OAM in case of m-based QoE). The CU configures the UE with the RVQoE metrics to be reported and, optionally, with a reporting periodicity. Even if the RVQoE metrics defined as of today are only two, the reporting periodicity is an essential parameter, and we think that the DU should at least be able to have a say on the value of reporting periodicity. So, the following options can be considered:

Alt1: The CU configures the reporting periodicity without consulting the DU. The DU can understand what the reporting periodicity is only after receiving two consecutive RVQoE reports (reporting periodicity = time elapsed between the two reports). If the reporting periodicity is appropriate (from the DU’s point of view) then no action is needed. If not, the DU can indicate its preferred reporting periodicity to the CU and the CU can reconfigure the UE with a new (updated) reporting periodicity. This approach is very simple, but has some severe limitations when the reported periodicity selected by the CU is not appropriate for the DU:

- The session may be completed when the second report arrives, meaning that the reports are pretty much wasted from the DU’s point of view.

- The network needs to send to the UE two RRCReconfiguration messages: one to configure the RVQoE measurements, and another one to re-configure it, if needed.

If we consider that more RVQoE metrics will be available in the future, then these same limitations are also applicable when the RVQoE metrics of interest for the DU are not identical to the ones selected by the CU.

Alt2: The CU does not configure the reporting periodicity. In this case, the RVQoE reports will be sent according to the reporting interval specified at the application layer in the QoE configuration. However, this parameter is optional (see table below from TS 26.247) – if not present, only one report should be sent after session ends. If the QoE reporting periodicity is not indicated in the XML file, the RAN will not be able to infer it from consecutive RVQoE reports because only one RVQoE report will be sent by the UE at the end of the session. In this case, configuring the RVQoE is de facto useless for the DU.

|  | @reportinginterval | O | Indicates the time(s) reports should be sent. If not present, then the client should send a report after the streaming session has ended. If present, @reportingInterval=n indicates that the client should send a report every n-th second provided that new metrics information has become available since the previous report. For each report sent, only the newly collected information since the previous report shall be reported. |
| --- | --- | --- | --- |

Alt3: The CU configures the reporting periodicity after consulting the DU. In this case, the DU can propose a reporting periodicity (and, possibly, a list of RVQoE metrics). Only after this is done, the CU configures the UE. This can be realized with a new F1AP class-1 procedure, where the CU indicates to the DU the available QoE metrics and the reporting periodicity that it intends to use. The DU confirms the proposal or suggests some changes. Finally, the CU configures the UE. It is reasonable that the CU, being the function in charge of configuring the UE, maintains the possibility to accept or discard the suggestions received from the DU. An example of the new F1AP UE-associated QMC Coordination class-1 procedure is shown in the figure below.

Alt3 has some clear advantages over the other options, and it is our preferred choice, since:

- It enables the DU to express its preference with respect to receiving the RVQoE reports.

- No signalling is wasted to transfer the unwanted RVQoE reports.

- The UE needs to be configured only once, rather than twice (as opposed to Alt1).

Proposal 4: Introduce a new class-1 UE-associated F1AP procedure) initiated by the CU, where:

- The CU indicates to the DU the available RVQoE metrics and the intended reporting periodicity.

- In the response, the DU indicates to the CU its preferred available RVQoE metrics and its preferred reporting periodicity.

Therefore, in addition to the existing class-2 F1AP QoE Information Transfer procedure, the following Rel-18 enhancements are needed:

- A (Rel-18) class-1 procedure (e.g., QMC Coordination) initiated by the CU, used to assemble the RVQoE configuration.

- A (Rel-18) class-2 procedure (e.g., QoE Information Transfer Control) initiated by the DU, and used to notify the CU to deactivate the transfer of RVQoE reports towards the DU.

A TP for QoE BL CR for TS 38.473 adding the two procedures above is provided in the Annex.

Proposal 5: Agree the TP for QoE BL CR for TS 38.473 in the Annex.

# Trigger-based RVQoE reporting

RAN3 has agreed to specify trigger-based RVQoE reporting, where the trigger is associated to a threshold value of application layer buffer level.

## Simultaneous use of two buffer level thresholds

An agreement from RAN3#120 meeting states the following:

The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

The start of the threshold-based RVQoE reporting can be associated to two main scenarios:

- Scenario 1 (poor RVQoE): The user experience (as indicated by the measured buffer level) has become poor, and some intervention by the RAN is needed, e.g., to provide additional scheduling resources for the UE. An indication of the above can be a buffer level lower than a certain threshold.

- Scenario 2 (good RVQoE): The user experience (as indicated by the measured buffer level) is very good, and it makes sense to re-allocate some of the radio resources currently used for this UE to other users instead. An indication of the above can be a buffer level higher than a certain threshold.

The second sentence in the agreement calls for further discussion regarding the simultaneous use of a maximum and a minimum threshold. If we consider the scenarios described above, only one event will be relevant for a certain UE at a certain time. Nevertheless, when a UE is configured, it is unknown in advance which (or both) of the two events will happen, so it makes sense to allow a gNB to configure both events.

Proposal 6: A UE can be simultaneously configured with two triggering events for threshold-based RVQoE reporting:

- One event to trigger the RVQoE reporting when the buffer level is below a threshold.

- Another event to trigger the RVQoE reporting when the buffer level is above a threshold (that may be different than the first threshold).

We also think that it is beneficial for the RAN to monitor how the RVQoE evolves once the reporting has started. For example, when one of the thresholds is crossed, and the RVQoE reporting starts, it is good for the RAN to verify if the action taken (e.g., an increase in the resources assigned to a UE suffering poor QoE) has improved the QoE for the user or not. At RAN3#119-bis-e, three options were considered, according to the agreement below:

RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options:

- Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports)

- Option 2: Periodically based on a gNB configured reporting periodicity

- Option 3: A certain number of times based on gNB configured report amount

One drawback of Option 1 is that it enables the reception of only one RVQoE report, which means that the RAN cannot monitor whether the measures taken based on the first (and only) RVQoE report have been fruitful. Moreover, we notice that Option 3 can be supported by means of Option 2. Namely, if Option 2 is used, the RAN can deactivate the reporting after N RVQoE reports have been received. received reports. Therefore, we prefer supporting Option 2.

With respect to Option 2, the following needs to be discussed:

- The RVQoE reporting periodicity.

- How to stop the reporting, when needed.

A related RAN3#119-bis-e agreement states:

If a UE is configured with periodic RVQoE reporting that automatically starts at the beginning of the application session or immediately upon reception of RVQoE configuration, it cannot be configured with a threshold-based trigger at the same time.

From the application layer point of view, we think that, for “threshold-based RVQoE reporting”, the approach already adopted for “non-threshold-based RVQoE reporting” can be reused, with one difference:

- For non-threshold-based RVQoE reporting, periodic RVQoE reporting automatically starts at the beginning of the application session.

- For threshold-based RVQoE reporting, periodic RVQoE reporting automatically starts upon fulfilment of (at least one of) the threshold-based trigger(s).

In our understanding, there is no need to define a new IE for RVQoE reporting periodicity. Namely, the above RAN3#119-bis-e agreement states that non-threshold-based and threshold-based RVQoE reporting cannot coexist, meaning that the existing IE for RVQoE reporting periodicity can be used for both types of reporting.

Proposal 7: When a trigger for threshold-based RVQoE reporting is satisfied, the RVQoE reports are sent periodically, based on the periodicity configured by the RAN.

A simple way to stop the RVQoE reporting, is to let the UE continue to report until the session ends. Alternatively, the gNB can, at any time, modify the RVQoE configuration and remove the threshold-based trigger for RVQoE reporting.

Proposal 8: Threshold-based RVQoE reporting can be stopped by updating the RVQoE configuration, i.e., by removing the threshold-based trigger(s) for RVQoE reporting. Otherwise, the RVQoE reporting continues until session end.

## The format of buffer level threshold in RVQoE reports

One remaining issue is the format of the buffer level threshold. We propose to express the threshold in the form of the remaining playout time of the content currently in the buffer, and to send an LS asking RAN2 to provide the corresponding RRC signalling support.

Proposal 9: Send an LS asking RAN2 to specify in RRC signalling the buffer level threshold for RVQoE reporting expressed in terms of remaining playout time of the content currently in the buffer.

## Which entity should decide when to trigger threshold-based reporting

So far, we have assumed that the buffer level threshold is set by the RAN. However, after the May meeting, RAN3 and RAN2 received an LS for SA4 in R3-233813 in which SA4 reverts their previous approval of RAN2 assumption that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. In the LS, SA4 motivate the reversal as follows:

- A higher buffer level may help reduce the probability of a playback stall, but may also increase the playback latency, so it’s a trade-off that needs to be balanced by applications. Different proprietary application mechanisms attempting to strike this balance may result in different application behavior for same buffer level or same content.

- Also, different applications (e.g. low-latency live vs. on-demand video applications) have very different behaviour of maintaining the buffer level.

In our understanding, the problem mentioned above is partly caused by the assumption that the buffer level should be represented as the remaining size of data in the buffer (e.g., kB). This is one more reason in favour of representing the buffer level threshold in the form of remaining playout time of the content currently in buffer, as proposed in Section 3.2.

Moreover, our understanding of the motivation provided by SA4 is that the application layer is the most relevant (i.e., more relevant than the RAN) when it comes to deciding how to set the buffer level threshold, i.e., when to trigger the RVQoE reporting. We therefore think that RAN3 should consider allowing the Application layer to decide the buffer level threshold. This would de facto mean that the Application layer decides on its own when to trigger threshold-based RVQoE reporting.

Proposal 10: RAN3 to discuss allowing the UE Application layer to decide the buffer level threshold for RVQoE reporting.

# QMC continuity upon intra-5GC inter-RAT mobility

RAN3 discussed the continuity of QoE measurements during intra-5GC inter-RAT HO, based on LS from RAN2 in R3-231110. The RAN3#119-bis-e and RAN3#120 meeting notes capture the following agreements:

Turn WA to agreement from LTE to NR: WA: For HO from LTE/5GC to NR, there is no impacts to RAN3.

WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.

We discuss and propose a way forward regarding the two HO scenarios.

## QMC configuration sent to an ng-eNB

In case of HO from LTE/5GC to NR, before the HO, the ng-eNB has received the QoE configuration, and it might have configured the UE with it. According to TS 36.331, only the XML file with the QoE container and the indication of service type are sent to the UE.

ASN.1 design in LTE specification

[[	measConfigAppLayer-r15		CHOICE{

release					NULL,

setup					SEQUENCE{

measConfigAppLayerContainer-r15		OCTET STRING (SIZE(1..1000)),

serviceType-r15						ENUMERATED {qoe, qoemtsi, spare6, spare5, spare4, spare3, spare2, spare1}

}

}	OPTIONAL,	-- Need ON

ailc-BitConfig-r15				BOOLEAN							OPTIONAL,	-- Need ON

bt-NameListConfig-r15		BT-NameListConfig-r15					OPTIONAL,	--Need ON

wlan-NameListConfig-r15		WLAN-NameListConfig-r15					OPTIONAL		--Need ON

]],

There can be a mismatch between the maximum size of the XML file that the ng-eNB can receive (up to 8000 bytes) and the maximum size of the XML file that the eNB can send to the UE (up to 1000 bytes). To avoid TS 36.331 impact, it needs to be clarified in RAN3 specifications that, when the Container for Application Layer Measurement Configuration IE is provided to an ng-eNB, the maximum size of the container is limited to 1000 bytes, as highlighted below in yellow.

| Container for Application Layer Measurement Configuration | O |  | OCTET STRING (SIZE(1.. 8000)) | Contains application layer measurement configuration, see Annex L in 26.247 [46], clause 16.5 in TS 26.114 [51] and clause 9 in TS 26.118 [52]. Present in case of initial QoE configuration, and shall be included in Source to Target Transparent Container IE for signalling-based QMC during NG-based handover. The maximum size of this IE is 1000 bytes if the NG-RAN node receiving the IE is an ng-eNB. |
| --- | --- | --- | --- | --- |

Proposal 11: Clarify in the semantics description of Container for Application Layer Measurement Configuration IE in TS 38.413 that, if the NG-RAN node receiving the QMC configuration is an ng-eNB, the maximum size of the container is 1000 bytes.

## Handover from LTE/5GC to NR

In case of Xn-based HO from LTE/5GC to NR, the source ng-eNB sends to the target gNB an XnAP HANDOVER REQUEST message containing the RRC Context IE (presence “M”) which contains a HandoverPreparationInformation message, as defined in TS 38.331. Our understanding is that the source ng-eNB can build such an IE, so we do not see any impact in the signalling or in the implementation to support this IE.

In the XnAP HANDOVER REQUEST message, the source ng-eNB also sends to the gNB the QMC Configuration Information IE. Given that, in LTE, there can be only one QoE measurement per UE, the list represented by the UE Application Layer Measurement Information List IE can only contain one item. It is necessary to clarify this in the semantics description in TS 38.423, as shown below in yellow. Note that this is valid both in case of HO from LTE/5GC to NR and vice versa, as discussed in Section 4.3.

=====================Start of excerpt from TS 38.423=====================

9.2.3.156	QMC Configuration Information

This IE contains the information about the QoE Measurement Collection (QMC) configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Application Layer Measurement Information List |  |  |  | In case of intra-system inter-RAT handover, the list contains only one item. |
| >UE Application Layer Measurement Information Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>UE Application Layer Measurement Configuration Information | M |  | 9.2.3.157 |  |

=====================End of excerpt from TS 38.423=====================

Proposal 12: Clarify in the semantics description of UE Application Layer Measurement Information List IE in TS 38.423 that, in case of intra-system inter-RAT HO, there can be only one item in the list.

Next, we analyse which parts of the XnAP UE Application Layer Measurement Configuration Information IE should be included in the XnAP HANDOVER REQUEST message for intra-system inter-RAT HO:

- QoE Reference: This IE is mandatory in current signalling, and available at the ng-eNB. This IE should be included.

- Measurement Configuration Application Layer ID: This IE is optional in current signalling. Since the ng-eNB does not support the concept of MeasConfigAppLayerId, this IE does not need to be included in case of inter-RAT intra-system HO. The target gNB will send the MeasConfigAppLayerId to the UE as part of the Handover Command in the HO execution, and that is sent to the UE transparently via the source ng-eNB. This IE should not be included.

- Service Type: This IE is mandatory in current signalling, and it is also readable by the ng-eNB. This IE should be included.

- QoE Measurement Status: This IE is optional in current signalling. The target gNB does not need to know this parameter, since, in any case, it will reconfigure the UE, using a MeasConfigAppLayerId. This IE should not be included.

- Container for Application Layer Measurement Configuration: This IE is optional in current signalling, and it contains the signalling-based QoE measurement configuration. Since the maximum size is 1000 bytes when one of the NG-RAN nodes is an ng-eNB, a clarification in the semantics description is needed, as shown below in yellow.

=====================Start of excerpt from TS 38.423=====================

| Container for Application Layer Measurement Configuration | O |  | OCTET STRING (SIZE(1..8000)) | Contains the signalling based QoE measurement configuration, see Annex L in TS 26.247 [47], clause 16.5 in TS 26.114 [53] and clause 9 in TS 26.118 [54]. In case of intra-system inter-RAT handover, the maximum size of the container is 1000 bytes. |
| --- | --- | --- | --- | --- |

=====================End of excerpt from TS 38.423=====================

Proposal 13: Clarify in the semantics description of Container for Application Layer Measurement Configuration IE in TS 38.423 that, for intra-system inter-RAT HO, the maximum size of the container is 1000 bytes.

- MDT Alignment Information: This IE is optional in current signalling, and, since MDT alignment is not supported in LTE, this IE should not be included.

- Measurement Collection Entity IP Address: This IE is optional in current signalling, and it should be present if the same MCE can receive the reports before or after the HO. In any case, there is no additional RAN3 impact.

- Area Scope of QMC: This IE is optional in current signalling, and the target gNB uses it to check the area scope and decide whether to release the QoE configuration. Similar as above, there is no additional RAN3 impact.

- S-NSSAI List: This IE is optional in current signalling, and there is no support in LTE for S-NSSAI, so this IE should not be included.

- Available RAN Visible QoE Metrics: This IE is optional in current signalling, and RVQoE is not supported in LTE, meaning that this IE should not be included.

The gNB (target node) replies to the source ng-eNB with an XnAP HANDOVER REQUEST ACKNOWLEDGE message containing a Target NG-RAN node To Source NG-RAN node Transparent Container IE, that includes a HandoverCommand message, as defined in TS 38.331. We do not see any specification or implementation impact.

Based on the above analysis, we conclude that the only RAN3 specification impact for supporting the QMC continuity upon Xn-based HO from LTE/5G to NR are the clarifications in semantics descriptions in Proposal 12 and Proposal 13.

In case of NG-based HO from LTE/5GC to NR, four steps are executed.

In the first step, the source ng-eNB sends to the AMF an NGAP HANDOVER REQUIRED message, containing a Source to Target Transparent Container IE (Mandatory) and the QMC Configuration Information IE (Optional).

- The Source to Target Transparent Container carries a Source NG-RAN Node to Target NG-RAN Node Transparent Container IE, which includes an RRC Container, carrying an HandoverPreparationInformation message as defined in the TS 38.331. The source ng-eNB can generate such an IE, so there is no specification or implementation impact.

- Regarding the QMC Configuration Information IE, since there can be only one item in the UE Application Layer Measurement Information List IE, the same type of clarification as proposed in Proposal 12 is needed. The analysis made for the IEs included in the QMC Configuration Information IE defined in XnAP applies to the NG-based HO as well, with the additional consideration that, for NG-based HO, the Container for Application Layer Measurement Configuration IE is included in the Source to Target Transparent Container, as already explained in the existing tabular:

=====================Start of excerpt from TS 38.413=====================

| Container for Application Layer Measurement Configuration | O |  | OCTET STRING (SIZE(1.. 8000)) | Contains application layer measurement configuration, see Annex L in 26.247 [46], clause 16.5 in TS 26.114 [51] and clause 9 in TS 26.118 [52]. Present in case of initial QoE configuration, and shall be included in Source to Target Transparent Container IE for signalling-based QMC during NG-based handover. |
| --- | --- | --- | --- | --- |

=====================End of excerpt from TS 38.413=====================

In the second step, the AMF sends to the target gNB the NGAP HANDOVER REQUEST message containing the Source to Target Transparent Container IE. No RAN3 impact is foreseen.

In the third step, the target gNB sends to the AMF the NGAP HANDOVER REQUEST ACKNOWLEDGE message containing the (mandatory) Target to Source Transparent Container IE for the source ng-eNB.

- Regarding the Target to Source Transparent Container IE in the NGAP HANDOVER REQUEST ACKNOWLEDGE message, in case of HO from LTE/5G to NR, this IE carries a Target NG-RAN Node to Source NG-RAN Node Transparent Container IE containing an HandoverCommand message encoded in TS 38.331 (which the gNB prepares for the UE). This IE can be the same as in the intra-system intra-RAT HO case, so there is no RAN3 signalling impact.

Finally, in the fourth step, the AMF sends to the target gNB the NGAP HANDOVER COMMAND message containing the (mandatory) Target to Source Transparent Container IE.

Based on the above analysis, we conclude that the only RAN3 impact for supporting the QMC continuity upon NG-based HO from LTE/5G to NR are the clarifications of semantics descriptions captured in Proposal 12.

Proposal 14: Clarify in the semantics description of UE Application Layer Measurement Information List IE in TS 38.413 that, in case of intra-system inter-RAT HO, there can be only one item in the list.

## Handover from NR to LTE/5GC

The RAN3#120 meeting notes capture the following WA:

WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.

Since the gNB might have configured the UE with multiple QoE measurements, it must be ensured that only one measurement can continue after the HO. So, it needs to be clarified which node decides the (one) QoE configuration to be kept.

In case of Xn-based HO from NR to LTE/5GC, the gNB sends to the target ng-eNB an XnAP HANDOVER REQUEST message containing an RRC Context IE, carrying an HandoverPreparationInformation-NB message encoded as per TS 36.331. We not that it is not possible for the gNB to send a list of QoE configurations without impacting the TS 36.331. Therefore, it must be the source gNB that decides the single QoE configuration to be kept. The same holds for the NG-based HO from NR to LTE/5GC.

Proposal 15: Confirm the WA that in case of HO from NR to LTE/5GC, the source gNB determines which QoE measurement configuration to keep.

In the XnAP HANDOVER REQUEST message, the gNB includes the QMC Configuration Information IE. Since, in LTE, there can be only one QoE measurement configured at the UE, the UE Application Layer Measurement Information List IE can only contain one item. To ensure this, the clarification captured in Proposal 12 is needed.

The content of the UE Application Layer Measurement Configuration Information IE has already been discussed for the HO from LTE/5GC to NR. With respect to the Container for Application Layer Measurement Configuration IE, since the target node is an ng-eNB, the maximum size of the container can be 8000 bytes. However, it might be so that the UE was not configured before the HO took place, and this IE may be used to configure the UE when the UE is served by the ng-eNB node. To avoid any impact on TS 36.331, this IE should not be larger than 1000 bytes. Hence, Proposal 13 applies to this scenario as well.

# Conclusion

In this paper we discuss the enhancements of Rel-17 features. The following is proposed:

DU participation in RVQoE measurements and reporting

Proposal 1: Introduce a new class-2 F1AP UE-associated procedure to control the transfer of RVQoE information from the CU to the DU.

Proposal 2: The deactivation of RVQoE reporting over F1AP is per RVQoE configuration.

Proposal 3: The-DU triggered deactivation of RVQoE reporting over F1 pertains only to the present application session.

Proposal 4: Introduce a new class-1 UE-associated F1AP procedure) initiated by the CU, where:

- The CU indicates to the DU the available RVQoE metrics and the intended reporting periodicity.

- In the response, the DU indicates to the CU its preferred available RVQoE metrics and its preferred reporting periodicity.

Proposal 5: Agree the TP for QoE BL CR for TS 38.473 in the Annex.

Trigger-based RVQoE reporting

Proposal 6: A UE can be simultaneously configured with two triggering events for threshold-based RVQoE reporting:

- One event to trigger the RVQoE reporting when the buffer level is below a threshold.

- Another event to trigger the RVQoE reporting when the buffer level is above a threshold (that may be different than the first threshold).

Proposal 7: When a trigger for threshold-based RVQoE reporting is satisfied, the RVQoE reports are sent periodically, based on the periodicity configured by the RAN.

Proposal 8: Threshold-based RVQoE reporting can be stopped by updating the RVQoE configuration, i.e., by removing the threshold-based trigger(s) for RVQoE reporting. Otherwise, the RVQoE reporting continues until session end.

Proposal 9: Send an LS asking RAN2 to specify in RRC signalling the buffer level threshold for RVQoE reporting expressed in terms of remaining playout time of the content currently in the buffer.

Proposal 10: RAN3 to discuss allowing the UE Application layer to decide the buffer level threshold for RVQoE reporting.

QMC continuity upon intra-5GC inter-RAT mobility

Proposal 11: Clarify in the semantics description of Container for Application Layer Measurement Configuration IE in TS 38.413 that, if the NG-RAN node receiving the QMC configuration is an ng-eNB, the maximum size of the container is 1000 bytes.

Proposal 12: Clarify in the semantics description of UE Application Layer Measurement Information List IE in TS 38.423 that, in case of intra-system inter-RAT HO, there can be only one item in the list.

Proposal 13: Clarify in the semantics description of Container for Application Layer Measurement Configuration IE in TS 38.423 that, for intra-system inter-RAT HO, the maximum size of the container is 1000 bytes.

Proposal 14: Clarify in the semantics description of UE Application Layer Measurement Information List IE in TS 38.413 that, in case of intra-system inter-RAT HO, there can be only one item in the list.

Proposal 15: Confirm the WA that in case of HO from NR to LTE/5GC, the source gNB determines which QoE measurement configuration to keep.

# Annex: TP for QoE BL CR for TS 38.473

-------------------------------------------Start of changes-------------------------------------------

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| >>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<< |  |  |  |
| QMC Coordination | QMC COORDINATION REQUEST | QMC COORDINATION RESPONSE |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| >>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<. |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| QoE Information Transfer Control | QOE INFORMATION TRANSFER CONTROL |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |

-------------------------------------------Next change-------------------------------------------

### 8.16.x	QoE Information Transfer Control

#### 8.16.x.1		General

This procedure is initiated by the gNB-DU to control the transfer of RAN visible QoE information from the gNB-CU to the gNB-DU.

The procedure uses UE-associated signalling.

#### 8.16.x.2	Successful Operation

Figure 8.16.x.2-1: QoE Information Transfer Control procedure.

The gNB-DU initiates the procedure by sending the QOE INFORMATION TRANSFER CONTROL message to the gNB-CU.

If the gNB-CU receives the QoE Information Transfer Action IE in the QOE INFORMATION TRANSFER CONTROL message with the value set to “deactivate”, the gNB-CU shall, if supported, consider that the termination of the ongoing QoE information transfer is requested from the gNB-DU for this UE, and act as specified in TS 38.300 [6].

### 8.16.y	QMC Coordination

#### 8.16.y.1		General

The purpose of the QMC Coordination procedure is to enable coordination of RAN Visible QoE configuration parameters for a UE between a gNB-CU and a gNB-DU.

The procedure uses UE-associated signalling.

#### 8.16.y.2	Successful Operation

Figure 8.16.y.2-1: QMC Coordination procedure.

A gNB-CU initiates the procedure by sending the QMC COORDINATION REQUEST message to a gNB-DU over the F1 interface. The gNB-DU responds with a QMC COORDINATION RESPONSE, including the appropriate information.

If the QMC COORDINATION REQUEST contains the gNB-CU RAN Visible QoE Configuration List IE, the gNB-DU shall, if supported, include in the QMC COORDINATION RESPONSE the gNB-DU RAN Visible QoE Configuration List IE, containing a RAN visible QoE measurement configuration preferred by the gNB-DU.

If the QMC COORDINATION RESPONSE contains the gNB-DU RAN Visible QoE Configuration IE, the gNB-CU may use it according to TS 38.300 [6].

-------------------------------------------Next change-------------------------------------------

#### 9.2.16.z1	QOE INFORMATION TRANSFER CONTROL

This message is sent by the gNB-DU to control the transfer of RAN visible QoE information from the gNB-CU to the gNB-DU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Visible QoE Reporting Action List |  | 0..1 |  |  | YES | ignore |
| >RAN Visible QoE Reporting Action Item |  |  | 1..<maxnoofRVQoEConfigurations> |  | Each | ignore |
| >>QoE Reference | M |  |  | Indicates the RAN visible QoE measurement configuration for which the action applies. | - | - |
| >>QoE Information Transfer Action | M |  | ENUMERATED (deactivate, …) | Indicates an action to apply to the RAN visible QoE reporting. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofRVQoEConfigurations | Maximum no. of QoE information for one UE, the maximum value is 16. |

#### 9.2.16.z2	QMC COORDINATION REQUEST

This message is sent by the gNB-CU to provide the QMC coordination information to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| gNB-CU RAN Visible QoE Configuration List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU RAN Visible QoE Configuration Item |  |  | 1..<maxnoofRVQoEConfigurations> |  | Each | ignore |
| >>QoE Reference | M |  |  |  | - | - |
| >>gNB-CU RAN Visible QoE Configuration | M |  | 9.3.1.t | Indicates the preferred RAN visible QoE configuration for the gNB-CU. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofRVQoEConfigurations | Maximum no. of QoE information for one UE, the maximum value is 16. |

#### 9.2.16.z3	QMC COORDINATION RESPONSE

This message contains the response to the QMC COORDINATION REQUEST message.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| gNB-DU RAN Visible QoE Configuration List |  | 0..1 |  |  | YES | ignore |
| >gNB-DU RAN Visible QoE Configuration Item |  |  | 1..<maxnoofRVQoEConfigurations> |  | Each | ignore |
| >>QoE Reference | M |  |  |  | - | - |
| >>gNB-DU RAN Visible QoE Configuration | O |  | 9.3.1.t | Indicates the preferred RAN visible QoE configuration for the gNB-DU. If absent, the gNB-DU is not interested to received RAN visible QoE measurements for the UE. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofRVQoEConfigurations | Maximum no. of QoE information for one UE, the maximum value is 16. |

-------------------------------------------Next change-------------------------------------------

#### 9.3.1.t	RAN Visible QoE Configuration

This IE contains the information about a RAN visible QoE measurement configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Buffer Level | M |  | ENUMERATED (true, ...) | This IE defines whether the Buffer Level can be collected as a RAN visible QoE metric from UE, for DASH streaming and VR service types. |
| Playout Delay for Media Startup | M |  | ENUMERATED (true, ...) | This IE defines whether the Playout delay can be collected as a RAN visible QoE metric from UE, for DASH streaming and VR service types. |
| Reporting Periodicity | O |  | ENUMERATED (ms120, ms240, ms480, ms 640, ms1024, …) | This IE indicates the periodicity for the reporting of RAN visible application layer measurements. Value ms120 indicates 120 ms, value ms240 indicates 240 ms and so on. |

-------------------------------------------End of changes-------------------------------------------