3GPP TSG-RAN WG3 Meeting #121	R3-234311

# Toulouse, France, August 21st – 25th 2023

Agenda Item:	10.2.2

Source:	Ericsson

Title:	(TP for SON BL CR for TS 37.340) SON enhancements for Mobility Robustness Optimization

Document for:	Discussion and Decision

# 1	Introduction

During previous meetings, a stage-2 definition of Mobility Failure during Voice Fallback and its detection mechanism were agreed for NR and LTE. Further stage-3 updates were also discussed. In this paper we discuss around the purpose of such updates and describe why we think the agreed stage-2 definition is enough.

MRO for Fast MCG Recovery was also discussed. In this contribution we will continue the discussion on the agreed scenario c/ and propose solutions. The forwarding of information about Fast MCG Recovery Failure to the SN (e.g. with RLF Report) will also be discussed.

The last topic discussed last meeting was MRO for CPAC. One remaining aspect for this topic was captured as to be continued during previous meetings: whether/how to support UHI for CPAC. A stage-2 solution is proposed, as well as some editorial changes on the new failure types definitions.

# 2	Discussion

## 2.1 MRO for IRAT Handover due to Voice Fallback

### 2.1.1 Background

The agreed problem definition for Inter-system Mobility Failure for Voice Fallback covers two different variants of the failure, namely:

- Failure during Voice Fallback triggered Mobility from NR: a failure occurs during a handover triggered due to Voice Fallback, and

- Radio Link Failure after Voice Fallback triggered Mobility from NR: an RLF occurs shortly after a successful handover triggered due to Voice Fallback.

When the UE reconnects to the network after the failure, the network will become aware of that the failure was related to a voice fallback handover through the voice fallback indication that has been agreed in RAN2 in the NR and LTE RLF report respectively. In the following we discuss the two different cases from information exchange point of view.

### 2.1.2 Failure during Voice Fallback Triggered Mobility from NR

- In case 1, Failure during Voice Fallback triggered Mobility from NR, the RLF report including a voice fallback indication is created in NR. Since reporting of an NR RLF report to an E-UTRA node is not supported, a UE experiencing failure in NR will indicate the existence of the RLF report to NR when coming back to NR.

- Observation 1.1: Reporting of an NR RLF report to an E-UTRA node is not supported, but the NR RLF report will be reported to an NG-RAN node when the UE comes back to NR.

The same principle should be applied when an NR RLF report is generated due to Mobility from NR Failure during Voice Fallback.

Since the source PCell is already included in the RLF report, the NG-RAN node receiving the RLF report can forward the information to the node hosting the source PCell by using the FAILURE INDICATION message over Xn or by using the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG. This is already supported in specifications and requires no changes.

Observation 1.2: The forwarding of the NR RLF information to the source node is already supported in specifications and requires no changes.

Proposal 1.1: No additional reporting and forwarding of the RLF report or information therein is needed to support MRO for voice fallback in case of Failure during Voice Fallback triggered Mobility from NR.

### 2.1.3 Failure after Voice Fallback Triggered Mobility from NR

Case 2, Failure after Voice Fallback Triggered Mobility from NR reflects the case when a RLF occurs shortly after a successful IRAT handover triggered due to voice fallback. This case is similar to sub-cases of too early (intra-RAT) handover or (intra-RAT) handover to wrong cell in the sense that the failure occurs shortly after a successful handover.

In the voice fallback case however, the IRAT handover cannot be considered as too early. The NG-RAN node had to perform the handover to establish the call, even if radio conditions in source cell were good. There are no parameters to adjust to change the timing of the handover in that case.

Looking at the legacy intra-RAT scenario of handover to wrong cell, where a RLF occurs shortly after a handover from a source to a target cell, and then reconnects in a third cell, this can typically be derived to two different reasons, ultimately caused by handover triggering configurations;

- The handover towards the target cell was initiated before good enough coverage was available from the target cell, or

- The handover towards the third cell was not initiated in time, so that the UE was instead incorrectly handed over towards the target cell.

Looking at the voice fallback triggered mobility from NR, none of these cases are likely to occur, for the same reasons as stated above - the NG-RAN node has to perform the handover to establish the voice call, and does so immediately when the voice call is requested. Further, good LTE coverage is expected in areas where NR is deployed and in the likely case of co-located sites one can also expect one LTE cell to have much stronger signal strength than other LTE cells, making the probability to handover to the wrong cell quite small. Due to this expected coverage of the legacy RAT, IRAT handover to wrong cell has not been supported previously in legacy MRO solutions, not from LTE to UMTS, nor from NR to LTE.

Therefore, Radio Link Failure after Voice Fallback Triggered Mobility from NR should merely be seen as an indication that the target E-UTRA cell coverage needs to be improved.

In RAN2 #122, the following was agreed:

Introduce a new indication in the LTE RLF report for the case an RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback

Using this indication, Radio Link Failure after Voice Fallback Triggered Mobility from NR can be detected by E-UTRAN, and any actions to improve the situation can also be taken in E-UTRAN. There is no need to inform NG-RAN.

Observation 1.3: There is no need to inform NG-RAN about a Radio Link Failure after Voice Fallback Triggered Mobility from NR.

Proposal 1.2: No additional reporting and forwarding of the RLF report or information therein is needed to support MRO for voice fallback in case of Failure after Voice Fallback Triggered Mobility from NR.

## 2.2 Fast MCG recovery

During RAN3#120, case c/ as defined below, has been agreed:

- Case c: Fast recovery near failure case, i.e. UE receives the response message from MN via SN while T316 is running which almost expires but not yet.

Agree to define case c in R18. Solution can be further discussed.

Case c/ could be interesting to optimize T316 value. If T316 is too short, fast MCG failure may occur. If T316 is too big, and in case the MN does not take any action, there will be an addition delay before UE attempts RRC re-establishment.

Before discussing the information needed by the network to optimize T316, it could be interesting to understand how the UE will report it. As for now, at MCG Failure, the RLF Report is generated in the UE. But the UE deletes it if fast MCG recovery is successful. An easy solution, to avoid generating a new report, would be to not delete it, and add the information needed for Fast MCG Recovery optimization in the RLF Report. This would not be an extra burden to the UE, as the RLF Report is generated in any case. And the network can benefit from the information contained in this report, and linked to the MCG Failure. This would also simplify the configuration of the UE participating to T316 optimization (e.g. no need for a new threshold or a new report).

Proposal 2.1: The RLF Report generated after the MCG failure is not deleted by the UE in case fast MCG recovery is successful.

In order to support T316 optimization, the network needs to understand its value at the time of the successful recovery (e.g. if it has almost expired). The more accurate and simple solution is to add the T316 elapsed time (i.e. recovery time if recovery was successful, configured value if not) to the RLF Report. This will of course be reported only if T316 has been configured.

An alternative to that solution was proposed during RAN3#121: SN reports to MN the elapsed time between receiving the MCGFailureInformation and receiving confirmation from the UE of the reception of the reconfiguration from the MN. This would replace the T316 elapsed time. However, this solution does not take into account the following, which may at the end lead to an important inaccuracy:

- Uu delay, for both UL and DL, including retransmissions, which can be important, as the UE may be close to cell edge;

- Time for the SN to receive the acknowledgment from the UE of the reception of the reconfiguration message from the MN.

Additionally, in case of CU-DU split, “receiving confirmation from the UE of the reception of the reconfiguration from the MN” is known only by the SN-DU, while the time is calculated by the SN-CU. Therefore, there will be either an extra inaccuracy element, or an additional F1AP impact.

It is also clear that the bottleneck is on the radio interface, and not the Xn interface. So these times which are omitted from the real value of T316 are of utter importance. Understanding what really happens over the air shall be prioritized in that case.

Therefore, the UE-based solution is preferred.

Proposal 2.2: Add the T316 elapsed time to the RLF Report

As for all the SON reports generated by the UE, there is no mandate to keep the UE Context for 48h. This includes T316 configured value for that UE. Therefore, and to understand how close the UE was to RRC re-establishment, it is proposed to add the T316 configured value in the RLF Report. In case of fast MCG Recovery Failure, T316 elapsed time and the T316 configured value are the same. Therefore, there is no need to report the T316 configured value if it has expired. It should be reported only in case fast MCG Recovery was successful.

Proposal 2.3: Add the T316 configured value in the RLF Report, in case fast MCG Recovery was successful (i.e. T316 was running but did not expire)

During RAN3#119, it was agreed to add “SCG was deactivated or other cases that SCG is not available” as a possible fast MCG recovery failure cause. This agreement was sent to RAN2, and it is now up to RAN2 to decide how to signal it to the network (e.g. using RLF Report). This information (e.g. that SCG was deactivated at the time of failure) would also be useful in the SN, for example to not deactivate SCG in some cases, so that MCG Failure can be attempted in case of RLF in MCG. One possible way to achieve this goal would be to forward the RLF report (if RAN2 decides to add this information into the RLF Report) to the SN. But most of the information contained in the RLF Report will not be useful for the SN. Therefore, and to avoid signalling useless information, it is proposed to send only the SCG Status (i.e. SCG was deactivated or other cases that SCG is not available) to the SN in case SCG status was part of the problem leading to the failure.

Proposal 2.4: SCG status is needed in SN in case of fast MCG recovery failure due to SCG being deactivated or not available.

## 2.3 MRO for CPAC

### 2.3.1 UHI for CPAC

During RAN3#118, the following topic was captured as “to be continued”:

whether/how to support UHI for CPAC

Also, during RAN3#119bis-e, the following understanding of the issue to be solved was captured:

During CPAC configuration, the value of the Time Stay IE for the source PSCell UHI, sent in S-NODE ADDITION REQUEST message, does not reflect the exact time the UE stayed in the source PSCell

Two solutions, as described in [1], were discussed. Solution A was aiming at stage-2 only impact, while stage-3 solution was described as solution B. At the end, it seemed that solution A was preferred by a majority of companies:

Solution A:

A similar solution was already agreed for UHI during CHO and this solution follows the same principle. This would consist in adding a note in TS 37.340 to indicate that the target SN needs to calculate the amount of time since arrival of the PSCell change request to when the UE performs RRC establishment. This will ensure that the SN updates the value of time stay in the latest PSCell entry (i.e. source PSCell) to accurately reflect actual time spent in the cell.

To accomplish this, it is required to add the note shown below in the section 10.5 of 37.340 v17.3.0:

- EN-DC: MN initiated conditional SN Change in Step 7.

- EN-DC: SN initiated conditional SN Change in step 12.

- MR-DC: MN initiated conditional SN Change in Step 7.

- MR-DC: SN initiated conditional SN Change in step 12.

Add the following NOTE:

NOTE: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

Observation 3.1: Notes can be added to TS 37.340 to instruct the target SN to update time stayed IE in SCG UHI in case of conditional PSCell change

At the end of the meeting, the TP was almost agreed, but some companies wanted to continue the discussion on the details. Therefore, it is now proposed to agree solution A and the associated TP for TS 37.340

Proposal 3.1: Agree solution A for UHI for CPAC and notes in TS 37.340

### 2.3.2 New failure definitions

During RAN3#119bis-e, new failure definitions were defined in TS 37.340: Conditional PSCell addition or change failure. The CPC/CPA Execution to wrong PSCell failure definition has been split into 2 failure cases:

- Wrong target PSCell selection at the (candidate) target SN

- Wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA

For better readability, the title of the failure case should be added at the front of the definition, as proposed in Annex.

Proposal 3.2: Agree editorial changes in BL CR for TS 37.340

Proposal 3.3: Agree to the TP in the Appendix

# 3	Conclusion

- On MRO for IRAT Handover due to Voice Fallback

- Observation 1.1: Reporting of an NR RLF report to an E-UTRA node is not supported, but the NR RLF report will be reported to an NG-RAN node when the UE comes back to NR.

Observation 1.2: The forwarding of the NR RLF information to the source node is already supported in specifications and requires no changes.

Proposal 1.1: No additional reporting and forwarding of the RLF report or information therein is needed to support MRO for voice fallback in case of Failure during Voice Fallback triggered Mobility from NR.

Observation 1.3: There is no need to inform NG-RAN about a Radio Link Failure after Voice Fallback Triggered Mobility from NR.

Proposal 1.2: No additional reporting and forwarding of the RLF report or information therein is needed to support MRO for voice fallback in case of Failure after Voice Fallback Triggered Mobility from NR.

On Fast MCG Recovery:

Proposal 2.1: The RLF Report generated after the MCG failure is not deleted by the UE in case fast MCG recovery is successful

Proposal 2.2: Add the T316 elapsed time to the RLF Report

Proposal 2.3: Add the T316 configured value in the RLF Report, in case fast MCG Recovery was successful (i.e. T316 was running but did not expire)

Proposal 2.4: SCG status is needed in SN in case of fast MCG recovery failure due to SCG being deactivated or not available

On MRO for CPAC:

Proposal 3.1: Agree solution A for UHI for CPAC and notes in TS 37.340

Proposal 3.2: Agree editorial changes in BL CR for TS 37.340

Proposal 3.3: Agree to the TP in the Appendix

# References

[1] R3-231585 - (TP for SON BL CR for TS 36.300, TS 38.300, TS 38.423) SON enhancements for MRO (Ericsson)

# Appendix: TP to SON BL CR to TS 37.340

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

## 10.5	Secondary Node Change (MN/SN initiated)

### 10.5.1	EN-DC

-- TEXT OMITTED --

MN initiated conditional SN Change

The MN initiated conditional inter-SN change procedure is used for CPC configuration and CPC execution.

-- TEXT OMITTED --

6a-6b.	The MN triggers the MeNB initiated SgNB Release procedure to inform the source SN to stop providing user data to the UE, and, if applicable, the address of the SN of the selected candidate PSCell to start data forwarding.

7a-7c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SgNB Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

NOTE x: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

8.	The UE synchronizes to the PSCell indicated in the RRCConnectionReconfiguration* message applied in step 5.

-- TEXT OMITTED --

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for CPC configuration and CPC execution.

The SN initiated conditional SN change procedure may also be initiated by the source SN, to modify the existing SN-initiated CPC configuration, or to trigger the release of the candidate SN by cancellation of all the prepared PSCells at the candidate SN and releasing the CPC related UE context at the candidate SN.

-- TEXT OMITTED --

11a-11b.	The MN triggers the MeNB initiated SgNB Release procedure to inform source SN to stop providing user data to the UE, and provide the address of the SN of the selected candidate PSCell and if applicable, start late data forwarding.

12a-12c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SgNB Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

NOTE x: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

13.	The UE synchronizes to the PSCell indicated in the RRCConnectionReconfiguration* message applied in step 10.

-- TEXT OMITTED --

### 10.5.2	MR-DC with 5GC

-- TEXT OMITTED --

MN initiated conditional SN Change

The Conditional Secondary Node Change procedure is initiated by the MN for CPC configuration and CPC execution.

-- TEXT OMITTED --

6a-6c.	The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and if applicable, triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell, to start late data forwarding.

7a-7c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SN Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

NOTE x: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

8.	The UE synchronizes to the PSCell indicated in the RRCReconfiguration* message applied in step 5.

-- TEXT OMITTED --

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for CPC configuration and CPC execution.

The SN initiated conditional SN change procedure may also be initiated by the source SN, to modify the existing CPC configuration, or to trigger the release of the candidate SN by cancellation of all the prepared PSCells at the candidate SN and releasing the CPC related UE context at the candidate SN.

-- TEXT OMITTED --

11a-11c.	The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell and if applicable, starts late data forwarding.

12a-12c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SN Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

NOTE x: The target SN updates the time UE stayed in cell of the latest PSCell entry (the source PSCell) when the UE successfully accesses a candidate cell of the target SN. The updated value of the time UE stayed in source PSCell is equal to the value received from the MN during SN addition plus the time from receiving the SN addition request to completion of the Random Access Procedure with the UE.

13.	The UE synchronizes to the PSCell indicated in the RRCReconfiguration* message applied in step 10.

-- TEXT OMITTED --

### 10.18.A	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases:

-	Wrong target PSCell selection at the (candidate) target SN: if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA to a (candidate) target SN, but not one of the candidate PSCells selected by the (candidate) target SN;

-	Wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA: CPC/CPA Execution to wrong PSCell failure cases not covered by the “Wrong target PSCell selection at the (candidate) target SN” definition.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>