3GPP TSG-RAN WG3 Meeting #121	R3-234448

Toulouse, 21 August - 25 August 2023

Agenda item:	26.3

Source:	CMCC

Title:	Discussion on network slice service continuity

Document for:	Discussion

Introduction

During last meeting, there is no conclusion achieved on network slice service continuity scenario, but some remaining issues are left as follows.

Potential signaling impact on Alternative S-NSSAI in PDU session Modify for backwards compatibility aspects (receiver so far is assumed to not accept a slice modification of ongoing slice)

Opt1: Reusing current S-NSSAI IE

Opt2: Adding a new S-NSSAI IE in the PDU Session Setup/Modification message and Initial UE context setup request message

Potential signaling impact on Alternative S-NSSAI I in PDU session Setup for RAN usage of resources or NS-AoS check

Opt1: Reusing current S-NSSAI IE

Opt2: Adding a new S-NSSAI IE in the PDU Session Setup/Modification message and Initial UE context setup request message

Mobility scenario

Adding Alternative S-NSSAI in HO request message for slice restoration and more generally how to handle restoration e.g. original slice becomes available again after the congestion.

This contribution will continue to discuss the network slice service continuity based on the left issues.

Discussion

Network Slice Service Continuity Scenario

First of all, we think the network slice service continuity scenario should be finalized in this meeting as the common understanding for all RAN3 people. From our view, the reference to scenario in SA2 TR is clearly enough, so that divergence of opinions on network slice service continuity scenario should be eliminated.

In SA2 TR 23700-41, SA2 lists following scenarios may happen:

1) No mobility scenario: Scenario 1a): network slice is overloaded in NG-RAN. Scenario 1b): network slice or network slice instance is overloaded or undergoing planned maintenance in CN (e.g. network slice termination). Scenario 1c): network performance of the network slice cannot meet the SLA. 2) Inter RA Mobility scenario: Scenario 2a): network slice is not supported in the target RAN node. Scenario 2b): network slice in target RAN node is overloaded. Scenario 2c): network slice is not supported in the target CN. Scenario 2d): network slice or network slice instance is overloaded in the target CN. This key issue is to study whether and how to provide service continuity for PDU sessions in network slices in the above scenarios 1b), 1c) and 2d).

From the highlights described, it is clearly shown that SA2 focuses on the status of network slice in CN side, e.g., network slice is overloaded or unavailable in the CN side or the slice cannot fulfil the requirement of SLA. The status of network slice in RAN side was discussed and studied in Rel-17, so it is not recommended to consider within network slice service continuity scenario in Rel-18.

Proposal 1: Network Slice Service Continuity Scenario in Rel-18 WI should focus on the status of network slice in CN side, not to consider the status of network slice in RAN side.

Network Slice Replacement

Aiming to support the service continuity scenario mentioned above, SA2 gives their principles related to network slice replacement.

0) The trigger in the AMF to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM) or based on AM PCF/NSSF notification. The UE capability to support this feature is a prerequisite for the execution of the optimizations in this conclusions.

During the SA2 WI discussion, as the TS 23.501 specified, support of network slice replacement is described as follows:

- 5.15.19 Support of Network Slice Replacement The Network Slice Replacement feature is used to replace an S-NSSAI with an Alternative S-NSSAI when an S-NSSAI becomes unavailable or congested. The Network Slice Replacement may be triggered in the following cases: - If the NSSF detects that an S-NSSAI becomes unavailable or congested (e.g., based on OAM or NWDAF analytics output), it sends network slice availability notification for the S-NSSAI to the AMF. The notification may include an Alternative S-NSSAI which can be used by the AMF to replace the S-NSSAI. The NSSF notifies the AMF when the S-NSSAI is available again. - If the PCF detects that an S-NSSAI becomes unavailable or congested for a UE (e.g., based on OAM or NWDAF analytics output), it sends access and mobility related policy notification to the AMF. The notification may include an Alternative S-NSSAI which can be used by the AMF to replace the S-NSSAI. The PCF notifies the AMF when the S-NSSAI is available again for the UE. - The OAM sends notification to AMF when an S-NSSAI becomes unavailable or congested (and also when this S-NSSAI becomes available again) and provides the Alternative S-NSSAI to AMF. Based on the notification above from NSSF or PCF or OAM, the AMF may determine that an S-NSSAI is to be replaced with Alternative S-NSSAI.

From the SA2 specification, it is found that the Network Slice Replacement feature is used to replace an S-NSSAI with an Alternative S-NSSAI when an S-NSSAI becomes unavailable or congested based on the notification originated from NSSF, PCF and OAM. After the AMF determine that an S-NSSAI is to be replaced with alternative S-NSSAI, PDU session could be established towards the alternative S-NSSAI or retained by using existing PDU session associated with alternative S-NSSAI. Anyway, the SMF shall sends the alternative S-NSSAI to NG-RAN in N2 SM information as specified in SA2 CR.

During a new PDU Session establishment procedure towards an S-NSSAI, - if the UE is provided with the mapping of the S-NSSAI to an Alternative S-NSSAI, the UE provides both the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message. When the AMF receives the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message, or when the AMF receives only the S-NSSAI in PDU Session Establishment message the AMF determines that the S-NSSAI is to be replaced with the Alternative S-NSSAI, the AMF includes both the Alternative S-NSSAI and the S-NSSAI to the SMF. - if the UE is not provided with the mapping of the S-NSSAI to the Alternative S-NSSAI, the UE provides the S-NSSAI in the PDU Session Establishment message. When the AMF determines that the requested S-NSSAI is to be replaced with the Alternative S-NSSAI and if the UE supports Network Slice Replacement, the AMF performs UE Configuration Update procedure to reconfigure the UE with the Alternative S-NSSAI. The AMF continues the PDU Session establishment procedure with the Alternative S-NSSAI and provides both the Alternative S-NSSAI and the S-NSSAI to the SMF. The SMF proceeds with the PDU Session establishment using the Alternative S-NSSAI. The SMF sends the Alternative S-NSSAI to NG-RAN in N2 SM information and to UE in PDU Session Establishment Accept message. For existing PDU Session associated with an S-NSSAI that is replaced with the Alternative S-NSSAI, after the AMF sends mapping of the S-NSSAI to the Alternative S-NSSAI to the supporting UE in UE Configuration Update message, the AMF sends updates to the SMF of the PDU Session, e.g. triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be transferred to Alternative S-NSSAI and includes the Alternative S-NSSAI as follows (see details in clause 4.3.3 of TS 23.502 [3]): - If the SMF determines that the PDU Session needs to be retained (e.g. if the anchor UPF can be reused with the alternative S-NSSAI and SSC mode 1), the SMF sends the Alternative S-NSSAI to the UPF in the N4 message, to the NG-RAN in N2 message and to the supporting UE in PDU Session Modification Command message. - If the SMF determines that the PDU Session needs to be re-established, the SMF sends the Alternative S-NSSAI to the supporting UE either in PDU Session Modification Command if the PDU Session is of SSC mode 3, or in PDU Session Release if the PDU Session is of SSC mode 2 or SSC mode 1, to trigger the re-establishment of the PDU Session. The UE includes both, the S-NSSAI and the Alternative S-NSSAI in the PDU Session Establishment message.

From the highlights described, there will be impact on NGAP associating with PDU session establishment procedure or PDU session modification procedure.

For PDU session establishment procedure, a S-NSSAI IE is already mandatory carried in PDU SESSION RESOURCE SETUP REQUEST message. When SMF sends the alternative S-NSSAI IE to NG-RAN node, the S-NSSAI IE can represent the alternative slice. However, we think it is necessary to introduce a new IE to differentiate between the original slice and alternative slice. It is possible that original slice in CN network is temporary overloaded or unavailable, after a short time period, the resource for the slice in CN network is available again. CN can send a notification to NG-RAN node indicating the original slice in CN network is relieved and the original slice can be reused. From our view, NG-RAN node also has the capability to decide which slice to be used combined with its congestion status and provides its feedback to CN about its preference, i.e., using original slice or alternative slice. While NG-RAN node receives the notification, it needs to know which slice is the original slice. Hence, in order to enable the NG-RAN differentiate between the original slice and alternative slice, we propose to introduce a new IE representing the original slice. For PDU session modification procedure, the same IE should be introduced in PDU SESSION RESOURCE MODIFY REQUEST message for the same reason as above.

Observation 1: It is possible that original slice in CN network is temporary overloaded or unavailable, the resource for the slice in CN network is recovered after a short time period. CN can send a notification to NG-RAN node indicating the original slice in CN network is relieved.

Proposal 2: NG-RAN node has the capability to decide which slice to be used combined with its congestion status and provides its feedback to CN about its preference.

Proposal 3: It is proposed to introduce a new slice IE representing the original slice in PDU SESSION RESOURCE SETUP REQUEST message and PDU SESSION RESOURCE MODIFY REQUEST message.

Network Slice Replacement during HO

Considered the handover scenario, the fundamental part is to maintain service continuity. It is inevitably that network slice replacement performing during handover will cause the service interruption results from PDU session reestablishment or modification. We think it is beneficial for performing network slice replacement before handover without impact on service continuity. Before handover, CN can trigger the network slice replacement by establishing new PDU session or modifying the existing PDU session, ensuring the service continuity for the potential upcoming handover.

Observation 2: It is inevitably that network slice replacement performing during handover will cause the service interruption results from PDU session reestablishment or modification.

Proposal 4: It is beneficial for performing network slice replacement before handover without impact on service continuity.

In addition, slice restoration should be considered during the handover. Assumed that gNB1 supports slice1 and slice2, target gNB2 supports slice2, a UE in gNB1 originally supports slice1. Due to the slice1 in CN is congested or unavailable, CN triggers the network slice replacement for UE1 by transferring associated slice from slice1 to slice2. If UE1 desires handover to a gNB2 which only supports the original slice1, the handover will fail since gNB2 does not support the alternative slice. Actually, gNB2 should have been a potential target for the UE, which brings UE back to original slice may provide a better SLA compared to alternative slice. Thus, in order to avoid the case mentioned above, we think it is required to add the original slice IE in the Handover Request message, helping target NG-RAN node identify the original slice for UE.

Proposal 5: It is proposed to add the original slice IE in the Handover Request message, helping target NG-RAN node identify the original slice for UE.

Furthermore, a scenario did not discuss in last meeting should be noted when RAN3 considers the slice for UE in CN is overloaded or unavailable. It is possible that target NG-RAN node and CN perform inconsistent behaviour for slice admission control during handover. RAN3 should discuss how to eliminate the inconsistent behaviour in the meeting.

Proposal 6: RAN3 should discuss how to eliminate the inconsistent behaviour between target NG-RAN node and CN for slice admission control during handover.

Conclusions

In this paper, we provide our insights on network slice service continuity. The following observations and proposals are listed below:

Observation 1: It is possible that original slice in CN network is temporary overloaded or unavailable, the resource for the slice in CN network is recovered after a short time period. CN can send a notification to NG-RAN node indicating the original slice in CN network is relieved.

Observation 2: It is inevitably that network slice replacement performing during handover will cause the service interruption results from PDU session reestablishment or modification.

Proposal 1: Network Slice Service Continuity Scenario in Rel-18 WI should focus on the status of network slice in CN side, not to consider the status of network slice in RAN side.

Proposal 2: NG-RAN node has the capability to decide which slice to be used combined with its congestion status and provides its feedback to CN about its preference.

Proposal 3: It is proposed to introduce a new slice IE representing the original slice in PDU SESSION RESOURCE SETUP REQUEST message and PDU SESSION RESOURCE MODIFY REQUEST message.

Proposal 4: It is beneficial for performing network slice replacement before handover without impact on service continuity.

Proposal 5: It is proposed to add the original slice IE in the Handover Request message, helping target NG-RAN node identify the original slice for UE.

Proposal 6: RAN3 should discuss how to eliminate the inconsistent behaviour between target NG-RAN node and CN for slice admission control during handover.

Reference

- TR 23.700-41, Study on enhancement of network slicing;

- TS 23.501, System architecture for the 5G System (5GS);