3GPP TSG-RAN3 Meeting #122	R3-23xxxx

Chicago, US, 13th - 17th Nov 2023

Title:	[Draft] Reply LS to SA2 on RedCap UE MBS Broadcast reception

Response to:	(S2-2311706/R3-237153) Reply LS on RedCap UE MBS Broadcast reception

Release:	Rel-18

Work Item:	TEI18

Source:	ZTE [To be RAN WG3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:                xxx

E-mail Address:   xx.xxx (at) xx (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks SA2 for the LS reply and the questions.

RAN3 discussed and agreed to support Scenario 2 by introducing the associated UE type on NGAP, as in the attached agreed CR.

RAN3 also discussed about question 2, “Are the same QoS parameters applicable for both Redcap UEs and non-Redcap UEs?”. Answer from RAN3 is: Yes. However, RAN3 would like to ask SA2 if RAN3 should consider the scenario where a broadcast service intended for both non-redcap and redcap UEs is delivered by 5GC using two QoS flows/MBS sessions, having different QoS requirements. This is because, if a single QoS flow/MBS session is used, the QoS requirement (e.g., bit rate) for non-RedCap UEs will need to be lowered to match the bandwidth compatible for reception of RedCap UEs.

2. Actions:

To SA2:

ACTION: RAN3 kindly asks SA2 to take above information into account.

3. Date of Next TSG-RAN RAN2 Meetings:

TSG-RAN3 Meeting #123			26 Feb - 01 Mar 2024			Athens, GR

TSG-RAN3 Meeting #123-bis			15 – 19 Apr 2024				China (TBC), CN