3GPP TSG-RAN WG3 Meeting #122	      					    R3-237701

Chicago, US, 13th - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1044 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RedCap UE MBS Broadcast reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Lenovo, CATT, Qualcomm, CBN, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the TEI18 led by RAN2 on how to support RedCap UE’s MBS broadcast reception (R2-2300797), gNB is not aware of the associated UE capability information based on current MBS broadcast session management, without which gNB won't be able to make a proper Uu configuration as RAN2 agreed. An indication is needed per SA2 discussion (R3-237153), e.g., associated UE type (e.g., non-RedCap UE, RedCap UE) in a per MBS broadcast session level, in existing session management for MBS broadcast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce “Supported UE type List” IE in the Broadcast Session Setup/Modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The gNB is not aware of the UE type of one MBS Broadcast Session is targeting to, and then gNB is not able to make the proper scheduling decision based on the UE type. A RedCap UE or a non-RedCap UE might not be able to receive the MBS Broadcast service correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.1.2; 8.17.2.2; 9.3.5.3; 9.4.4; 9.4.5; 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specification |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

// change starts

- 8.17.1	Broadcast Session Setup

- 8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

- 8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44].

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Supported UE type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node, if supported, shall take it into account in determining the resources based on the UE type (e.g., non-RedCap UE, RedCap UE).

- 8.17.1.3	Unsuccessful Operation

Figure 8.17.1.3-1: Broadcast Session Setup, unsuccessful operation.

If the NG-RAN node is not able to provide the requested MBS session resources for all the MBS QoS flows in the MBS session in any of its cells, it shall send the BROADCAST SESSION SETUP FAILURE message.

- 8.17.1.4		Abnormal Conditions

Void.

- 8.17.2	Broadcast Session Modification

- 8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

- 8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node, if supported, shall take it into account in determining the resources based on the UE type (e.g., non-RedCap UE, RedCap UE).

- 8.17.2.3	Unsuccessful Operation

Figure 8.17.2.3-1: Broadcast Session Modification, unsuccessful operation.

If the NG-RAN node fails to update any requested modification, the NG-RAN node shall send the BROADCAST SESSION MODIFICATION FAILURE message.

- 8.17.2.4	Abnormal Conditions

Void.

// next change

- 9.3.5	MB-SMF Related IEs

- 9.3.5.1	Void

- 9.3.5.2	Void

- 9.3.5.3	MBS Session Setup or Modification Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session TNL Information 5GC | O |  | 9.3.2.15 |  | YES | reject |
| MBS QoS Flows To Be Setup or Modified List | M |  | MBS QoS Flows To Be Setup List 9.3.1.236 |  | YES | reject |
| MBS Session FSA ID List |  | 0..<maxnoofMBSFSAs> |  |  | YES | ignore |
| >MBS Frequency Selection Area Identity | M |  | OCTET STRING (SIZE(3)) |  | - |  |
| Supported UE type List |  | 0..<maxnoofUETypes> |  | Supported UE type List | YES | ignore |
| >Supported UE type | M |  | ENUMERATED (non-RedCap UE, RedCap UE, …) |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSQoSFlows | Maximum no. of QoS Flows allowed within one MBS session. Value is 64. |
| maxnoofMBSFSAs | Maximum no. of FSA IDs for one MBS session. Value is 64. |
| maxnoofUETypes | Maximum no. of associated UE types. Value is 8. |

- 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

AMF-TNLAssociationToRemoveList,

AMF-TNLAssociationToUpdateList,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

AuthenticatedIndication,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

CancelAllWarningMessages,

Cause,

// unrelated skipped

RRCState,

SecurityContext,

SecurityKey,

SerialNumber,

ServedGUAMIList,

SliceSupportList,

S-NSSAI,

SONConfigurationTransfer,

SourceToTarget-TransparentContainer,

SourceToTarget-AMFInformationReroute,

SRVCCOperationPossible,

SupportedUEtypeList,

SupportedTAList,

Suspend-Request-Indication,

Suspend-Response-Indication,

TAI,

// unrelated skipped

id-SliceSupportList,

id-S-NSSAI,

id-SONConfigurationTransferDL,

id-SONConfigurationTransferUL,

id-SourceAMF-UE-NGAP-ID,

id-SourceToTarget-TransparentContainer,

id-SourceToTarget-AMFInformationReroute,

id-SRVCCOperationPossible,

id-SupportedUEtypeList,

id-SupportedTAList,

id-Suspend-Request-Indication,

id-Suspend-Response-Indication,

id-TAI,

id-TAIListForPaging,

id-TAIListForRestart,

id-TargetID,

// unrelated skipped

- 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

// unrelated skipped

maxnoofTAIforInactive,

maxnoofTAIforMBS,

maxnoofTAIforPaging,

maxnoofTAIforRestart,

maxnoofTAIforWarning,

maxnoofTAIinAoI,

maxnoofTargetS-NSSAIs,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofUEAppLayerMeas,

maxnoofUEsforPaging,

maxnoofUETypes,

maxnoofWLANName,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofThresholdsForExcessPacketDelay

// unrelated skipped

MBSSessionSetupOrModRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MBSSessionSetupOrModRequestTransferIEs} },

...

}

MBSSessionSetupOrModRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionTNLInfo5GC				CRITICALITY reject	TYPE MBS-SessionTNLInfo5GC				PRESENCE	optional		}|

{ ID id-MBS-QoSFlowsToBeSetupModList		CRITICALITY reject	TYPE MBS-QoSFlowsToBeSetupList		PRESENCE	mandatory	}|

{ ID id-MBS-SessionFSAIDList				CRITICALITY ignore	TYPE MBS-SessionFSAIDList				PRESENCE	optional		}|

{ ID id-SupportedUEtypeList					CRITICALITY ignore	TYPE SupportedUEtypeList			PRESENCE	optional	},

...

}

SupportedUEtypeList ::= SEQUENCE (SIZE(1.. maxnoofUETypes)) OF SupportedUEtype

SupportedUEtype ::= ENUMERATED {non-RedCap-UE, redCap-UE, ...}

MBS-SessionFSAIDList ::= SEQUENCE (SIZE(1.. maxnoofMBSFSAs)) OF MBS-SessionFSAID

MBS-SessionFSAID ::= OCTET STRING (SIZE(3))

// unrelated skipped

- 9.4.7	Constant Definitions

// unrelated skippe

-- ASN1START

-- **************************************************************

--

- -- Lists

--

-- **************************************************************

// unrelated skipped

maxnoofTimePeriods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEsforPaging							INTEGER ::=	4096

maxnoofUETypes,								INTEGER ::=	8

maxnoofWLANName								INTEGER ::= 4

maxnoofXnExtTLAs							INTEGER ::= 16

maxnoofXnGTP-TLAs							INTEGER ::= 16

maxnoofXnTLAs								INTEGER ::= 2

-- **************************************************************

--

- -- IEs

--

-- **************************************************************

// unrelated skipped

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-HashedUEIdentityIndexValue							ProtocolIE-ID ::= 365

id-SupportedUEtypeList									ProtocolIE-ID ::= xxx

END

-- ASN1STOP

// change ends