3GPP TSG-RAN3 Meeting #122	R3-237836

Chicago, US, 13-17 Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.420 | CR | 0037 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.420) Support of mobile IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mobile_IAB-Core |  |  |  |  |  | Date: |  |  | 2023-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add changes to support mobile IAB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: - Add definitions for mobile IAB-node and RRC-terminating IAB-donor of the mobile IAB-node. - Support IAB Transport Migration function between F1-terminating IAB-donor-CU and RRC-terminating IAB-donor-CU of a mobile IAB-node. - Support IAB Resource Coordination function between F1-terminating IAB-donor-CU and RRC-terminating IAB-donor-CU of a mobile IAB-node. - Delete “-CU” in definitions of F1-terminating IAB-donor-CU and Non-F1-terminating IAB-donor-CU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobile IAB cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.2.10.2, 5.2.10.3, 6.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.423 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of Change------------------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Boundary IAB-node: as defined in TS 38.401 [2].

corresponding node: as defined in TS 38.425 [7].

F1-terminating IAB-donor: as defined in TS 38.401 [2].

IAB-DU: as defined in TS 38.300 [8].

IAB-MT: as defined in TS 38.300 [8].

Mobile IAB-node: as defined in TS 38.300 [8].

Non-F1-terminating IAB-donor: as defined in TS 38.401 [2].

NG-RAN node: as defined in TS 38.300 [8].

RRC-terminating IAB-donor: as defined in TS 38.401 [2].

secondary node: as defined in TS 37.340 [9].

-------------------------------------------Next Change------------------------------------------

#### 5.2.10.2	IAB Transport Migration function

This function allows the exchange of information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, for the purpose of managing the migration of the boundary and descendant IAB-node traffic between the topologies managed by the two IAB-donor-CUs. This function also allows the exchange of information between the F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU of a mobile IAB-node, for the purpose of managing the migration of the mobile IAB-node traffic between the topologies managed by the two IAB-donor-CUs.

#### 5.2.10.3	IAB Resource Coordination function

This function is used to exchange information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node in order to support resource multiplexing between the IAB-MT and the IAB-DU of the boundary IAB-node. This function is also used to exchange information between the F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU of a mobile IAB-node in order to support resource multiplexing between the IAB-MT and the IAB-DU of the mobile IAB-node.

-------------------------------------------Next Change------------------------------------------

### 6.2.10	IAB procedures

The IAB procedures are used to enable the transfer of F1/non-F1 traffic for IAB, to exchange information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, to enable the delivery of F1-C traffic between the M-NG-RAN node and the S-NG-RAN node serving a dual-connected non-boundary IAB-node, to exchange resource multiplexing related information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, to exchange information between the F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU of a mobile IAB-node, and to exchange resource multiplexing related information between the F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU of a mobile IAB-node:

-	F1-C Traffic Transfer

-	IAB Transport Migration Management

-	IAB Transport Migration Modification

-	IAB Resource Coordination

-------------------------------------------End of Change-------------------------------------------