3GPP TSG-RAN WG3 Meeting #122	R3-237195

Chicago, USA, 13 - 17 November 2023

Agenda item:	26.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.413 BL CR) Resolution of open issues for TRS and URLLC

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we evaluate and resolve open issues for timing resiliency and RAN feedback according to the following WID objectives [1]:

1. 5GS network timing synchronization status and reporting [RAN3, RAN2]: a. AMF providing clock quality reporting control information per-UE to the gNB. [RAN3] b. gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3] Note 1: Details of the 5G clock quality information will be decided by RAN3. c. UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2] d. gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3] 3. Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2]: a. RAN enhancements in order for application to adapt scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication.

# 2	Discussion

## 2.1	TRS: Granularity of the Clock Accuracy IE

At RAN3#121bis, it was agreed to introduce an encoding option for the Clock Accuracy IE that indicates the absolute clock accuracy value. Two different granularities were discussed:

Option A: 25ns granularity, which is the same granularity as the uncertainty field in ReferenceTimeInfo defined in TS 38.331; or

Option B: 10ns granularity, which is the same granularity as the time field in ReferenceTimeInfo defined in TS 38.331.

In our view, there is no need to support clock accuracy values smaller than 25ns, similar to the uncertainty field in RRC (i.e., Option A). Therefore, the FFS can be removed.

Proposal 1:	Remove the FFS on the 25ns granularity for the Clock Accuracy Value IE within the RAN Timing Synchronisation Status Information (NGAP, F1AP), and for the Clock Accuracy IE within the Clock Quality Acceptance Criteria (NGAP).

## 2.2	TRS: RRC_INACTIVE support

RAN2 has reached agreements to be considered in the discussion for UEs in RRC_INACTIVE

RAN2#121 agreements: - RAN2 to confirm that there is no need to support “group of cells across gNBs” for scope of the report ID. The scenario, as per RAN3 will be supported - different cells within gNB that are served by different DUs and different clock quality is possible. RAN2#122 agreements: - Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED. - If the UE is in RRC_INACTIVE, the UE can acquire clock quality information using the SDT procedure, if it supports and/is configured with SDT procedure. RAN2#123 agreements: - Confirm in RAN2 that no AS capability is needed for the UE to support clock quality information mechanisms. RAN2#123B agreements: - Differentiation of access attempt cause is not needed for the purpose of obtaining Clock Quality Information. - event ID is included in the DLInformationTransfer message. The UE maintains an updated variable of the eventID coupled with the associated Clock Quality Information in DLInformationTransfer.

As agreed in RAN2#122 if the UE is in RRC_INACTIVE, the UE can acquire clock quality information using the SDT procedure, if it supports and/is configured with SDT procedure. This agreement can be seen in RAN2 Stage 2 running CR on timing resiliency and URLLC (R2-2310689) with the following figure (Figure 16.8-3 in 38.300 CR):

Figure 16.8-3: Signalling procedure of gNB reporting clock quality information to a UE

In RAN2 discussions, SDT is not precluded to be used for a UE to reconnect with the gNB, but the support of MO-SDT procedure case was assumed to not have impacts in the specification (e.g., there is no new cause for reconnection added). In the last RAN3 meeting, R3-235575 discussed the support for RRC INACTIVE UEs using MO-SDT from RAN3 perspective. A UE in RRC INACTIVE subscribed to clock quality information will need to monitor gNB ID and Event ID changes to retrieve the last RAN TSS update. When the UE goes to RRC INACTIVE, it presumably stores an Event ID with associated clock quality information (i.e., gNB-ID = X, Event ID = Y, RAN TSS). If later the UE detects a change on the gNB ID, it will need to reconnect to the new serving gNB to retrieve the current available RAN TSS. Additionally, Event ID integer applies for a group of cells within a gNB (as per RAN2#121 agreement), then, when the UE moves to a cell served by a different gNB, a different Event ID will be broadcasted, then the UE will need to reconnect to the new serving gNB.

In both cases, two scenarios can be considered for a UE in RRC INACTIVE:

1)	The UE performs RRC Resume procedure to receive clock quality information associated with this new Event ID and gNB ID. In this case, gNB anchor relocation is needed to enable the new receiving gNB to provide RAN TSS to the UE via DLInformationTransfer.

2)	The UE performs MO-SDT procedure to receive clock quality information associated with this new Event ID and gNB ID. In this case, the gNB implementation should determine that a UE subscribed to RAN TSS should always relocate the context to the new receiving gNB, as the new gNB does not know why the UE reconnected in the MO-SDT.

Therefore, the last serving gNB should always relocate the UE context when the UE is monitoring RAN TSS.

Proposal 2:	The last serving gNB should always relocate the context to the receiving gNB for UE that are monitoring RAN TSS, but this can be left to gNB implementation.

## 2.3	RAN Feedback: maximum number of Allowed Periodicities

For proactive feedback, the TSCAI can contain a list of Periodicity value(s), from which the gNB can select an adjusted Periodicity.  There current maximum number of allowed periodicities in the list is 8 with FFS. In our view, 8 is a reasonable number since applications should typically not have many periodicity options available especially when considering that a certain service quality is required.

Also, there does not seem to be a need to make the Periodicity List an extensible sequence. It can simply be a flat list of periodicities, and if something more is needed in the future (doubtful) then an additional CHOICE can be added (i.e. the choice structure provides the desired level of extensibility).

Proposal 3:	Remove the FFS on the value of 8 for maxnoofPeriodicities and avoid encoding the Allowed Periodicity List IE as an extensible SEQUENCE.

# 3	Conclusions

In this paper, we evaluated open issues for timing resiliency and made the following observations and proposals:

Proposal 1:	Remove the FFS on the 25ns granularity for the Clock Accuracy Value IE within the RAN Timing Synchronisation Status Information (NGAP, F1AP), and for the Clock Accuracy IE within the Clock Quality Acceptance Criteria (NGAP).

Proposal 2:	The last serving gNB should always relocate the context to the receiving gNB for UE that are monitoring RAN TSS, but this can be left to gNB implementation.

Proposal 3:	Remove the FFS on the value of 8 for maxnoofPeriodicities and avoid encoding the Allowed Periodicity List IE as an extensible SEQUENCE.

An NGAP TP for the above proposals is in Annex A.

# References

- RP-231106 Revised WID on NR Timing Resiliency and URLLC enhancements, Nokia, Nokia Shanghai Bell

# Annex	A: Text Proposal for TS 38.413

Start of modifications

#### 9.3.1.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | BIT STRING { locked (0), holdover (1), freeRun (2) } (SIZE (8, …)) | Each position in the bitmap represents a synchronisation state. If a bit is set to “1”, the respective synchronisation state is acceptable. If a bit is set to “0”, the respective synchronisation state is not acceptable. Bits 3-7 reserved for future use. |
| Traceable to UTC | O |  | ENUMERATED (true, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | INTEGER (1..40000000, …) | Clock accuracy expressed in units of 25 ns. |
| Parent Time Source | O |  | BIT STRING { syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4), serialTimeCode (5), nTP (6), handset (7), other (8) } (SIZE (16, …)) | Each position in the bitmap represents a parent time source. If a bit is set to “1”, the respective parent time source is acceptable. If a bit is set to “0”, the respective parent time source is not acceptable. Bits 9-15 reserved for future use. |

Next modification

#### 9.3.1.x5	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >Value |  |  |  |  |
| >>Clock Accuracy Value | M |  | INTEGER (1.. 40000000, …) | Indicates the absolute clock accuracy value. Unit in 25ns. |
| >Index |  |  |  |  |
| >>Clock Accuracy Index | M |  | INTEGER (32..47, ...) | Indicates the clockAccuracy enumeration value specified in Table 5 of clause 7.6.2.6 of IEEE Std 1588 [AA]. |

Next modification

#### 9.3.1.z2	Periodicity Range

This IE indicates the periodicity range for the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Periodicity Range | M |  |  |  |
| >Periodicity Bound |  |  |  |  |
| >>Periodicity Lower Bound | M |  | Periodicity 9.3.1.132 |  |
| >>Periodicity Upper Bound | M |  | Periodicity 9.3.1.132 |  |
| >Periodicity List |  |  |  |  |
| >>Allowed Periodicity List |  | 1..<maxnoofPeriodicities> |  |  |
| >>>Allowed Periodicity |  |  | Periodicity 9.3.1.132 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPeriodicities | Maximum no. of allowed periodicities. Value is 8. |

Next modification

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

** skip unmodified text **

Periodicity ::= INTEGER (0..640000, ...)

PeriodicityBound ::= SEQUENCE {

periodicityLowerBound					Periodicity,

periodicityUpperBound					Periodicity,

iE-Extensions				ProtocolExtensionContainer { {PeriodicityBound-ExtIEs} } OPTIONAL,

...

}

PeriodicityBound-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AllowedPeriodicityList ::= SEQUENCE (SIZE(1..maxnoofPeriodicities)) OF Periodicity

PeriodicityRange ::= CHOICE {

periodicityBound				PeriodicityBound,

periodicityList					AllowedPeriodicityList,

choice-extensions				ProtocolIE-SingleContainer { {PeriodicityRange-ExtIEs} }

}

PeriodicityRange-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

PeriodicRegistrationUpdateTimer ::= BIT STRING (SIZE(8))

End of modifications