3GPP TSG-RAN WG3 Meeting #122	R3-237231

Chicago, US, 13-17 Nov, 2023

Title: 	(TP to BLCR for TS 38.413, 38.423, 38.473 and 38.401) Support of 5G Timing Resiliency enhancements

Source: 	Huawei, China Unicom

Agenda item:	26.2

Document Type:	Other

# 1. Introduction

At the previous RAN3-121bis meeting, the encoding details of the TSS parameters were agreed as follows.

Encode the Clock Quality Acceptance Criteria IE with information as listed in R3-235147. The encoding of the Clock Accuracy IE with one choice structure (including Option a and Option b) over NGAP from gNB to AMF and F1AP.

The LS [1] was sent to other WGs to provide the signalling details. This contribution will further discuss the following issues:

- the triggering conditions of RAN TSS reporting from the DU to the CU;

- the remaining FFS on the unit of the Clock Accuracy IE.

# 2. Discussion

## 2.1 RAN TSS reporting from DU to CU

For NGAP, the new Class 1 procedure was agreed, so the AMF can enable the CU to start or stop the timing synchronization status (TSS) report. Similarly, for F1AP, a new Class 1 procedure is introduced to let the CU indicate the DU to start or stop the TSS report.

The DU can report the node level timing synchronization status to the CU, via the new Class 2 message TIMING SYNCHRONISATION STATUS REPORT, as follows [2]:

### 8.xx.2 Timing Synchronisation Status Report #### 8.xx.2.1 General The purpose of the Timing Synchronisation Status Report procedure is to enable the gNB-DU to provide RAN timing synchronisation status information to the gNB-CU. The procedure uses non-UE associated signalling. #### 8.xx.2.2 Successful Operation Figure 8.xx.2.2-1: Timing synchronisation status report The gNB-DU initiates the procedure by sending a TIMING SYNCHRONISATION STATUS REPORT message to the gNB-CU. #### 8.xx.2.3 Abnormal Conditions Void.

As specified in TS 23.501 [3], it seems that the gNB may be pre-configured with a single threshold for each timing synchronization status attribute.

gNBs may be pre-configured with thresholds for each timing synchronization status attribute, if supported, that is described in Table 5.27.1.12-1. When the network timing synchronization status exceeds the thresholds (i.e. status degradation), or the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB notifies the TSCTSF (either using N2 node level signalling via AMF, or via OAM) with the scope of the timing synchronization status (i.e. gNB ID or a list of Cell IDs within a single gNB) and the corresponding network timing synchronization status (TSS) attributes as described in this clause. The gNB indicates the status change to the UEs via SIB information: - When the network timing synchronization status exceeds any of the pre-configured thresholds, the gNB includes in SIB information a reference report ID. When the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB broadcasts the reference report ID in SIB information. Either event serves as a notification for the UEs reading the SIB information that there is new TSS information available. NOTE 1: NG-RAN is assumed not to provide clock quality metrics better than the pre-configured threshold, i.e. if a clock quality metric is better than the corresponding threshold, the NG-RAN reports the threshold value to the UE in an RRC message instead. NOTE 2: It is assumed the pre-configured thresholds in the gNB(s) are sufficient to meet UE time sync performance requirement which are configured by the operator.

In this case, the DU may only trigger the TSS report to provide the latest timing synchronization status to the CU when the specific attribute exceeds the specific threshold or meets the threshold again.

For the RAN clock quality reporting control information (CQRCI) with clock quality detail level set to “acceptable/not acceptable indication”, the CU can determine whether it needs to send the new 5G clock quality information (i.e. acceptable/not acceptable indication) to the UE, based on the TSS report reported by the DU and the per-UE level clock quality acceptance criteria from the AMF. However, it would be difficult for the CU to determine whether the current timing synchronization status is acceptable or not for a specific UE, if the CU can only know if the node level timing synchronization status is better or worse than a pre-configured threshold.

Let us take the Clock Accuracy as an example.

- If the threshold pre-configured for the DU is 500ns, and the acceptance criteria of the Clock Accuracy for two UEs are 500ns (UE A) and 1μs (UE B), respectively.

- When the Clock Accuracy is worse than 500ns, the DU will report its latest Timing Synchronisation Status Information to the CU (e.g., the clock accuracy is 600ns), the CU can derive that the timing synchronization status is acceptable to UE B but not for the UE A.

- But when the DU’s Clock Accuracy becomes even worse (e.g., 1.5us), it will not report to the CU, so the CU cannot update the timing synchronization status for the UE A and UE B.

The issue has been discussed during the last RAN3 meeting, and the following potential solutions were listed in the moderator’s summary [4]:

If the problem is acknowledged, then potential solutions: Option A: Proposal 5 in [1]: Add an optional Clock Accuracy Threshold List IE to the F1AP: TIMING SYNCHRONISATION STATUS REQUEST message, that includes a list of clock accuracy thresholds corresponding to the acceptance criteria of UEs served by the gNB-CU. Option B: Proposal 2 in [3]: If only one threshold is pre-configured for each attribute, DU can trigger periodic report of the timing synchronization status if enabled by the CU.

For Option A, the DU may store a list of thresholds, and have to determine if the attribute exceeds any threshold or meet any threshold again each time when the clock quality changes. For Option B, it seems difficult for the network to control a suitable reporting periodicity.

Hence it is better to leave the triggering of timing synchronization status report to the DU implementation. RAN3 has already designed the encoding of the TSS as follows [1]. Each time when one attribute of the timing synchronization status changes, e.g. the synchronization state changes from locked state to holdover state, or the clock accuracy changes from one level to another one, the DU can report the latest timing synchronization status to the CU. In our understanding, the timing synchronization status would not change frequently in usual cases. And it is expected that this solution will not incur signalling overhead issue. To reflect this, a note can be added in the procedures in the BLCR for TS 38.401.

IE/Group Name; Presence; IE type and reference; Semantics description. Synchronisation State; O; ENUMERATED (locked, holdover, freeRun, …). Traceable to UTC; O; ENUMERATED (true, false, …). Traceable to GNSS; O; ENUMERATED (true, false, …). Clock Frequency Stability; O; BIT STRING (SIZE (16)); Indicates the offsetScaledLogVariance as specified in TS 23.501 [9].. Clock Accuracy; O; see below. Parent Time Source; O; ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …). IE/Group Name; Presence; IE type and reference; Semantics description. CHOICE Clock Accuracy; M. >value. >>Clock Accuracy Value; M; INTEGER (1..40000000, …); Indicates the absolute clock accuracy value. Unit in 25ns [FFS].. >index. >>Clock Accuracy Index; M; INTEGER (32..47, …); Indicates the clockAccuracy enumeration value specified in Table 5 of clause 7.6.2.6 of IEEE Std 1588.

- It is up to the DU’s implementation to trigger the timing synchronization status report to the CU. A note can be added in the BLCR for TS 38.401.

## 2.2 Clock Accuracy encoding

For the encoding of the RAN Timing Synchronisation Status Information IE, one remaining issue is the unit of the Clock Accuracy IE. One possible way is to introduce a smaller unit than 25ns, e.g. 10ns. But it seems to us that a finer unit is not necessary. In the 5G timing synchronization featured specified in Rel-16 IIoT, the granularity of the uncertainty parameter takes the unit of 25ns, as shown in the following text excerpted from TS 38.331.

ReferenceTimeInfo-r16 ::= SEQUENCE { time-r16 ReferenceTime-r16, uncertainty-r16 INTEGER (0..32767) OPTIONAL, -- Need S timeInfoType-r16 ENUMERATED {localClock} OPTIONAL, -- Need S referenceSFN-r16 INTEGER (0..1023) OPTIONAL -- Cond RefTime } uncertainty This field indicates the uncertainty of the reference time information provided by the time field. The uncertainty is 25ns multiplied by this field. If this field is absent, the uncertainty is unspecified.

Indeed, the uncertainty also reflects the clock accuracy of the 5G clock provided by the RAN node. Such granularity has already been sufficient for IIoT scenarios. As for TRS feature, the same granularity is sufficient for the target scenarios. And, the unit of Clock Accuracy IE in the Clock Quality Acceptance Criteria also take 25ns.

- The unit of Clock Accuracy Value IE in the RAN Timing Synchronisation Status Information IE is 25ns.

- The unit of Clock Accuracy IE in the Clock Quality Acceptance Criteria IE is 25ns.

## 2.3 Stage 3 details

There are still FFSs in the XnAP BLCR, e.g., the Clock Quality Acceptance Criteria. It can take the same format as the one in the NGAP BLCR.

- For XnAP, specify the details for Clock Quality Acceptance Criteria IE, and remove the FFS.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

- It is up to the DU’s implementation to trigger the timing synchronization status report to the CU. A note can be added in the BLCR for TS 38.401.

- The unit of Clock Accuracy Value IE in the RAN Timing Synchronisation Status Information IE is 25ns.

- The unit of Clock Accuracy IE in the Clock Quality Acceptance Criteria IE is 25ns.

- For XnAP, specify the details for Clock Quality Acceptance Criteria IE, and remove the FFS.

The TPs to TS 38.413, TS 38.473, TS 38.423 and TS 38.401 are provided in Annexes respectively.

# 4. Reference

- R3-235941, LS on timing resiliency, 3GPP TSG-RAN WG3 Meeting #121bis.

- R3-234823, (BLCR to 38.473) Introduction of 5G Timing Resiliency and URLLC enhancements, 3GPP TSG-RAN WG3 Meeting #121.

- 3GPP TS 23.501 V18.3.0, System architecture for the 5G System (5GS); Stage 2 (Release 18).

- R3-235941, LS on timing resiliency, 3GPP TSG-RAN WG3 Meeting #121bis.

- R3-235750, Summary of Offline Discussion for CB # R18URLLC, 3GPP TSG-RAN WG3 #121bis.

# Annex A – TP to BLCR for TS 38.413 (on top of R3-236009)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | BIT STRING { locked (0), holdover (1), freeRun (2) } (SIZE (8, …)) | Each position in the bitmap represents a synchronisation state. If a bit is set to “1”, the respective synchronisation state is acceptable. If a bit is set to “0”, the respective synchronisation state is not acceptable. Bits 3-7 reserved for future use. |
| Traceable to UTC | O |  | ENUMERATED (true, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | INTEGER (1..40000000, …) | Clock accuracy expressed in units of 25 ns. |
| Parent Time Source | O |  | BIT STRING { syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4), serialTimeCode (5), nTP (6), handset (7), other (8) } (SIZE (16, …)) | Each position in the bitmap represents a parent time source. If a bit is set to “1”, the respective parent time source is acceptable. If a bit is set to “0”, the respective parent time source is not acceptable. Bits 9-15 reserved for future use. |

#### 9.3.1.x3	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided towards the AMF as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | 9.3.1.x5 |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

#### 9.3.1.x5	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >Value |  |  |  |  |
| >>Clock Accuracy Value | M |  | INTEGER (1.. 40000000, …) | Indicates the absolute clock accuracy value. Unit in 25ns . |
| >Index |  |  |  |  |
| >>Clock Accuracy Index | M |  | INTEGER (32..47, ...) | Indicates the clockAccuracy enumeration value specified in Table 5 of clause 7.6.2.6 of IEEE Std 1588 [AA]. |

#### 9.3.1.x6	RAN TSS Scope

This IE indicates the scope of the RAN timing synchronisation status as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE RAN TSS scope | M |  |  |  |
| >RAN node level |  |  |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >cell list level |  |  |  |  |
| >>RAN TSS Cell List |  | 1 |  |  |
| >>>RAN TSS Cell Item |  | 1..<maxnoofCellsTSS> |  |  |
| >>>>NR CGI | M |  | 9.3.1.7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsTSS | Maximum no. of Cell IDs in the scope of the RAN timing synchronisation status. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex B – TP to BLCR for TS 38.473 (on top of R3-236011)

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x1	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [21]. |
| Clock Accuracy | O |  | 9.3.1.x3 |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

#### 9.3.1.x3	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >Value |  |  |  |  |
| >>Clock Accuracy Value | M |  | INTEGER (1.. 40000000, …) | Indicates the absolute clock accuracy value expressed in units of 25 ns. |
| >Index |  |  |  |  |
| >>Clock Accuracy Index | M |  | INTEGER (32..47, ...) | Indicates the clockAccuracy enumeration value specified in Table 5 of clause 7.6.2.6 of IEEE Std 1588 [AA]. |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

# Annex C – TP to BLCR for TS 38.423 (on top of R3-236010)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x1	Clock Quality Reporting Control Information

This IE indicates the clock quality reporting control information as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Quality Detail Level | M |  |  |  |
| >clock quality metrics |  |  |  |  |
| >acceptance indication |  |  |  |  |
| >>Clock Quality Acceptance Criteria | M |  | 9.3.1.x2 |  |

#### 9.2.3.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | BIT STRING { locked (0), holdover (1), freeRun (2) } (SIZE (8, …)) | Each position in the bitmap represents a synchronisation state. If a bit is set to “1”, the respective synchronisation state is acceptable. If a bit is set to “0”, the respective synchronisation state is not acceptable. Bits 3-7 reserved for future use. |
| Traceable to UTC | O |  | ENUMERATED (true, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [21]. |
| Clock Accuracy | O |  | INTEGER (1..40000000, …) | Clock accuracy expressed in units of 25 ns. |
| Parent Time Source | O |  | BIT STRING { syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4), serialTimeCode (5), nTP (6), handset (7), other (8) } (SIZE (16, …)) | Each position in the bitmap represents a parent time source. If a bit is set to “1”, the respective parent time source is acceptable. If a bit is set to “0”, the respective parent time source is not acceptable. Bits 9-15 reserved for future use. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex D – TP to BLCR for TS 38.401 (on top of R3-235131)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.x	Timing resiliency service

The following clauses describe the overall procedures for RAN Timing Synchronisation Status (TSS) reporting involving gNB-CU/gNB-DU.

### 8.x.1	RAN TSS reporting towards the CN

The signaling flow for RAN TSS reporting towards the CN is shown in Figure 8.x.1-1. This procedure is used when the TSCTSF subscribes to RAN TSS reporting at the AMF as described in TS 23.502 [x].

Figure 8.x.1-1:	RAN TSS reporting towards the CN

0.	The gNB-DU is pre-configured with thresholds for each RAN TSS attribute it supports.

NOTE 1: 	It is assumed the pre-configured thresholds in the gNB-DU are sufficient to meet UE time sync performance requirement which are configured by the operator.

1.	The AMF requests RAN TSS reporting by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-CU.

2.	The gNB-CU requests RAN TSS reporting from at least one gNB-DU by sending the TIMING SYNCHRONISATION STATUS REQUEST message to the gNB-DU.

NOTE 2:	It is up to gNB-CU implementation whether to send the request to all its gNB-DUs or to a particular subset of its gNB-DUs, depending on e.g. network topology.

3.	The gNB-DU replies to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS RESPONSE message.

4.	The gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS RESPONSE message. If the gNB-CU does not receive a successful response from at least one gNB-DU, the gNB-CU replies to the AMF by sending the TIMING SYNCHRONISATION STATUS FAILURE message and the flow stops at this step.

5. 	Upon sending the response to the gNB-CU, the gNB-DU provides a first RAN TSS report to the gNB-CU by sending the TIMING SYNCHRONISATION STATUS REPORT message.

NOTE 3:	The RAN TSS attributes included in the report is up to gNB-DU implementation.

6.	The gNB-CU sends the TIMING SYNCHRONISATION STATUS REPORT message to the AMF. The message contains the RAN TSS attributes received from the gNB-DU and the RAN TSS Scope IE to indicate whether the scope of the RAN TSS report is “RAN node level” or “cell list level”.

7.	Later, the gNB-DU detects a primary source event, for example a RAN TSS attribute exceeds a pre-configured threshold (i.e. status degradation) or a RAN TSS attribute meets the threshold again (i.e. status improvement).

8.	Upon detecting the primary source event (e.g., degradation, failure, recovery), the gNB-DU provides an updated RAN TSS report to the gNB-CU by sending a TIMING SYNCHRONISATION STATUS REPORT message.

NOTE 4:	In addition to primary source event, it is also up to the gNB-DU’s implementation to determine when to provide an updated RAN TSS report to the gNB-CU so that the gNB-CU can decide the clock quality status for UEs with clock quality detail level equalling “acceptable/not acceptable indication”.

9. 	Same as step 6.

### 8.x.2	RAN TSS reporting towards the UE

The signaling flow for RAN TSS reporting towards the UE in RRC_CONNECTED state is shown in Figure 8.x.2-1.

Figure 8.x.2-1:	RAN TSS reporting towards the UE in RRC_CONNECTED state

NOTE 1:	In this signalling flow, it is assumed that RAN TSS reporting is already enabled at the gNB-DU.

1.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU, containing the Clock Quality Reporting Control Information IE within the Time Synchronization Assistance Information IE. The clock quality reporting control information indicates the clock quality detail level to provide to the UE, i.e. “metrics” or “acceptable/not acceptable indication”. If clock quality detail level equals “acceptable/not acceptable indication”, the clock quality reporting control information also contains the clock quality acceptance criteria.

NOTE 2:	The clock quality reporting control information can also be provided in the UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, or PATH SWITCH REQUEST ACKNOWLEDGE messages.

2.	The gNB-CU replies to the AMF by sending the INITIAL CONTEXT SETUP RESPONSE message.

3.	The gNB-CU sends the latest clock quality information to the UE by sending the DLInformationTransfer message. The clock quality information provided to the UE depends on the clock quality detail level received in step 1 (i.e., “metrics” or “acceptable/not acceptable indication”).

4.	Later, the gNB-DU detects a primary source event, for example a RAN TSS attribute exceeds a pre-configured threshold (i.e. status degradation) or a RAN TSS attribute meets the threshold again (i.e. status improvement).

5.	Upon detecting the primary source event (e.g., degradation, failure, recovery), the gNB-DU provides an updated RAN TSS report to the gNB-CU by sending a TIMING SYNCHRONISATION STATUS REPORT message.

NOTE 3:	In addition to primary source event, it is also up to the gNB-DU’s implementation to determine when to provide an updated RAN TSS report to the gNB-CU so that the gNB-CU can decide the clock quality status for UEs with clock quality detail level equalling “acceptable/not acceptable indication”.

6.	Same as step 3.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>