3GPP TSG-RAN WG3 Meeting #122	R3-237380

Chicago, US, November 13 – 17, 2023

Title: 	(TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Mobility Enhancements use case

Source: 	Huawei

Agenda item:	12.2.2.2

Document Type:	Discussion and decision

# 1. Introduction

In last RAN3#121bis meeting the following agreements and left issues were captured concerning the inference-based Mobility Enhancements (ME) use case [1][2]:

Encode the Measured UE Trajectory with new IE with Global NG-RAN Cell Identity and Time UE Stays in Cell as mandatory. Define a new Time Duration IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message. FFS on the details of the Time Duration IE. Define a new Number of Visited Cell IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message. FFS on the details of the Number of Visited Cell IE. The maximum number of cells for predicted UE trajectory is 16. The maximum number of cells for measured UE trajectory is 16. No consensus on signaling of an explicit indication of exit conditions for the collection of measured UE trajectory.

In this paper, we discuss the above listed left issues for enabling the ME use case by means of AI/ML.

# 2. Discussion

## 2.1 Measured UE trajectory collection and reporting

In last RAN3 meeting there was a significant progress towards the definition of the configuration to be used by the target NG-RAN node for the collection and reporting of the measured UE trajectory. Similar as for the Load Balancing (LB) use case concerning the UE Performance Feedback collection at and reporting from the requested NG-RAN node, RAN3 agreed to introduce a dedicated measurement collection time window for the collection of the measured UE trajectory information (to be reported only once, as per the agreement in RAN3#121 meeting [3]), which is different from the measurement collection time window introduced for the collection of the UE Performance Feedback [1]:

The measurement collection durations for UE performance feedback and UE trajectory are not expressed as single IE.

Define a new Time Duration IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message.

FFS on the details of the Time Duration IE.

Moreover, by recalling the list of triggering (exit) conditions for the one-time reporting of the measured UE trajectory agreed in RAN3#121 meeting [3]:

The conditions triggering the one-time reporting are at least:

- Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE);

- Condition B: UE leaves the target node;

- Condition C: Expiration of the report time duration;

- Condition D: The configured number of hand-overs within the same target node is reached.

RAN3 agreed that Condition D above needs to be introduced in the DATA COLLECTION REQUEST message [1], and Condition D is now revised as Number of Visited Cell IE for alignment with the current description of the UE History Information (UHI).

Define a new Number of Visited Cell IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message.

FFS on the details of the Number of Visited Cell IE.

The above agreements have been captured in the latest available version of the BLCR for TS 38.423 in [4], and a new IE Additional UE Trajectory Reporting Conditions (FFS on the IE name) has been introduced in the DATA COLLECTION REQUEST message.

Let’s now focus on the FFSs from the last meeting reported above. In a very similar way as proposed in our paper for the LB use case in [5], let’s consider the scenario where the collection and reporting of the measured UE trajectory by the target NG-RAN node after an inference-based ME action (handover) are configured via a Data Collection Reporting Initiation procedure which is also used to configure the reporting of other items/objects, as in Figure 1: here, the same Data Collection Reporting Initiation procedure is used to configure:

- periodic reporting of the Predicted Radio Resource Status, reported with periodicity expressed by the existing Reporting Periodicity IE == Trep, and

- one-time reporting of the measured UE trajectory, collected after the successful handover execution within the agreed Time Duration (‘Measured UE trajectory measurement collection duration’ in Figure 1) and reported immediately after the expiration of such Time Duration.

Figure 1 – The same Data Collection Reporting Initiation procedure is used to configure the reporting of multiple items/objects: periodic reporting of Predicted Radio Resource Status and one-time reporting of the measured UE trajectory.

In the scenario as per Figure 1 we are assuming that, during the collection of the measured UE trajectory information within the agreed Time Duration, no UE events occur, i.e., only Condition C: Expiration of the report time duration holds. To reach a common solution with the LB use case for the UE Performance Feedback collection and reporting, at the time at which the Time Duration expires, the measured UE trajectory should be sent to the source NG-RAN node within the first available DATA COLLECTION UPDATE message (DATA COLLECTION UPDATE message #5 in Figure 1), along with a new prediction of the Radio Resource Status.

Such approach, however, can be problematic in case there are no DATA COLLECTION UPDATE messages sent periodically because, e.g., there are no items/objects (such as predictions) that need to be reported periodically but only one-time, refer to the case where the one-time reporting of the Energy Cost (EC) from target NG-RAN node to the source NG-RAN node is requested together with the measured UE trajectory reporting.

In order to address this issue, it should be noted that the reporting of the measured UE trajectory is only one-time, therefore we think that the easiest way is to allow sending a single DATA COLLECTION UPDATE message with only the measured UE trajectory immediately after the expiration of the Time Duration. Figure 2 shows the concept.

Figure 2 – One-time reporting of the measured UE trajectory within a dedicated DATA COLLECTION UPDATE message sent at the expiration of the Time Duration (‘Measured UE trajectory measurement collection duration’).

The same principle applies in case a UE event occurs, i.e., triggering conditions other than Condition C: Expiration of the report time duration are fulfilled, e.g. the UE is handed over to another target NG-RAN node or the UE changes its RRC state (it goes to idle/inactive from connected). In such case the target NG-RAN node stops collecting the measured UE trajectory and reports the collected information up to the time at which such event occurred; the collected information is sent within a single dedicated DATA COLLECTION UPDATE message, exactly as for the case of no UE events occurred, refer to Figure 3.

Figure 3 – One-time reporting of the measured UE trajectory within a dedicated DATA COLLECTION UPDATE message sent immediately after a UE event occurs.

Hence, based on the analysis done so far, we propose the following:

Proposal 1: RAN3 to agree that, for the one-time reporting of the measured UE trajectory, if at least one of the reporting triggering conditions is fulfilled the target NG-RAN node will report the collected information to the source NG-RAN node within a dedicated DATA COLLECTION UPDATE message, which is sent independently from any other (periodic) DATA COLLECTION UPDATE message.

The TP to the BLCR for TS 38.423 in the Annex reflects the above proposal and provides changes to the agreed new IE Additional UE Trajectory Reporting Conditions (FFS on the IE name). The TP also addresses the FFS on the details of the Number of Visited Cell IE.

## 2.2 On the need of the UE trajectory prediction subscription

In RAN3#121 meeting there was a proposal in [6] to specify the subscription mechanism to allow the target NG-RAN node to request the UE trajectory prediction to be then provided by the source NG-RAN node within the HANDOVER REQUEST message. By referring to Figure 4, a bit in the Report Characteristics IE of the DATA COLLECTION REQUEST message (previously known as AI/ML INFORMATION REQUEST) is introduced to request the UE trajectory prediction and, if this bit is set to “1”, the node accepting the request (Node2 in the endorsed BLCR to TS 38.423 in [4]) shall send the Cell Based UE Trajectory Prediction IE in the HANDOVER REQUEST message.

Figure 4 - Subscription mechanism to request the UE trajectory prediction.

However, RAN3 was not able to reach consensus and the following FFS was minuted in [3]:

Whether the predicted UE Trajectory report needs to be indicated in the Report Characteristics IE of DATA COLLECTION REQUEST message is FFS.

We think that the AI/ML model in the source NG-RAN node could predict and send the UE trajectory without any subscription-based mechanism, also considering the fact that there is no guarantee that the target NG-RAN node will base the UE handover decision according to such UE trajectory prediction even if the target NG-RAN node subscribes for being provided with this information. Therefore, introducing a subscription-based mechanism to request the UE trajectory prediction is not needed, and the signalling over Xn required to implement such mechanism seems not motivated, hence we have the following proposal:

Proposal 2: RAN3 to agree that there is no need to introduce the UE trajectory prediction bit in the Report Characteristics IE of the DATA COLLECTION REQUEST message.

## 2.3 Remaining issues

- Similarly for the other RAN AI/ML use case, we think that the (next-hop) target NG-RAN node (i.e., the requested node) should be able to distinguish an incoming handover request originated from the source NG-RAN node (i.e., the requesting node) for the purpose of inference-based ME with respect to a “legacy” (i.e., not inference-based) handover request; this would allow the requested node to reject such incoming handover request due to the fact that mobility enhancement actions from the requesting node are based on predictions, hence not absolutely accurate. Another reason motivating the need of such indication within the handover request is that a mobility action from the requesting node could be in contrast with other inferred strategies considered in the requested node (e.g., energy saving or load balancing strategies), hence we think RAN3 should discuss and agree whether an incoming handover for the purpose of inference-based ME should be identifiable by the target NG-RAN node.

- Proposal 3: RAN3 to discuss and agree whether an incoming handover for the purpose of inference-based ME should be identifiable by the target NG-RAN node.

- Once the target cell and target NG-RAN node are selected, the source NG-RAN node should inform the target NG-RAN node of the time of handover before it starts. In our view, the target NG-RAN node should be aware of the timestamp of the inference-based handover, in order to prepare for the handover resource timely.

- Proposal 4: RAN3 to agree that the source NG-RAN node informs the target NG-RAN node of the handover timestamp related to the inference-based handover.

- The corresponding TP to TS 38.423 reflecting the above proposals is provided in the Annex.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

Proposal 1: RAN3 to agree that, for the one-time reporting of the measured UE trajectory, if at least one of the reporting triggering conditions is fulfilled the target NG-RAN node will report the collected information to the source NG-RAN node within a dedicated DATA COLLECTION UPDATE message, which is sent independently from any other (periodic) DATA COLLECTION UPDATE message.

Proposal 2: RAN3 to agree that there is no need to introduce the UE trajectory prediction bit in the Report Characteristics IE of the DATA COLLECTION REQUEST message.

- Proposal 3: RAN3 to discuss and agree whether an incoming handover for the purpose of inference-based ME should be identifiable by the target NG-RAN node.

- Proposal 4: RAN3 to agree that the source NG-RAN node informs the target NG-RAN node of the handover timestamp related to the inference-based handover.

# 4. Reference

- 3GPP RAN3#121bis Chairman’s notes (RAN3_121bis_agenda_20231013_EOM1.doc)

- R3-235739, Summary of Offline Discussion on CB: # AIRAN3_ME, ZTE

- 3GPP RAN3#121 Chairman’s notes (RAN3_121_agenda_20230825_EOM1.doc)

- R3-235955, (BLCR to 38.423) for AI/ML for NG-RAN, Ericsson, Nokia, Nokia Shanghai Bell

- R3-237379, (TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Load Balancing use case, Huawei

- R3-234295, Cell based UE trajectory prediction configuration, Ericsson, InterDigital, Deutsche Telekom

# 5. Annex: TP to 38.423 (based on BLCR in R3-235955)

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

****** skip unchanged parts ******

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Cell Based UE Trajectory Prediction IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider the content of this list as the predicted cells for UE trajectory by the source NG-RAN node, and may use it for e.g. mobility decisions.

If the Intended Timestamp for Handover Execution IE is contained in the HANDOVER REQUEST message, the target NG-RAN node considers the time as the predicted time the UE accesses the target cell, and may use it for e.g. access parameter optimization.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.AA	Data Collection Reporting Initiation

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of information to another NG-RAN node to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request IE is set to “stop”; or

-	FFS

If the Registration Request IE is set to “start” in the DATA COLLECTION REQUEST message and the Report Characteristics IE indicates cell-specific information reporting, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable  of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Per Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the Measurement Collection and Reporting Time Duration IE contained in the UE Trajectory Collection and Reporting Configuration IE in the DATA COLLECTION REQUEST message is present, the NG-RAN node2 shall start collecting the measured UE trajectory information and shall use this value as a triggering condition for reporting the measured UE Trajectory. If the Number of visited cells IE contained in the UE Trajectory Collection and Reporting Configuration IE in the DATA COLLECTION REQUEST message is present, the NG-RAN node2 shall use this value as a triggering condition for reporting the UE Trajectory.

If the UE Performance Configuration (FFS on the name) IE in the DATA COLLECTION REQUEST message is present, it indicates the configuration for the collection of UE performance. The NG-RAN node2 shall take it into account when collecting the UE performance.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Cell Based UE Trajectory Prediction | O |  | 9.2.3.x | It contains information about cells that a UE is predicted to be connected to | YES | ignore |
| Data Collection ID | O |  | 9.2.3.M |  | YES | Ignore |
| Intended Timestamp for Handover Execution | O |  | INTEGER (0..1000000, …) | Expressed in units of 1ms. | – | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.CC	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. For periodic reporting, it indicates the points in time, measured from the reception of the DATA COLLECTION REQUEST message and shifted by each reporting period, for which predictions are provided. | YES | ignore |
| UE Trajectory Collection and Reporting Configuration | O |  | 9.2.3.P |  | YES | ignore |
| UE Performance Configuration (FFS on the name) | O |  | 9.2.3.N |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.Z	Predicted Trajectory Cell Information

The Predicted Trajectory Cell Information IE contains the cell ID of the predicted NG-RAN cell for cell based UE trajectory prediction, and optionally the time the UE is expected to stay in the predicted NG-RAN cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Predicted Trajectory Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |
| >>Predicted Time UE Stays in Cell | O |  | INTEGER (0..4095) | The duration of time the UE is expected to stay in the cell, in seconds. If the duration is more than 4095s, this IE is set to 4095. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.P2	Measured Trajectory Cell Information

The Measured Trajectory Cell Information contains the cell ID of the NG-RAN cell where a UE connected after being handed over to the target NG-RAN node, and the time the UE stayed in the NG-RAN cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Measured Trajectory Cell Information |  |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 |  |
| >>Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, in seconds. If the duration is more than 4095s, this IE is set to 4095. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.P	UE Trajectory Collection and Reporting Configuration

This IE contains conditions triggering the NG-RAN node2 to collect and report the measured UE Trajectory after successful handover execution.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement Collection and Reporting Time Duration | O |  | INTEGER (1..120) | The maximum time duration, in minutes, starting from the successful handover execution, within which the measured UE trajectory is collected and at the expiration of which the NG-RAN node2 reports the measured UE Trajectory. |
| Number of visited cells | O |  | INTEGER (1..16) | Maximum number of intra- NG-RAN node2 cells where the UE connects following the successful handover execution, and after which the NG-RAN node2 reports the measured UE Trajectory. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the XnAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Set ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Resources not available for the slice(s), UE Maximum integrity protected data rate reason, CP Integrity Protection Failure, UP Integrity Protection Failure, Slice(s) not supported by NG-RAN, MN Mobility, SN Mobility, Count reaches max value, Unknown Old NG-RAN node UE XnAP ID, PDCP Overload, DRB ID not available, Unspecified, …, UE Context ID not known, Non-relocation of context, CHO-CPC resources to be changed, RSN not available for the UP, NPN access denied, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, UE Power Saving, Not existing NG-RAN node2 Measurement ID, Insufficient UE Capabilities, Normal Release, Value out of allowed range, SCG activation deactivation failure, SCG deactivation failure due to data transmission, Handover Desirable for inference-based mobility enhancements) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

The meaning of the different cause values is specified in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Cell not Available | The concerned cell is not available. |
| Handover Desirable for Radio Reasons | The reason for requesting handover is radio related. |
| Handover Target not Allowed | Handover to the indicated target cell is not allowed for the UE in question. |
| Invalid AMF Set ID | The target NG-RAN node doesn’t belong to the same AMF Set of the source NG-RAN node, i.e. NG handovers should be attempted instead. |
| No Radio Resources Available in Target Cell | The target cell doesn’t have sufficient radio resources available. |
| Partial Handover | Provides a reason for the handover cancellation. The target NG-RAN node did not admit all PDU Sessions included in the HANDOVER REQUEST and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
| Reduce Load in Serving Cell | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| Resource Optimisation Handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Value out of allowed range | The action failed because the proposed Handover Trigger parameter change in the NG-RAN node2 Proposed Mobility Parameters IE is too low or too high. |
| Time Critical Handover | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| TXnRELOCoverall Expiry | The reason for the action is expiry of timer TXnRELOCoverall. |
| TXnRELOCprep Expiry | Handover Preparation procedure is cancelled when timer TXnRELOCprep expires. |
| Unknown GUAMI ID | The target NG-RAN node belongs to the same AMF Set of the source NG-RAN node and recognizes the AMF Set ID. However, the GUAMI value is unknown to the target NG-RAN node. |
| Unknown Local NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node does not recognise the local NG-RAN node UE XnAP ID. |
| Inconsistent Remote NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node considers that the received remote NG-RAN node UE XnAP ID is inconsistent.. |
| Encryption And/Or Integrity Protection Algorithms Not Supported | The target NG-RAN node is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided to the NG-RAN node. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided to the NG-RAN node. |
| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving NG-RAN node in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| TXnDCoverall Expiry | The reason for the action is expiry of timer TXnDCoverall. |
| TXnDCprep Expiry | The reason for the action is expiry of timer TXnDCprep |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity only. |
| Reduce Load | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity only. |
| Resource Optimisation | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity only. |
| Time Critical action | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity only. |
| Target not Allowed | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity only. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity only. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity only. |
| Encryption Algorithms Not Supported | The requested NG-RAN node is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity only. |
| RRM purpose | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity only. |
| Improve User Bit Rate | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity only. |
| User Inactivity | The action is requested due to user inactivity on all PDU Sessions. The action may be performed on several levels: - on UE Context level, if NG is requested to be released in order to optimise the radio resources; or S-NG-RAN node didn’t see activity on the PDU session recently. - on PDU Session Resource or DRB or QoS flow level, e.g. if Activity Notification indicate lack of activity In the current version of this specification applicable for Dual Connectivity only. |
| Radio Connection With UE Lost | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Failure in the Radio Interface Procedure | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity only. |
| Bearer Option not Supported | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity only. |
| UP integrity protection not possible | The PDU session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU session cannot be accepted according to the required user plane confidentiality protection policy. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| UE Maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum data rate for integrity protection supported by the UE. |
| CP Integrity Protection Failure | The request is not accepted due to failed control plane integrity protection. |
| UP Integrity Protection Failure | The procedure is initiated because the SN (hosting node) detected an Integrity Protection failure in the UL PDU coming from the MN. |
| Slice(s) not supported by NG-RAN | The failure is due to slice(s) not supported by the NG-RAN node. |
| MN Mobility | The procedure is initiated due to relocation of the M-NG-RAN node UE context. |
| SN Mobility | The procedure is initiated due to relocation of the S-NG-RAN node UE context. |
| Count reaches max value, | Indicates the PDCP COUNT for UL or DL reached the max value and the bearer may be released. |
| Unknown Old NG-RAN node UE XnAP ID | The action failed because the Old NG-RAN node UE XnAP ID or the S-NG-RAN node UE XnAP ID is unknown. |
| PDCP Overload | The procedure is initiated due to PDCP resource limitation. |
| DRB ID not available | The action failed because the M-NG-RAN node is not able to provide additional DRB IDs to the S-NG-RAN node. |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| UE Context ID not known | The context retrieval procedure cannot be performed because the UE context cannot be identified. |
| Non-relocation of context | The context retrieval procedure is not performed because the old RAN node has decided not to relocate the UE context. |
| CHO-CPC resources to be changed | The prepared resources for CHO or CPC for a UE are to be changed. |
| RSN not available for the UP | The redundant user plane resources are not available. |
| NPN Access denied | Access denied, or release is required, due to NPN reasons. |
| Report Characteristics Empty | The action failed because there is no measurement object in the report characteristics. |
| Existing Measurement ID | The action failed because the measurement ID is already used. |
| Measurement Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object. |
| Measurement not Supported For The Object | At least one of the concerned object(s) does not support the requested measurement. |
| UE Power Saving | The procedure is initiated to accommodate the preference indicated by UE to release the S-NG-RAN node for UE power saving purpose. |
| Not existing NG-RAN node2 Measurement ID | The action failed because the NG-RAN node2 Measurement ID is not used. |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |
| Normal Release | The release is due to normal reasons. |
| SCG activation deactivation failure | The action failed due to rejection of the SCG activation deactivation request. |
| SCG deactivation failure due to data transmission | The SCG deactivation failure due to ongoing or arriving data transmission. |
| Handover Desirable for inference-based mobility enhancements | The reason for requesting this action is inference-based mobility enhancements strategies. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>