3GPP TSG-RAN3 Meeting #122	R3-237383

# Chicago, USA, November 13 – 17, 2023

Agenda item:	11.2

Source: 	Xiaomi

Title: 	(TP for TS 38.300) QMC support for RRC_IDLE

Document for:	Discussion & decision

Introduction

In this contribution, we’d like to discuss QMC support for RRC_IDLE UE. In previous meeting, UE-based solution and CN-based solution had been discussed, and UE-based solution is taken as working assumption in RAN3#121-bis meeting.

Considering this is the last meeting for R18 discussion, in this paper, we discussed the possible impacts for all possible options including the compromise option proposed during the offline discussion, for RAN3 to down-select in this meeting.

Discussion

Information needed for new gNB

In previous RAN3 meetings, the following information is agreed to be provided to the new gNB when UE switches from RRC_IDLE to RRC_CONNECTED state. Part of the information is already available in UE. And for core-network, all the QoE configuration are not available in AMF, for s-based QoE configuration, although the AMF participates the configuration, but it will not store it as UE context, according to TS 23.501 (i.e. there is no QoE related UE context in AMF), the s-based QoE configuration may be stored in UDM, for m-based QoE configuration, the CN has no idea of it. The following table includes the detail analysis.

| Information to be informed to new gNB | Available Information in UE | Available Information in CN |
| --- | --- | --- |
| QoE reference | No | For m-based QoE: No For s-based QoE: yes in UDM, no in AMF |
| Measurement Collection Entity Information | No | For m-based QoE: No For s-based QoE: yes in UDM, no in AMF |
| QoE measurement type (s-based or m-based measurement) | No | No |
| RRC level ID (measConfigAppLayerID) | Yes | No |
| Service type | No | For m-based QoE: No For s-based QoE: yes in UDM, no in AMF |
| Available RVQOE metrics | No | For m-based QoE: No For s-based QoE: yes in UDM, no in AMF |
| Session status | Yes (can only be provided by UE) | No |
| Area Scope | Yes | For m-based QoE: No For s-based QoE: yes in UDM, no in AMF |

For s-based QoE configuration, although AMF can know it when AMF receives from UDM, AMF does not know whether and when the s-based QoE configuration is send to UE (which is based on gNB’s area scope check and slice scope check), so the NG-RAN node needs to notify the s-based QoE configuration that has been configured to UE including the APP ID to the AMF, then AMF can store the corresponding configurations as UE context.

Observation 1, although AMF participate s-based QoE configuration, there is no s-based QoE information in UE context in AMF.

Observation 2, when s-based QoE configuration is sent to UE depends on the gNB’s scope check, AMF is not aware of whether the s-based QoE configuration is sent to UE or whether the s-based QoE configuration is already released.

Observation 3, AMF is not aware of the m-based QoE configuration configured to UE.

Observation 4, AMF cannot provide the sessions status of the QoE configuration.

CN-based solution

The following figure shows the possible call flow for CN-based solution.

Figure 1 CN-based solution

The possible specification impacts are marked in blue in the above figure.

At step 3, if UE has been configured with QoE configuration, the NG-RAN node needs to send the QoE configuration information (including the information discussed in 2.1) for both m-based and s-based to the AMF, the UE context release complete message should be enhanced, and the behaviour of old AMF should be updated for this purpose.

At step 6, when UE setup RRC connection with a new gNB, it needs to notify new gNB the list of application ID for QoE configurations, and also the configuration status (please note that the configuration status can be implicitly indicated by the presence of the APP ID) and session status for each QoE configuration.

At step 7, the new gNB send the initial UE message to the new AMF, the new gNB should indicate the QoE configurations it wants to retrieve from the core network, so the QoE configuration request including the list of APP IDs needs to be included in Initial UE message.

At step 8, if the AMF is changed, the new AMF needs to fetch the QoE configurations from the old AMF. This have impacts on SA2 and CT4.

At step 9, the new AMF send the QoE configurations including new information (e.g. configuration type) in the QoE configurations.

The summary of spec impacts for CN-based solution can be as follows:

| RAN2 impacts | RAN3 impacts | SA2 impacts | CT4 impacts |
| --- | --- | --- | --- |
| Enhance RRC Setup Complete message | Enhance the following messages and IEs: - UE context release complete message - Initial UE message - UE Application Layer Measurement Configuration Information | Adding the following behaviour for AMF: - Store s-based QoE configuration if it’s configured to UE. - Store m-based QoE configurations received from gNB - Store s-based QoE configurations - Fetch QoE configurations for a UE between AMFs | Adding new IEs for QoE configurations between AMFs |

Observation 5, if CN-based solution is adopted, the impacted WGs include RAN2, RAN3, SA2 and CT4.

Observation 6, the issue for CN-based solution is that the UE may release the QoE configurations already configured in UE, but Core network have no idea of it, this will lead to waste of core network storage resources.

Observation 7, if CN-based solution is adopted, the UE needs to inform the configuration status for each configuration, the new gNB only needs to request and get the available QoE configurations in UE from CN.

Proposal 1, if CN-based solution is adopted, RAN3 needs to enhance the following procedures and IEs:

- Include m-based QoE configuration in UE context release complete message.

- Include QoE request including the APP IDs in Initial UE message.

- Include configuration type in UE Application Layer Measurement Configuration Information IE.

Proposal 2, if CN-based solution is adopted, RAN3 send LS to RAN2/SA2/CT4 to support CN-based solution, including:

- Notify RAN2 to introduce configuration status.

- Notify SA2/CT4 to introduce new AMF behaviour and signalling between AMFs.

UE-based solution

The following figure shows the possible call flow for UE-based solution.

Figure 2 UE-based solution

At step 0, when configuring UE with the QoE configuration that can be applied for RRC_ILDE mode, the gNB also include the additional configuration information in the QoE configuration, e.g. the QoE reference, service type, configuration type, available RVQoE metrics, the additional configuration information will be kept when UE enters to RRC_IDLE mode.

At step 4, after UE setup RRC with the new gNB, it includes the QoE configuration information for the new gNB, including the sessions status as well in the RRC setup complete message.

The summary of spec impacts for UE-based solution can be as follows:

| RAN2 impacts | RAN3 impacts | SA2 impacts | CT4 impacts |
| --- | --- | --- | --- |
| Enhance the following message and IE: - appLayerMeasConfig - RRC Setup Complete message | No impacts | No impacts | No impacts |

Observation 8, if UE-based solution is adopted, only RAN2 will be impacted.

Proposal 3, if UE-based solution is adopted, send LS to RAN2 to notify them to enhance the following message and IE:

-	appLayerMeasConfig IE

-	RRC Setup Complete message.

UE-based-bis solution

According to the previous discussion, the concerns on UE-based solution are UE storage and security. Although we don’t think these are issues, as RAN2 replies that the existing UE storage space can be used, and SA3 doesn’t provide any feedback. If companies are interested in a compromised UE-based solution, we proposed the following UE-based-bis solution to address the concerns.

Figure 3 UE-based-bis solution

At step 0, when configuring UE with the QoE configuration that can be applied for RRC_ILDE mode, the gNB include the QoE reference in the QoE configuration.

At step 4, after UE setup RRC with the new gNB, it includes QoE reference, configuration status and session status in the RRC setup complete message.

At step 5, the new gNB can request and get the available QoE configurations from OAM by indicating the QoE reference by implementation. We think this can be captured in stage2.

Observation 9, for UE-based-bis solution, only QoE reference needs to be indicated to UE.

Observation 10, QoE reference for QoE is similar to trace id for MDT, there is no security issue for trace id stored in UE, then there will be no security issue for QoE reference if its’s stored in UE.

Proposal 4, if UE-based-bis solution is adopted, send LS to RAN2 to notify them introduce QoE reference following message and IE:

-	appLayerMeasConfig IE

-	RRC Setup Complete message

In the Annex, we provided stage 2 descriptions for different solutions.

Proposal 5, it is proposed to capture the stage2 descriptions for the solution selected by RAN3.

Conclusion

In this contribution, we had the following observations and proposals:

Information needed for new gNB

Observation 1, although AMF participate s-based QoE configuration, there is no s-based QoE information in UE context in AMF.

Observation 2, when s-based QoE configuration is sent to UE depends on the gNB’s scope check, AMF is not aware of whether the s-based QoE configuration is sent to UE or whether the s-based QoE configuration is already released.

Observation 3, AMF is not aware of the m-based QoE configuration configured to UE.

Observation 4, AMF cannot provide the sessions status of the QoE configuration.

CN-based solution

Observation 5, if CN-based solution is adopted, the impacted WGs include RAN2, RAN3, SA2 and CT4.

Observation 6, the issue for CN-based solution is that the UE may release the QoE configurations already configured in UE, but Core network have no idea of it, this will lead to waste of core network storage resources.

Observation 7, if CN-based solution is adopted, the UE needs to inform the configuration status for each configuration, the new gNB only needs to request and get the available QoE configurations in UE from CN.

Proposal 1, if CN-based solution is adopted, RAN3 needs to enhance the following procedures and IEs:

- Include m-based QoE configuration in UE context release complete message.

- Include QoE request including the APP IDs in Initial UE message.

- Include configuration type in UE Application Layer Measurement Configuration Information IE.

Proposal 2, if CN-based solution is adopted, RAN3 send LS to RAN2/SA2/CT4 to support CN-based solution, including:

- Notify RAN2 to introduce configuration status.

- Notify SA2/CT4 to introduce new AMF behaviour and signalling between AMFs.

UE-based solution

Observation 8, if UE-based solution is adopted, only RAN2 will be impacted.

Proposal 3, if UE-based solution is adopted, send LS to RAN2 to notify them to enhance the following message and IE:

-	appLayerMeasConfig IE

-	RRC Setup Complete message.

UE-based-bis solution

Observation 9, for UE-based-bis solution, only QoE reference needs to be indicated to UE.

Observation 10, QoE reference for QoE is similar to trace id for MDT, there is no security issue for trace id stored in UE, then there will be no security issue for QoE reference if its’s stored in UE.

Proposal 4, if UE-based-bis solution is adopted, send LS to RAN2 to notify them introduce QoE reference following message and IE:

-	appLayerMeasConfig IE

-	RRC Setup Complete message

300 TP

Proposal 5, it is proposed to capture the stage2 descriptions for the solution selected by RAN3.

Annex A (TP for TS 38.300)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<UE-based solution>>>>>>>>

## 21.y	QoE Measurement Collection Handling when UE switches from RRC_IDLE to RRC_CONNECTED

When the gNB sends to the UE the QoE configuration that can be applied for RRC_DILE mode, it may also include the assistance information (e.g. the QoE reference, MCE ID, service type, measurement type, available RVQoE metrics).

The QoE configuration for RRC_IDLE and the corresponding assistance information will be kept when UE enters to RRC_ILDE mode.

After UE switches from RRC_IDLE to RRC_CONNECTED state and setup RRC connection with the new gNB, UE includes the QoE configuration information including the assistance information received from the old gNB in the RRC setup complete message, the new gNB will consider the QoE configuration information when managing the QoE configurations including configuring, releasing and updating the QoE configuration.

<<<<<<<<UE-based-bis solution>>>>>>>>

## 21.y	QoE Measurement Collection Handling when UE switches from RRC_IDLE to RRC_CONNECTED

When the gNB sends to the UE the QoE configuration that can be applied for RRC_DILE mode, it also includes QoE reference.

The QoE configuration including the QoE reference will be kept when UE enters to RRC_ILDE mode.

After UE switches from RRC_IDLE to RRC_CONNECTED state and setup RRC connection with the new gNB, UE includes the QoE reference(s) and the corresponding session status in the RRC setup complete message, the new gNB can request and get the QoE configuration information that is stored in OAM by indicating the corresponding QoE reference(s) received from UE, and the new gNB will consider the QoE configuration information when managing the QoE configurations including configuring, releasing and updating the QoE configuration.

<<<<<<<<CN-based solution>>>>>>>>

## 21.y	QoE Measurement Collection Handling when UE switches from RRC_IDLE to RRC_CONNECTED

When the gNB sends RRC release message to UE, it includes the QoE configuration information that can be applied to RRC_DILE and already configured to UE in the UE context release complete message to AMF.

AMF stores the received QoE configuration information as UE context.

QoE configuration will be kept when UE enters to RRC_ILDE mode.

After UE switches from RRC_IDLE to RRC_CONNECTED state and setup RRC connection with the new gNB, UE includes the RRC level ID(s), configuration status and session status in the RRC setup complete message, the new gNB can request and get the QoE configuration information that is stored in AMF by indicating the corresponding RRC level ID(s), and the new gNB will consider the QoE configuration information when managing the QoE configurations including configuring, releasing and updating the QoE configuration

<<<<<<<<<<<<<<<<<<<< Change End>>>>>>>>>>>>>>>>>>>>