3GPP TSG-RAN WG3 Meeting #122	R3-237493

# Chicago, US, 13th – 17th November 2023

Agenda Item:	12.2.2.2

Source:	Ericsson, Deutsche Telekom, AT&T, InterDigital

Title:	Cell based UE trajectory prediction configuration

Document for:	Discussion and Decision

# 1	Introduction

During RAN3#121bis, the cell-based UE trajectory prediction was discussed at length and the following agreements were captured:

Encode the Measured UE Trajectory with new IE with Global NG-RAN Cell Identity and Time UE Stays in Cell as mandatory.

Define a new Time Duration IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message.

Define a new Number of Visited Cell IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message.

The maximum number of cells for predicted UE trajectory is 16.

The maximum number of cells for measured UE trajectory is 16.

These agreements were captured in the XnAP BL CR, but few aspects remain FFS:

FFS on the details of the Time Duration IE

FFS on the details of the Number of Visited Cell IE.

No consensus on signaling of an explicit indication of exit conditions for the collection of measured UE trajectory.

In addition, some IE names in the XnAP BL CR are still FFS. Therefore, this paper aims at resolving above open issues and it proposes solutions to be captured in RAN3 specifications.

# 2	Discussion

## 2.1 Reporting conditions for Measured UE Trajectory

There was no consensus during RAN3#112bis on the “explicit vs implicit” signaling for the 2 following reporting conditions:

- Condition A: Switch of UE state (e.g. UE goes to RRC_IDLE/RRC_INACTIVE);

- Condition B: UE leaves the target node;

Some companies would like to have these conditions explicitly signaled to the reporting node. However, this would cause some issues. For example, what happens if only the “Number of handovers” IE is sent by the requesting node as reporting condition (and conditions A & B are left out), and if the UE exits the requesting node (or goes to IDLE) before the maximum number of HOs signaled in the condition is fulfilled? Does it mean that the reporting node will never send the Measured UE Trajectory? Also, if the UE goes to idle, or if the UE is handed over to another NG-RAN node, the NG-RAN node may delete the UE context. In that case there is no need to keep the Measured UE Trajectory any longer in the target node but rather to report it to the source node. Therefore, it can be understood that conditions A & B needs to be always active.

In that case, there is no need to have an explicit signaling for conditions A and B because the target node will always report the UE Trajectory when the UE switches state or leaves the target node, if not done already (e.g. via conditions C or D).

Proposal 1: Reporting conditions A and B are always active and are signaled implicitly.

However, these conditions need to be explicitly described in the specifications. And the best location is in the Data Collection Reporting Initiation procedural text, together with the Measured UE Trajectory IE description in the “Interaction with the Data Collection Reporting procedure” section, as it refers to the reporting node and the DATA COLLECTION UPDATE message. And in order to have the full picture (i.e. all 4 reporting conditions), it is proposed to describe all reporting conditions, including C and D.

Proposal 2: Describe all the reporting conditions into the procedure text of the Data Collection Reporting Initiation procedure.

The definition of the already agreed reporting conditions “Time Duration” and “Number of visited cells” are still FFS. However, the version visible in the latest BL CR correspond to the definition discussed during previous meeting:

- The “Time Duration” is the time after which the reporting node should send the Measured UE Trajectory. The value (in seconds) signaled by the requesting node is the length of the dataset needed by the requesting node. Therefore, the reporting node shall not report the Measured UE Trajectory before the expiration of this timer, unless another condition is met beforehand. Also, it is obvious that the time duration starts from the successful HO from source to target node.

- Similarly, the “Number of visited cells” corresponds to the number of cells needed by the requesting node to verify its prediction. There is no need for the reporting node to collect more cells.

The existing definitions correspond to this understanding, and it is therefore proposed to keep it as it is in the BL CR.

Proposal 3: Keep Report Time Duration and Number of handovers IEs as in BL CR and remove FFSs.

It is also proposed to keep the Additional UE Trajectory Reporting Conditions IE name as it is in the BL CR and remove the FFS.

Proposal 4: Keep the Additional UE Trajectory Reporting Conditions IE name as in the BL CR and remove FFS.

## 2.2 Signaling of the triggering reporting condition

One aspect which was discussed and not concluded is the signalling of the triggering reporting condition to the requesting node. As the measure UE trajectory may be used as feedback, it is beneficial for the node performing the prediction to understand what event triggered the report. For example, if the source node predicted that the UE will go from cell B to cell C, both belonging to the target node, it needs to understand that such prediction did not happen because the UE moved from cell B to a cell belonging to a different node than the target node. Or that the UE moved to RRC_IDLE. This would help correcting the ML algorithm. Also, doing this does not cause any additional load for the reporting node, which already knows which condition triggered the report. Therefore, it is proposed to include the condition that triggered the report together with the Measured UE Trajectory, in the DATA COLELCTION UPDATE message.

Proposal 5: Include the condition that triggered the report together with the Measured UE Trajectory, in the DATA COLELCTION UPDATE message.

# 3	Conclusion

In this paper, cell-based UE trajectory prediction configuration and reporting was discussed and the following proposals were made:

Proposal 1: Reporting conditions A and B are always active and are signaled implicitly.

Proposal 2: Describe all the reporting conditions into the procedure text of the Data Collection Reporting Initiation procedure.

Proposal 3: Keep Report Time Duration and Number of handovers IEs as in BL CR and remove FFSs.

Proposal 4: Keep the Additional UE Trajectory Reporting Conditions IE name as in the BL CR and remove FFS.

Proposal 5: Include the condition that triggered the report together with the Measured UE Trajectory, in the DATA COLELCTION UPDATE message.

A TP mirroring the above is presented in R3-237494 and it is proposed to be agreed