3GPP TSG-RAN WG3 Meeting #122	R3-237512

# Chicago, USA, November 13th – 17th 2023

Agenda Item:	10.2.2

Source:	Ericsson

Title:	(TP for SON BL CR for TS 38.413) SON enhancements for Mobility Robustness Optimization

Document for:	Discussion and Decision

# 1	Introduction

During previous meetings, the working assumption to “Define a new handover report type in the Inter-system HO Report over S1 and NG” was turned into an agreement. In this paper we will discuss the remaining stage-3 details, which were left FFS.

MRO for Fast MCG Recovery was also discussed. In this contribution the forwarding of information about Fast MCG Recovery Failure to the SN (e.g. with RLF Report) will be discussed.

The last topic discussed last meeting was MRO for CPAC. RAN3 has selected option 1 has way forward, but a remaining point on the signaling of the “suitable “PSCell” was left open.

# 2	Discussion

## 2.1 MRO for IRAT Handover due to Voice Fallback

## 2.1	Inter-system Mobility Failure for Voice Fallback

In RAN3#121, the working assumption to “Define a new handover report type in the Inter-system HO Report over S1 and NG” was turned into an agreement, and the text proposal R3-234625, introducing Inter-system Mobility Failure for Voice Fallback in the report was agreed. Some details were however left for further study, see excerpt below.

[Parts omitted]

Figure 1: Excerpt from R3-234625, introducing “Inter-system Mobility 
Failure for Voice Fallback” in the Inter-system HO Report

When a failure occurs in LTE shortly after a successful handover from NR triggered due to voice fallback, and the UE reconnects to a new cell (belonging to either an E-UTRAN node or an NG-RAN node), an LTE RLF report including a voice fallback indication may be fetched by the new node. This RLF report may be forwarded to the eNB that served the UE before the failure using existing signalling. The Inter-system HO Report with Handover Report Type set to Inter-system Mobility Failure for Voice Fallback (as agreed in RAN3#121) will then be used to transfer information about the failure back to the source NG-RAN node. The purpose of this is for the NG-RAN node to be able to select a better E-UTRAN node for next voice fallback handover if possible.

In the following, we will go through the FFSs in the Inter-system HO Report type Inter-system Mobility Failure for Voice Fallback, keeping the purpose and usage of the report as described above in mind.

FFS1: Re-connect / Re-establishment Cell: Name, IE type and Semantics description.

The cell in which the UE comes back after the RLF may be of interest for the source NG-RAN cell to identify a more suitable target cell for the voice fallback handover next time. In RAN3#121, the IE name, type and semantics description of the cell ID was left for further study.

For the naming of the cell ID, both “Re-connect Cell ID and “Re-establish Cell ID” were discussed. Considering the intra-RAT handover report for E-UTRAN, one can see that the corresponding IE in case of handover to wrong cell is “Re-establishment cell ECGI” [1]. In the intra-RAT handover report for NG-RAN, the corresponding IE is named “Re-establishment Cell CGI” [2]. The semantics description further clarifies that either attempted re-establishment cell or re-connect cell can be captured in the IE.

- Observation 1.1: In the existing intra-RAT HO report, the term “Re-establishment cell” is used for the cell the UE comes back to after RLF, and the semantics clarify that this is the cell where UE attempted re-establishment or where the UE successfully re-connected after the failure.

To align with the existing intra-RAT HO report, we suggest that the same approach is used for the IE under discussion, i.e. use the term “Re-establishment Cell” and clarify in the semantics description that this could be the attempted re-establishment cell or the re-connect cell.

Further, looking at other IEs in the Inter-system HO report, it can be seen that for the source cell and target cell, the term “ID” is typically used in the IE name instead of “CGI” or “ECGI”, independent of IE type (NG-RAN CGI or E-UTRA CGI) [2]. This is also the case for the “Source Cell ID” and “Failure Cell ID” included in the Inter-system Mobility Failure for Voice Fallback introduced in RAN3#121.

- Observation 1.2: In the existing inter-system HO report, the term “ID” is typically used in the IE name instead of “CGI” or “ECGI”.

To align with the existing inter-system HO report, we suggest that the term “ID” is used in the IE name.

- Proposal 1.1: Change the IE name “Reconnect Cell ID” (currently stated as FFS) in the Inter-system Mobility Failure for Voice Fallback of the Inter-system HO Report to “Re-establishment Cell ID”, and include in the Semantics description that this is the cell where UE attempted re-establishment or where the UE successfully re-connected after the failure.

As mentioned above, the purpose of including the Re-establishment Cell ID in the Inter-system Mobility Failure for Voice Fallback report type of the Inter-system HO Report is to help the source node in identifying a more suitable target cell for coming voice fallback handover. It can be concluded that the suitable target cell for voice fallback handover needs to be an E-UTRAN cell (i.e. where voice service is available), hence it is only of interest to know the re-establishment cell in case the UE comes back in E-UTRAN.

- Observation 1.3: The re-establishment cell after a RLF occurring shortly after a successful handover from NR to LTE triggered due to voice fallback is only of interest if it is an E-UTRAN cell.

We therefore propose to use E-UTRA CGI as the IE type.

- Proposal 1.2: The “Re-establishment Cell ID” IE type in the Inter-system Mobility Failure for Voice Fallback of the Inter-system HO Report should be set to E-UTRA CGI.

FFS2: Presence of UE RLF Report Container

The presence of the UE RLF Report Container in the in the Inter-system Mobility Failure for Voice Fallback of the Inter-system HO Report was also discussed in RAN3#121, and left for further study. It should be noted that all the necessary information for the source node to change the voice fallback handover settings is already available from the handover report type (which will be set to “Inter-system Mobility Failure for Voice Fallback”), Failure Cell ID and Re-establishment Cell ID. The UE RLF Report Container could provide additional information such as neighbour cell measurements, but given that the RLF occurred shortly after handover from NR the measurement time was short and number of measurements is probably few, so the gains from this is limited. Since the necessary information is already available also without the UE RLF Report Container, we think that the IE could be provided optionally, which is also the case in other failure types.

- Observation 1.4: The necessary information to improve the voice fallback handover settings is already available from the handover report type, Failure Cell ID and Re-establishment Cell ID in the Inter-system Mobility Failure for Voice Fallback of the Inter-system HO Report.

- Proposal 1.3: The UE RLF Report Container should be optionally included in the Inter-system Mobility Failure for Voice Fallback of the Inter-system HO Report.

In Annex A, a TP implementing Proposal 1 to Proposal 3 is included.

- Proposal 1.4: Agree on the TP for 38.413 included in Annex A.

## 2.2 Fast MCG recovery

During RAN3#119, it was agreed to add “SCG was deactivated or other cases that SCG is not available” as a possible fast MCG recovery failure cause. This agreement was sent to RAN2 where it was discussed. Then during previous RAN2 meeting, the following agreements were taken:

Agreements:

1	UE reports the elapsed T316 between the transmission of MCGFailureInformation and receiving RRC reconfiguration or RRC release message.

2	No T316 related triggering threshold is introduced.

3	Reuse existing RLF report to capture fast MCG recovery related information.

The information that SCG was deactivated at the time of failure would also be useful in the SN, for example to not deactivate SCG in some cases, so that MCG Failure can be attempted in case of RLF in MCG.

Observation 2.1: In some cases, it is beneficial for the SN to understand that Fast MCG Recovery failure was due to SCG status.

One possible way to achieve this goal would be to forward the RLF report to the SN. But most of the information contained in the RLF Report will not be useful for the SN. Therefore, and to avoid signalling useless information, one other possibility would be to send only the SCG Status (i.e. SCG was deactivated or other cases that SCG is not available) to the SN in case SCG status was part of the problem leading to the failure.

Therefore it is proposed that RAN3 agrees one of these 2 solutions:

- RLF Report is forwarded to SN in case of fast MCG recovery failure due to SCG being deactivated or not available.

- SCG status is sent to SN in case of fast MCG recovery failure due to SCG being deactivated or not available.

Proposal 2.1: SCG status is sent to SN (e.g. via RLF Report) in case of fast MCG recovery failure due to SCG being deactivated or not available.

## 2.3 MRO for CPAC

During RAN3#121bis, it was agreed that option 1 will be used, as captured in stage-2:

The MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.

For CPA or MN initiated CPC, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the target SN, MN sends the SCGFailureInformation to the target SN.

For SN initiated CPC, the MN sends the SCGFailureInformation to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCGFailureInformation to the target SN.

But the following aspect still need some conclusion:

For SN-initiated PSCell Change/CPC, FFS if “suitable cell” needs to be signaled to target SN

For SN-initiated, in case the source SN detects that it had proposed a “suitable cell” which was not selected by the target SN, it will send the SCG FAILURE TRANSFER to the MN. In return, as described in stage-2 text above, the MN will forward the SCGFailureInformation to the target SN, in the SCG FAILURE INFORMATION REPORT message. However, neighbour cells measurements are contained in the SCGFailureInformation (i.e. MeasResultSCG-Failure). Therefore, the target SN will be able to determine that a suitable cell was proposed during CPAC preparation, but not selected. Also, there might be other parameters in admission control such as load issue, or UE capability issue in terms of band combinations in DC, etc… which lead to the “non-selection” of this cell as PSCell candidate. Therefore, the MN or the source SN are not the suitable node to perform this analysis. In conclusion, there is no need to add “suitable cell” in existing XnAP messages.

Proposal 3.1: No need to add “suitable cell” in existing XnAP messages.

# 3	Conclusion

- On MRO for IRAT Handover due to Voice Fallback

- Observation 1.1: In the existing intra-RAT HO report, the term “Re-establishment cell” is used for the cell the UE comes back to after RLF, and the semantics clarify that this is the cell where UE attempted re-establishment or where the UE successfully re-connected after the failure.

- Observation 1.2: In the existing inter-system HO report, the term “ID” is typically used in the IE name instead of “CGI” or “ECGI”.

- Proposal 1.1: Change the IE name “Reconnect Cell ID” (currently stated as FFS) in the Inter-system Mobility Failure for Voice Fallback of the Inter-system HO Report to “Re-establishment Cell ID”, and include in the Semantics description that this is the cell where UE attempted re-establishment or where the UE successfully re-connected after the failure.

- Observation 1.3: The re-establishment cell after a RLF occurring shortly after a successful handover from NR to LTE triggered due to voice fallback is only of interest if it is an E-UTRAN cell.

- Proposal 1.2: The “Re-establishment Cell ID” IE type in the Inter-system Mobility Failure for Voice Fallback of the Inter-system HO Report should be set to E-UTRA CGI.

- Observation 1.4: The necessary information to improve the voice fallback handover settings is already available from the handover report type, Failure Cell ID and Re-establishment Cell ID in the Inter-system Mobility Failure for Voice Fallback of the Inter-system HO Report.

- Proposal 1.3: The UE RLF Report Container should be optionally included in the Inter-system Mobility Failure for Voice Fallback of the Inter-system HO Report.

- Proposal 1.4: Agree on the TP for 38.413 included in Annex A.

On Fast MCG Recovery:

Observation 2.1: In some cases, it is beneficial for the SN to understand that Fast MCG Recovery failure was due to SCG status.

Proposal 2.1: SCG status is sent to SN (e.g. via RLF Report) in case of fast MCG recovery failure due to SCG being deactivated or not available.

On MRO for CPAC:

Proposal 3.1: No need to add “suitable cell” in existing XnAP messages.

# References

[1] 3GPP TS 36.423 V17.5.0 (2023-06), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol (X2AP) (Release 17)

[2] 3GPP TS 38.413 V17.5.0 (2023-06), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NG-RAN; NG Application Protocol (NGAP) (Release 17)

# Annex A: TP to SON BL CR to TS 38.413

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |  |  |  |
| >Too early Inter-system HO |  |  |  |  | - |  |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |  |  |  |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |  |  |  |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |  |  |  |
| >Inter-system Unnecessary HO |  |  |  |  | - |  |  |
| >>Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |  |  |  |
| >>Target Cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |  |  |  |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |  |  |  |
| >>Candidate Cell List |  | 1 |  |  |  |  |  |
| >>>Candidate Cell Item |  | 1..<maxnoofCandidateCells> |  |  |  |  |  |
| >>>>CHOICE Candidate Cell Type | M |  |  |  |  |  |  |
| >>>>>Candidate CGI |  |  |  |  |  |  |  |
| >>>>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |  |  |  |
| >>>>>Candidate PCI |  |  |  |  |  |  |  |
| >>>>>>Candidate PCI | M |  | INTEGER (0..1007, …) | This IE includes the NR Physical Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |  |  |  |
| >>>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNARFCNA) | RF Reference Frequency as defined in TS 38.104 [39], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |  |  |  |
| > Inter-system Mobility Failure for Voice Fallback |  |  |  |  |  | YES | ignore |
| >>Source Cell ID |  | M |  | NG-RAN CGI 9.3.1.73 | CGI of the source cell for HO |  |  |
| >>Failure Cell ID |  | M |  | E-UTRA CGI 9.3.1.9 | CGI of the failure cell for HO |  |  |
| >>Re-establishment Cell ID |  | O |  | E-UTRA CGI 9.3.1.9 | E-UTRA CGI of the cell where UE attempted re-establishment or where the UE successfully re-connected after the failure. |  |  |
| >>UE RLF Report Container |  | O |  | 9.3.3.41 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. Value is 32 |
| maxNARFCN | Maximum value of NR carrier frequency, defined in TS 38.331 [18] |

<<<<<<<<<<<<<<<<<<<< Next  Change >>>>>>>>>>>>>>>>>>>>

IntersystemMobilityFailureforVoiceFallback ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

reestablishmentCellID	EUTRA-CGI                      OPTIONAL,

uERLFReportContainer	UERLFReportContainer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { IntersystemMobilityFailureforVoiceFallback-ExtIEs} }			OPTIONAL,

...

}

IntersystemMobilityFailureforVoiceFallback-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>