3GPP TSG-RAN WG3 Meeting #122	R3-237605

# 13th – 17th Nov 2023, Chicago,USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR |  | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of user consent for MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA3 provides updates on solution for user consent in LS S3-234267 - R3-234493: A1: The existing user consent mechanism is only intended for internal use within the 3GPP operators (controllers) domain for collection MDT measurements at the RAN and reporting them to the Trace Collection Entity. A2: Further, user consent is given to the operator so that the 3GPP system can be provisioned/configured based on the operator-subscriber agreed permissions stored in UDM to make it feasible for the 3GPP system to comply with local laws and regulations. Whether the RAN needs to check if user consent is required for a specific type of information/data of a subscriber for a particular purpose can be configured by the OAM. Such configuration is done based on local regulations, which is likely to change infrequently. From the UDM, per UE basis, the RAN receives the yes/no information on whether a user has given consent for the information/data configured by the OAM to be used by the RAN for a particular purpose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated description of MDT PLMN List , Report retrieval of logged MDT and MDT initiation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | User consent mechanism not aligned with SA3 agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.1.1.3.2, 5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------Start change--------------------------------------------------------

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] apply.

Immediate MDT: MDT functionality involving measurements performed by the UE in CONNECTED state and reporting of the measurements to RAN available at the time of reporting condition as well as measurements by the network for MDT purposes.

Logged MDT: MDT functionality involving measurement logging by UE in IDLE mode, INACTIVE state, CELL_PCH, URA_PCH states and CELL_FACH state when second DRX cycle is used (when UE is in UTRA) for reporting to eNB/RNC/gNB at a later point in time, and logging of MBSFN measurements by E-UTRA UE in IDLE and CONNECTED modes.

Management Based MDT PLMN List: MDT PLMN List applicable to management based MDT.

MDT measurements: Measurements determined for MDT.

MDT PLMN List: A list of PLMNs where MDT measurements are allowed for a user. It is a subset of the EPLMN list and RPLMN at the time when MDT is initiated.

The determine of specific MDT measurements initiation and MDT reports route to TCE is configured to the RAN by the OAM.

NOTE:	The determine specific measurements in RAN is not apply to UTRA and E-UTRA

Signalling Based MDT PLMN List: MDT PLMN List applicable to signalling based MDT.

-------------------------------------------------Next of change--------------------------------------------------------

5.1.1.3.2	Report retrieval

For Logged MDT the measurement reporting is triggered by an on-demand mechanism, i.e. the UE is asked by the network to send the collected measurement logs via RRC signalling. UE Information procedure defined in TS 25.331 [4] and TS 36.331 [5] and TS 38.331 [15] is used to request UE to send the collected measurement logs. The reporting may occur in different cells than which the logged measurement configuration is signalled.

Transport of Logged MDT reports in multiple RRC messages is supported. With every request, the network may receive a part of the total UE log. To indicate the reported data is a segment, the UE shall include data availability indicator in UEInformatonResponse message to convey the information that further measurement information is available, as specified in 5.1.1.3.1. In multiple RRC transmissions for segmented Logged MDT reporting, FIFO order is followed, i.e. the UE should provide oldest available measurement entries in earliest message. There is no requirement specified on the size of particular reporting parts. However, each reported part should be "self-decodable", i.e. interpretable even in case all the other parts are not available.

The UE shall send an empty report when retrieval is attempted and the RPLMN is not in the MDT PLMN list.

The RAN node determines the logged MDT measurement reports route to TCE are subject to user consent by means of OAM configuration.

-------------------------------------------------Next of change--------------------------------------------------------

5.1.3	MDT Initiation

There are two cases that RAN should initiate a MDT measurements collection task. One is that the MDT task is initiated without targeting a specific UE by the cell traffic trace, i.e. management based trace function from OAM. The other is that the MDT task is initiated towards a specific UE by the signalling trace activation messages from CN nodes, i.e. the Initial Context Setup message, the Trace Start message or the Handover request message in E-UTRAN or NR, the CN Invoke Trace message in UTRAN. The detailed procedures to transfer the MDT configurations to RAN are specified in TS 32.422 [6].

For signalling based MDT, the CN shall initiate MDT towards a particular user if at least one of the requested MDT measurements is subject to user consent .

For management based MDT, the CN indicates to the RAN whether MDT is allowed to be configured by the RAN for this user considering e.g. user consent and roaming status (see TS 32.422 [6]), by providing management based MDT allowed information. For E-UTRAN/UTRAN, the MDT allowed information consists of the Management Based MDT Allowed indication and optionally the Management Based MDT PLMN List. For NR, the MDT allowed information only consists of the Management Based MDT PLMN List. The management based MDT allowed information propagates during inter-PLMN handover or inter-PLMN UE context retrieval if the Management Based MDT PLMN List is available and includes the target PLMN.

For signalling based and management based MDT, the RAN determines the MDT measurements that are subject to user consent by means of OAM configuration information reflecting local regulations.

A UE is configured with an MDT PLMN List only if user consent is valid for the RPLMN.

-------------------------------------------------End of change--------------------------------------------------------