3GPP TSG-RAN WG3 Meeting #122	R3-237668

Chicago , US, 13–17 Nov, 2023

Agenda Item:	10.2.2

Source:	CMCC

Title:	Discussion on MRO for CPAC

Document for:	Discussion and Decision

# 1 Introduction

In this contribution, we would further discuss on the open issues in overall procedures of MRO for CPAC and provide some proposals on these issues.

# 2 Discussion

In the summary in R3-235728[2], RAN3 has agreed Option1 in the following overall procedures as way forward for CPA/MN initiated CPC and SN initiated CPC.

For CPA or MN initiated CPC,  there are two options:

Option 1:

  - if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN via SCG Failure Information Report message.

The following information are included in the message from the MN to the (candidate) target SN:

    - Suitable PSCell ID

    - FFS: Indication whether a candidate PSCell is accepted by the (candidate) target (The candidate PSCell list recommended by the initiating node is already agreed).

  - Otherwise, the MN makes the optimization

Option 2:

  - MN always send the SCG Failure Information Report message to the last serving SN.

  - The last serving SN feedback if the problem is not brought by itself.

  - the MN makes the optimization

For SN initiated CPC, there are two options:

Option 1:

  - The MN sends SCG failure information report message to source SN, and source SN performs root cause analysis

  - if the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the source SN sends message to the MN, then the MN sends message to the (candidate) target SN.

  - Otherwise, the source SN makes the optimization

    - SCG FAILURE TRANSFER is used from the source SN to the MN

    - SCG Failure Information Report message is used from the MN to the (candidate) target SN

- Option 2: the same as the option 2 for MN initiated CPC.

In light of the following agreement achieved in RAN3#121, we can know that the MN performs the initial analysis to identify the procedure is CPA or CPC and the initiating node (e.g. MN or source SN) .

The MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.

After that, for CPA or MN initiated CPC, the MN which initiates the procedure performs root cause analysis. The MN is responsible to find the suitable PSCell from the measurement report included in SCG failure information reported from UE. Based on the suitable PSCell, the MN can identify the problem node since if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, we can see the wrong candidate PSCell selection at the (candidate) target SN; and if the suitable PSCell is not one of the candidate PSCells provided by the MN, we can see the wrong candidate PSCell selection at MN.

Proposal 1: For CPA or MN initiated CPC, the MN performs root cause analysis, it is responsible to find the suitable PSCell and identify the problem node (MN or the (candidate) target SN).

For SN initiated CPC, the procedure is initiated by source SN and the possible problem node is the source SN or the (candidate) target SN. So the source SN should to perform root cause analysis, it is responsible to find the suitable PSCell from the measurement report included in SCG failure information reported from UE and identify the problem node (the source SN or the (candidate) target SN).

Proposal 2: For SN initiated CPC, the source SN performs root cause analysis, it is responsible to find the suitable PSCell and identify the problem node (the source SN or the (candidate) target SN).

Based on the overall procedures of MRO for CPAC, we can see that the (candidate) target SN receive the SCG FAILURE INFORMATION REPORT message from MN only when the wrong candidate PSCell selection at the (candidate) target SN. So it can be used as an implicit indication, that is to say, the candidate PSCell list recommended by the initiating node(the MN or the source SN) is not accepted by the (candidate) target SN. Therefore, there is no need to include an indication whether a candidate PSCell is accepted by the (candidate) target SN and the candidate PSCell list selected by the (candidate) target SN in the SCG FAILURE INFORMATION REPORT message from MN to the (candidate) target SN.

Observation 1: The (candidate) target SN receive the SCG FAILURE INFORMATION REPORT message from MN only when the wrong candidate PSCell selection at the (candidate) target SN. So it can be used as an implicit indication, that is to say, the candidate PSCell list recommended by the initiating node(the MN or the source SN) is not accepted by the (candidate) target SN.

Proposal 3: The following information is included in the SCG FAILURE INFORMATION REPORT message from the MN to the (candidate) target SN:

- Suitable PSCell ID is needed.

- Indication whether a candidate PSCell is accepted by the (candidate) target SN is not needed.

- The candidate PSCell list selected by the (candidate) target SN is not needed.

# 3 Conclusion

In this paper, we discuss on overall procedure of MRO for CPAC, and we have the following observation and proposals:

Proposal 1: For CPA or MN initiated CPC, the MN performs root cause analysis, it is responsible to find the suitable PSCell and identify the problem node (MN or the (candidate) target SN).

Proposal 2: For SN initiated CPC, the source SN performs root cause analysis, it is responsible to find the suitable PSCell and identify the problem node (the source SN or the (candidate) target SN).

Observation 1: The (candidate) target SN receive the SCG FAILURE INFORMATION REPORT message from MN only when the wrong candidate PSCell selection at the (candidate) target SN. So it can be used as an implicit indication, that is to say, the candidate PSCell list recommended by the initiating node(the MN or the source SN) is not accepted by the (candidate) target SN.

Proposal 3: The following information is included in the SCG FAILURE INFORMATION REPORT message from the MN to the (candidate) target SN:

- Suitable PSCell ID is needed.

- Indication whether a candidate PSCell is accepted by the (candidate) target SN is not needed.

- The candidate PSCell list selected by the (candidate) target SN is not needed.

# Reference

- RAN3_121bis_agenda_20231013_EOM1

- R3-235728, Summary of Offline Discussion on MRO