3GPP TSG-RAN3 Meeting #122	R3-237754

# Chicago, USA, 13th November – 17th November 2023

Agenda Item:	21.2

Source:	Ericsson (Rapporteur)

Title:		Summary of proposals for eRedCap online session

Document for:	Other

# List of proposals for online discussion

Topic #1 Provision of Data size over NGAP for MT-SDT Paging

Proposal 1 (Rapporteur): Wait for SA2 and CT4 reply LSes on feasibility and signalling load concerns before discussing any RAN3 NGAP enhancement. If both replies not received at this meeting, the solution can be discussed in R18 maintenance phase/TEI. R18 eRedCap WI completion is not impacted.

Proposal 2 : In the RAN Paging Request message, remove the FFS

Topic #2 Stage 3 fixes

Proposal 3: For F1AP, to support of eRedCap indication during F1 paging:

- Option a: reuse the R17 RedCap Indication IE in the UE Paging Capability IE in the PAGING message and add dedicated procedure texts for eRedCap UE.  (Huawei)

- Option b: define a new eRedCap Indication in the UE Paging Capability IE (Ericsson, CATT, ZTE)

Proposal 4: Add procedural text in NGAP on how NG-RAN node should use the CN MT Communication Handling IE when it is included in the Core Network Assistance Information for RRC INACTIVE IE during the INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST and PATH SWITCH REQUEST ACKNOWLEDGE messages. (Ericsson, CATT, ZTE, QC)

Proposal 5: Update TS 38.300 BL CR as follows: Upon receiving the RAN Paging Request message from the AMF while the UE is in RRC_INACTIVE with eDRX beyond 10.24 seconds, the last serving gNB may page in its cells comprised in the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s), in order for the UE to resume connection. (ZTE)

Proposal 6: miscellaneous (Nokia) proposals:

- move the description of the NR Paging Long eDRX Information for RRC INACTIVE IE together with the description of the HLCom Activate and remove the “if”.

- add a reference to TS 23.501 in the unsuccessful operation of the MT communication handling procedure.

- for MO-SDT the gNB should be able to disable the CN based communication handling. This is gNB implementation triggered. No additional standards impact.

Proposal 7: Add eRedCap indication in TS 38.470, 38.401? (Huawei)