3GPP TSG-RAN WG3 Meeting #122	R3-237795

Chicago, US, 13-17 Nov, 2023

Agenda Item:	24.2

Source:	Huawei

Title:	Proposed summary for Network Energy saving online session

Document for:	Discussion and approval

1	Introduction

This contribution provides the proposals to be discussed at the network energy saving online session after offline discussion.

2	List of proposals to be discussed at online session

Paging enhancements

Proposal 1: Confirm the support of the paging enhancement for RRC inactive UE. Stage 2 TP considers to add texts on paging fallback case.

Proposal 2: No support the paging enhancement for RRC idle UE at this release.

Proposal 3: No reply LS is needed to SA2.

Proposal 4: No support for the XnAP RAN paging support for RRC inactive UE at this release

Proposal 5: For TP for TS38.470, highlight the paging enhancement for RRC inactive UE, and delete the editor’s note

Cell DTX/DRX

Proposal:  No exchange of cell DTX/DRX configuration over Xn at this release (i.e. do nothing)

Inter-node beam activation

Proposal 1: For network energy saving cause for SSB deactivation

- Over XnAP, the network saving cause is included in the Coverage Modification Cause IE

- Over F1AP, the network saving cause is included in the Coverage Notification Indication IE in the gNB-DU Configuration update response message, and the CCO Assistance Information IE in the gNB-CU configuration update request message

- The Beam level cause value is signalled from the DU to the CU, and the CU to DU, and Xn.

Proposal 2: Change the presence of SSBs Activated List in the Tabular to be “0..1” in the CELL ACTIVATION RESPONSE message for XnAP.

Proposal 3: for stage 2 TP

- For TS 38.300, move the inter-node beam activation section into intra-system energy saving section

- To have TP for TS 38.420.

[Online discussion]: Two options for the SSB deactivation for the CU/DU case

- Option 1: The gNB-CU controls deactivating SSBs due to Network Energy Optimization and sends request to the DU, then the DU makes final decision.

- Option 2: The DU itself determines deactivating SSBs without assistance information from the CU.

NES CHO

[online discussion if time allows]

Proposal 1: In TS 38.401, update the CHO procedure to specify that a L1 signalling is used to trigger the evaluation of execution condition of NES CHO

Proposal 2: In TS 38.473, Source gNB-CU can indicate to source gNB-DU that the UE is configured with NES-specific CHO configuration(s) via a NES CHO indicator in F1AP

3 Discussion

3.1 Paging enhancements

The RAN2 reply LS RAN2 was copied below.

RAN2 would like to thank RAN3 for their LS on the enhancements to restricting paging in a limited area. RAN2 discussed Q1 and would like to provide the following answer. Q1: Whether RAN3’s agreements have any impact on, or could be impacted by, TS 38. 304 clause 7 Paging, e.g., related to the following paragraph. TS 38.304 clause 7.1: “In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.” RAN2's Answer: No RAN2 impact is identified but it was understood that paging may fail as UE assumes same paging message to be sent on all beams. Regardless it is RAN2 understanding that UE behavior does not need to be changed, as the potential paging failure could be handled by NW implementation e.g. by paging on all the beams in subsequent paging.

Issue 1: Paging enhancement for RRC inactive UE

Moderator observation:

- Confirm the support – Majority companies

- No support - Nokia

Add stage2 texts: if the paging over the limited set of beams is failed, the gNB fall backs to the normal RAN paging under section 9.2.5.

Rapporteur summary:  Confirm the support of the paging enhancement for RRC inactive UE.  Stage 2 TP considers to add texts on paging fallback case.

Issue 2: Paging enhancement for RRC idle UE

Moderator observation:

- No support: Nok, ZTE, HW

- Support: Qualcomm, Samsung, Ericsson

Rapporteur summary:  No support the paging enhancement for RRC idle UE at this release.

Issue 3: Reply LS to SA2

The baseline texts decided at the previous meeting are copied as follows.

Assumption validation: SA2 would also like to validate the following assumption: If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how paging escalation happen will be controlled by the AMF based on existing procedures. Answer: RAN3 confirms and expects the AMF can do the paging escalation based on the existing mechanisms. Q1: SA2 understands that the beam-based paging is most efficient towards low mobility and static UEs. How is the gNB expected to determine whether the UE is static or not across multiple RRC connections from the same UE considering TS 33.501 contains requirements on 5G-S-TMSI reallocation? Answer: NG-RAN can determine the UE mobility state at least based on the subscription information i.e., the "Expected UE behaviour" IE from the AMF. Q2: SA2 would like to ask about the validity condition of the potential “List of recommended beams” container that is stored in the AMF. What is the AMF condition to delete the stored "List of recommended beams"? Answer: RAN3 considers that the AMF itself can decide the validity condition of the potential “List of recommended beams”, e.g., the same as the handling of the existing Recommended Cells for Paging, Last Visited Cell Information in TS 38.413. It is up to the AMF implementation to determine to delete it when the UE moves to the connected state, or under other conditions.

Rapporteur summary: No reply LS is needed to SA2.

Issue 4: XnAP RAN paging enhancement for RRC inactive UE

Moderator observation:

Discussed at previous meeting but no conclusion.

- No support: ZTE, Ericsson

Rapporteur summary: No support for the XnAP RAN paging support for RRC inactive UE at this release

Issue 5: State 2 TP

For TP for TS38.470

- Highlight the paging enhancement for RRC inactive UE, and delete the editor’s note - Huawei

Rapporteur summary: For TP for TS38.470, highlight the paging enhancement for RRC inactive UE, and delete the editor’s note .

3.2 Cell DTX/DRX

Issue 1: exchange of the cell DTX/DRX configuration over Xn.

Previous meeting had the following WA:

- WA: Support the exchange of the Cell DTX/DRX configuration over Xn.

Moderator observation:

All companies seem to agree to exchange the Cell DTX/DRX configuration.

About the activation/deactivation status, the following options are proposed.

- Option 1: current activation/deactivation status (referring to the RRC IE) - HW, ZTE, Sam, CATT

- Option 2: probability parameter/ onDuration usage probability” and “offDuration usage probability – Nok, Ericsson

- Option 3: initial activation status as decided by RRC (referring to the RRC IE) - Qualcomm

In which XnAP messages:

- Option 1: Updating the existing Served Cell Information IE – Majority companies

- Option 2: New “Resource Status exchange procedure” including “Cell DTRX information” - Ericsson

Rapporteur summary: No exchange of cell DTX/DRX configuration over Xn at this release (i.e. do nothing)

Issue 2: In CU/DU split architecture the CU or DU make final decision of the Cell DTX/DRX parameters configuration?

Moderator observation:

- The DU is responsible for determining the configuration of cell DRX/DTX, and the activation/deactivation status: CATT, Huawei

- To discuss: ZTE

Rapporteur summary: No conclusion on the DU is responsible for determining the configuration of cell DRX/DTX, and the activation/deactivation status. This is not excluded to include the assistance information from the CU at the next release.

3.3 Inter-node beam activation

Issue 1: Beams deactivation for CU/DU split case

Moderator observation:

In R3-237398 - Ericsson

- RAN3 discusses and agrees that the gNB-CU controls whether the gNB-DU is allowed deactivating SSBs due to Network Energy Optimization

- RAN3 discusses and agrees to enable the gNB-CU informing the gNB-DU that Cells and SSBs of cells not served by the gNB-DU have been deactivated due to Network Energy Optimization.

- RAN3 discusses and agrees to indicate over Xn/F1 AP the time gNB-DU desires to keep the SSB deactivated

In R3-237452 - Nokia

- RAN3 first needs to decide whether beam switch-off decision for Rel-18 energy saving is taken by the:

  - Option 1: The DU - following legacy principles for CCO;

  - Option 2: The CU - following legacy principles for cell switch-off.

- Beam switch-off decision to be taken by the gNB-DU due to higher energy saving potential.

Question: Two options for the CU/DU case

- Option 1: The gNB-CU controls deactivating SSBs due to Network Energy Optimization and sends request to the DU, then the DU makes final decision.

- Option 2: The DU itself determines deactivating SSBs without assistance information from the CU.

Rapporteur summary: The DU determines deactivating SSBs only when the assistance information is received from the CU??

- The network saving cause is included in the XnAP Coverage Modification Cause IE

- The network saving cause is included in the F1AP Coverage Notification Indication IE in the gNB-DU Configuration update response message, and the F1AP CCO assistance info IE in the gNB-CU configuration update request message.

Issue 2: Encoding ‘energy saving’ cause over Xn

- R3-237227-Huawei, R3-237452-Nokia

| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

If the Coverage Modification Cause IE set to “coverage” or “cell edge capacity” is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue detected at NG-RAN node1 and for configuring coverage state of its served cell(s). If the Coverage Modification Cause IE set to “network energy saving” is present and the SSB Coverage State IE is zero in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to decide the beam activation for the concerned beams when necessary.

Rapporteur summary: TBD.

Issue 3: Encoding details of the cause value over F1

- Option 1: R3-237226 – Cell level - Huawei

- Option 2: R3-237452 - beam level - Nok

Rapporteur summary: The Beam level cause value is signalled from the DU to the CU, and the CU to DU, and Xn.

Issue 4: Stage3 details.

In R3-237226 - Huawei,

- Change the presence of SSBs Activated List to be “0..1” in the CELL ACTIVATION RESPONSE message for XnAP

9.1.3.8 CELL ACTIVATION RESPONSE This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that one or more cell(s) previously switched-off has (have) been activated. Direction: NG-RAN node2 NG-RAN node1. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. CHOICE Activated Served Cells; M; YES; reject. <<<<<<<<<<<<<<<<<<<< Unmodified Texts Omitted >>>>>>>>>>>>>>>>>>>>. >NR Cells and SSBs; YES; ignore. >>NR Cells and SSBs List; 1; –. >>>NR Cells and SSBs Item; 1 .. < maxnoofCellsinNG-RANnode>; –. >>>>NR CGI; M; 9.2.2.7; –. >>>>SSBs Activated List; 1; –. >>>>>SSB Activated Item; 1 .. < maxnoofSSBAreas >; –. >>>>>>SSB Index; M; INTEGER (0..63); Identifier of the activated SSB beam.; –. Activation ID; M; INTEGER (0..255); Allocated by the NG-RAN node1; YES; reject. Criticality Diagnostics; O; 9.2.3.3; YES; ignore. Interface Instance Indication; O; 9.2.2.39; YES; reject.

Rapporteur summary: 	Change the presence of SSBs Activated List in the Tabular to be “0..1” in the CELL ACTIVATION RESPONSE message for XnAP.

Issue 5: Stage2 TP

For State 2 TP

- For TS 38.300, move the inter-node beam activation section into intra-system energy saving section: Huawei

- To have TP for TS 38.420:  CATT

Rapporteur summary: TBD.

3.4 NES CHO

- In TS 38.401, update the CHO procedure to specify that a L1 signalling is used to trigger the evaluation of execution condition of NES CHO – Huawei

- In TS 38.473, Source gNB-CU can indicate to source gNB-DU that the UE is configured with NES-specific CHO configuration(s) via a NES CHO indicator in F1AP - Qualcomm

Rapporteur summary: no conclusion.

3.5 TPs/LS

| Stage 3 TPs | XnAP: F1AP: |
| --- | --- |
| Stage 2 TPs | TS 38.300 TP: TS 38.470 TP: TS 38.420 TP |
| Reply LS to SA2 |  |

3.6 completion of NES WI

Rapporteur summary: The R18 NES is completed in RAN3.

4	References

| R3-237145 | Reply LS on paging (RAN2(Huawei)) | LS in |
| --- | --- | --- |
| R3-237226 | (TP to BLCR for TS 38.473, 38.401, 38.470 and 38.300) Finalizing network energy saving techniques (Huawei, Deutsche Telekom) | Other Resp in R3-237733 |
| R3-237227 | (TP to BLCR for TS 38.423) Finalizing network energy saving techniques (Huawei) | other |
| R3-237327 | Discussion on network energy saving (Samsung) | discussion |
| R3-237328 | Introduction of Network Energy Saving for Paging IDLE UE (Samsung) | CR1037r, TS 38.413 v17.6.0, Rel-18, Cat. B |
| R3-237398 | (TP to BL CR for TS 38.423) Cell DTRX in Network Energy Saving (Ericsson) | other |
| R3-237451 | (TP to TS 38.423) Discussion on Paging in Subset of Beams and Inter-node Cell DTX/DRX Configuration (Nokia, Nokia Shanghai Bell) | other |
| R3-237452 | (TP to TS 38.473) Beam deactivation decision and signalling for energy saving (Nokia, Nokia Shanghai Bell) | other |
| R3-237460 | Open issues on NES techniques (Qualcomm Incorporated) | discussion |
| R3-237516 | Introduction of Network Energy Saving (Ericsson) | Other ok |
| R3-237660 | Discussion on network energy saving (ZTE) | discussion |
| R3-237661 | TP to BL CR of XnAP for Cell DTX-DRX (ZTE) | other |
| R3-237662 | TP to BLCR of 38.300 for network energy saving (ZTE, Samsung, Ericsson, Qualcomm Incorporated) | other |
| R3-237618 | (TP to NES BL CR for TS38.423) Discussion on inter-node beam activation and cell DTXDRX for NES (CATT) | Other Late contribution |
| R3-237619 | TP to NES BLCR for TS 38.420 (CATT) | Other Late contribution |