3GPP TSG-RAN3 Meeting #122	R3-237850

# Chicago, USA, November 13 – 17, 2023

Agenda item:	25.2.1

Source:	Xiaomi, Ericsson, Nokia, Nokia Shanghai Bell

Title:		(TPs for 38.473 BL) Support of UL Traffic Information for XR

Document for:	Other

1	Introduction

According to the following running CR R2-2312603

UL-TrafficInfo-r18 ::=	SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionUL-TrafficInfo-r18

PDU-SessionUL-TrafficInfo-r18 ::= SEQUENCE {

pdu-SessionID-r18                   PDU-SessionID,

qos-FlowUL-TrafficInfoList-r18      SEQUENCE (SIZE (1..maxNrofQFIs)) OF QOS-FlowUL-TrafficInfo-r18

}

QOS-FlowUL-TrafficInfo-r18 ::=       SEQUENCE {

qfi-r18                             INTEGER (0..maxQFI),

jitterRange-r18                     ENUMERATED {ffs}        OPTIONAL,

burstArrivalTime-r18                CHOICE {

referenceTime                      ReferenceTime-r16,

referenceSFN-AndSlot               ReferenceSFN-AndSlot-r18

}                                                           OPTIONAL,

trafficPeriodicity-r18				ENUMERATED {ffs}        OPTIONAL,

pduSetIdentification-r18            BOOLEAN                 OPTIONAL,

...

}

the following information is provided from UE via UAI.

-     trafficPeriodicity

-	burstArrivalTime

-	jitterRange,

The trafficPeriodicity, burstArrivalTime and jitterRange are traffic information and used for DRX setting for power saving purpose according to TS 23.501.

In CU-DU split architecture, the UEAssistanceInformation IE in CU to DU RRC Information can be re-used to transfer the UL traffic information from UE from gNB-CU to gNB-DU, but the existing procedure text is not proper for C-DRX configuration, this TP provides proper procedure descriptions for XR service when the UL traffic information is transferred to gNB-CU via UAI, and the gNB-CU sends the information to gNB-DU.

TP for TS 38.473 BL CR

<<<<<<<<<<<<<<<<<<<< Change Start>>>>>>>>>>>>>>>>>>>>

8.3	UE Context Management procedures

8.3.1	UE Context Setup

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the case of RACH based SDT procedure, the CellGroupConfig IE shall be ignored by the gNB-CU.

<<<<<<<<<<<<<<<<<<<< unchanged parts omitted >>>>>>>>>>>>>>>>>>>>

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE. If the UEAssistanceInformation IE containing the uplink traffic information is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider it to configure UE power saving management scheme for connected mode DRX.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3.4	UE Context Modification (gNB-CU initiated)

8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

<<<<<<<<<<<<<<<<<<<< unchanged parts omitted >>>>>>>>>>>>>>>>>>>>

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE. If the UEAssistanceInformation IE containing the uplink traffic information is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider it to configure UE power saving management scheme for connected mode DRX.

<<<<<<<<<<<<<<<<<<<< Change End>>>>>>>>>>>>>>>>>>>>