3GPP RAN WG3 Meeting #122	R3-237853

# Chicago, USA, 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0093 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on multiple F1-U tunnels for MRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, Nokia Shanghai Bell, ZTE, Qualcomm, CMCC, CBN, Samsung, Lenovo, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For split gNB scenario, it is possible that MBS session contexts for the same MBS session are established in multiple gNB-DUs which means there maybe multiple F1-U tunnels for one MRB.However, management of mutiple F1-U tunnels for one MRB is not supported for broadcast service. Current E1AP functionality for BC Bearer Context modification is limited as in order to enable setup of multiple F1-U bearer contexts, one per DU, every new instance of the gNB-CU-CP initiated BC Bearer Context modification procedure containing a per MRB BC Bearer Context F1-U TNL Info at DU IE procedure is interpreted as an addition of one (set of) new F1-U tunnel(s) (towards one gNB-DU). |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Adding a new IE “Additional F1-U TNL Info List” into “BC MRB To Modify List” into the “BC MRB To Modify Item” structure for providing TNL info at DUs. 2. Introducing a Broadcast F1-U Context Reference in order to be able to correlate F1-U resources to the respective resources at the gNB-CU-UP. 3. For the case that one TNL info at CU is required for each F1-U tunnel, the CU-UP is still able to allocate respective resources, it could, however, allocate a single TEID per MRB as well, which is left for implementation and deployment choices. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the broadcast MBS session management function. This CR is backward compatible from a protocol and functional point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Management of a BC Bearer context, which is performed by means of E1AP BC Beaer Modification, would be limited, if if multiple F1-U tunnels for one broadcast MRB (towards multiple gNB-DUs) are established. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.1.1.2, 8.6.1.2.2, 9.3.1.114, 9.3.1.139a, 9.3.3.27, 9.3.3.28, 9.3.3.29, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1:Introduce Broadcast F1-U Context Reference to identify F1-U context for different DU |  |  |  |  |  |  |  |  |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

8.6.1.1	BC Bearer Context Setup

8.6.1.1.1	General

The purpose of the BC Bearer Context Setup procedure is to allow the gNB-CU-CP to establish MBS session resources for a broadcast MBS session in the gNB-CU-UP. The procedure uses MBS-associated signalling.

8.6.1.1.2	Successful Operation

Figure 8.6.1.1.2-1: BC Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BC BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested MBS session resources, it replies to the gNB-CU-CP with the BC BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BC BEARER CONTEXT SETUP RESPONSE message, the result of all the requested resources in the following way:

-	A list of BC MRBs which are successfully established shall be included in the BC MRB Setup Response List IE;

-	A list of BC MRBs which failed to be established shall be included in the BC MRB Failed List IE;

-	For each established BC MRB, a list of MBS QoS Flows which are successfully established shall be included in the MBS QoS Flow Setup List IE;

-	For each established BC MRB, a list of MBS QoS Flows which failed to be established shall be included in the MBS QoS Flow Failed List IE;

-	For each established BC MRB, if the Broadcast F1-U Context ReferenceE1 IE was included for an MRB within the BC MRB Setup Configuration IE, the gNB-CU-UP shall store it to correlate future modifications and include this reference for that MRB within the BC Bearer Context To Setup Response IE.The BC Bearer Context Setup procedure establishes per MRB only one F1-U resource, which, if included, is associated with a Broadcast F1-U Context ReferenceE1.

When the gNB-CU-UP reports the unsuccessful establishment of a BC MRB or MBS QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Requested Action for Available Shared NG-U Termination IE in the BC Bearer Context To Setup IE in the BC BEARER CONTEXT SETUP REQUEST message is set to

-	"apply available configuration" and an appropriate Shared NG-U Termination is available, the gNB-CU-UP shall apply the radio bearer configuration of the Shared NG-U Termination, and indicate in the BC BEARER CONTEXT SETUP RESPONSE message within the Available BC MRB Configuration IE in the BC Bearer Context To Setup Response IE the radio bearer configuration of the Shared NG-U Termination, if the radio bearer configuration of the Shared NG-U Termination is different than the one requested by the gNB-CU-CP.

-	"apply requested configuration" the gNB-CU-UP shall make use of an available appropriate Shared NG-U Termination if the radio bearer configuration of the Shared NG-U Termination, is the same as the one requested by the gNB-CU-CP, otherwise allocate separate resources as requested by the gNB-CU-CP and indicate in the BC BEARER CONTEXT SETUP RESPONSE message within the Available BC MRB Configuration IE in the BC Bearer Context To Setup Response IE the radio bearer configuration of the Shared NG-U Termination.

-	"apply available configuration if same as requested" the gNB-CU-UP shall make use of an available appropriate Shared NG-U Termination only if the radio bearer configuration of the Shared NG-U Termination is the same as the one requested by the gNB-CU-CP and reply with BC BEARER CONTEXT SETUP RESPONSE message.

8.6.1.1.3	Unsuccessful Operation

Figure 8.6.1.1.3-1: BC Bearer Context Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot establish the requested resources for the MBS session, it shall consider the procedure as failed and respond with the BC BEARER CONTEXT SETUP FAILURE message and an appropriate cause value.

If the Requested Action for Available Shared NG-U Termination IE in the BC Bearer Context To Setup IE in the BC BEARER CONTEXT SETUP REQUEST message is set to "apply available configuration if same as requested" and the requested configuration does not match the available shared NG-U termination, the gNB-CU UP shall reply with BC BEARER CONTEXT SETUP FAILURE message.

8.6.1.1.4	Abnormal Conditions

void.

8.6.1.2	BC Bearer Context Modification (gNB-CU-CP initiated)

8.6.1.2.1	General

The purpose of the gNB-CU-CP initiated BC Bearer Context Modification procedure is to allow the gNB-CU-CP to modify MBS session resources for a broadcast MBS session. The procedure uses MBS-associated signalling.

8.6.1.2.2	Successful Operation

Figure 8.6.1.2.2-1: BC Bearer Context Modification procedure, gNB-CU-CP initiated: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BC BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to perform at least partially the requested modifications it replies to the gNB-CU-CP with the BC BEARER CONTEXT MODIFICATION RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BC BEARER CONTEXT MODIFICATION RESPONSE message, the result of all the requested MBS session resources in the following way:

-	A list of BC MRBs which are successfully established or modified shall be included in the BC MRB Setup or Modify Response List IE;

-	A list of BC MRBs which failed to be established or modified shall be included in the BC MRB Failed List IE;

-	For each newly established or modified BC MRB, a list of MBS QoS Flows which are successfully established or modified shall be included in the MBS QoS Flow Setup List IE;

-	For each newly established or modified BC MRB, a list of MBS QoS Flows which failed to be established or modified shall be included in the MBS QoS Flow Failed List IE;

-	For each newly established BC MRB, as indicated in the BC MRB To Setup List IE, if the Broadcast F1-U Context ReferenceE1 IE was included for an MRB within the BC MRB Setup Configuration IE, the gNB-CU-UP shall store it to correlate future modifications and include this reference for that MRB within the BC Bearer Context To Setup Response IE and include this reference for that MRB within the BC Bearer Context To Setup Response IE. The gNB-CU-CP initiated establishment of new BC MRBs for a broadcast session establishes per MRB only one F1-U resource, which, if included, is associated with a Broadcast F1-U Context ReferenceE1.

-	If the BC Bearer Context F1-U TNL Info at DU IE is included for an MRB indicated in the BC Bearer Context To Modify IE, either within the BC MRB To Modify List IE or in the Additional F1-U TNL Info List IE, the gNB-CU-UP shall allocate respective F1-U resources and indicate in the BC Bearer Context To Modify Response IE the BC Bearer Context F1-U TNL Info at CU IE; if the Broadcast F1-U Context ReferenceE1 IE was included for a F1-U resource for an MRB, the gNB-CU-UP shall include the same reference in the BC Bearer Context To Modify Response IE.

When the gNB-CU-UP reports the unsuccessful establishment of a BC MRB or MBS QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the BC Bearer Context NG-U TNL Info at 5GC To Setup or Modify IE is contained in the BC BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the previously received BC Bearer Context NG-U TNL Info at 5GC.

8.6.1.2.3	Unsuccessful Operation

Figure 8.6.1.2.3-1: BC Bearer Context Modification procedure, gNB-CU-CP intiated: Unsuccessful Operation.

If the gNB-CU-UP cannot successfully perform any of the requested modifications, it shall respond with a BC BEARER CONTEXT MODIFICATION FAILURE message and an appropriate cause value.

8.6.1.2.4	Abnormal Conditions

void.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

9.3.1.114	BC MRB Setup Configuration

This IE contains MRB configuration information for a BC Bearer Context Context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BC MRB To Setup List |  | 1..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >MBS PDCP Configuration | M |  | PDCP Configuration 9.3.1.38 |  | - |  |
| >MBS QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. | - |  |
| >Broadcast F1-U Context ReferenceE1 |  |  | 9.3.1.139a |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

9.3.1.139a	Broadcast F1-U Context ReferenceE1

This IE contains a reference to a Broadcast F1-U Context used within an MBS-associated logical E1-connection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Broadcast F1-U Context ReferenceE1 | M |  | OCTET STRING (SIZE(4)) | This value is allocated to uniquely denote a Broadcast F1-U Context within an MBS-associated logical E1-connection. |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

9.3.3.27	BC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm BC Bearer Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.116 |  | - |  |
| BC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - |  |
| >BC Bearer Context F1-U TNL Info at CU | M |  | 9.3.1.118 |  | - |  |
| >Broadcast F1-U Context ReferenceE1 | O |  | 9.3.1.139a |  | YES | ignore |
| BC MRB Failed List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >Cause | M |  | 9.3.1.2 |  | - |  |
| Available BC MRB Configuration | O |  | BC MRB Setup Configuration 9.3.1.114 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

9.3.3.28	BC Bearer Context To Modify

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at 5GC To Setup or Modify | O |  | BC Bearer Context NG-U TNL Info at 5GC 9.3.1.112 |  | - |  |
| BC MRB To Setup List | O |  | BC MRB Setup Configuration 9.3.1.114 |  | - |  |
| BC MRB To Modify List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >BC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.119 |  | - |  |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 |  | - |  |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. | - |  |
| >Broadcast F1-U Context ReferenceE1 | O |  | 9.3.1.139a | Absence of this IE means removal of corresponding F1-U Tunnel | YES | ignore |
| >Additional F1-U TNL Info at DU List |  | 0..1 |  | To maintain current list if absent. | YES | ignore |
| >>Additional F1-U TNL Info at DU Item |  | 0..<maxnoofDUs-1> |  | Entire list of additional F1-U TNL Info | - |  |
| >>>Broadcast F1-U Context ReferenceE1 | M |  | 9.3.1.139a |  | - |  |
| >>>BC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.119 | This IE should be present if the F1-U tunnel is newly added or updated | - |  |
| BC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |
| maxnoofDUs-1 | Maximum no. of DUs in a RAN node minus one. Value is 511. |

9.3.3.29	BC Bearer Context To Modify Response

This IE contains MBS session resource related information used to confirm a BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.116 |  | - |  |
| BC MRB Setup or Modify Response List |  | 1..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >MBS QoS Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  | - |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - |  |
| >BC Bearer Context F1-U TNL Info at CU | O |  | 9.3.1.118 |  | - |  |
| >Broadcast F1-U Context ReferenceE1 | O |  | 9.3.1.139a |  | YES | ignore |
| >Additional F1-U TNL Info at CU List |  | 0..1 |  |  | YES | ignore |
| >>Additional F1-U TNL Info at CU Item |  | 1..<maxnoofDUs-1> |  | Include newly added F1-U TNL Info | - |  |
| >>>Broadcast F1-U Context ReferenceE1 | M |  | 9.3.1.139a |  | - |  |
| >>>BC Bearer Context F1-U TNL Info at CU | M |  | 9.3.1.118 |  | - |  |
| BC MRB Failed List |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >Cause | M |  | 9.3.1.2 |  | - |  |
| Available BC MRB Configuration | O |  | BC MRB Setup Configuration 9.3.1.114 | In case the shared MBS NG-U termination had a different MRB Configuration applied. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |
| maxnoofDUs-1 | Maximum no. of DUs in a RAN node minus one. Value is 511. |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-QoSMonitoringDisabled,

id-PDCP-StatusReportIndication,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

id-RedundantPDUSessionInformation,

id-RedundantPDUSessionInformation-used,

id-QoS-Mapping-Information,

id-MDTConfiguration,

id-TraceCollectionEntityURI,

id-EHC-Parameters,

id-DAPSRequestInfo,

id-EarlyForwardingCOUNTReq,

id-EarlyForwardingCOUNTInfo,

id-AlternativeQoSParaSetList,

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-DataForwardingtoNG-RANQoSFlowInformationList,

id-MaxCIDEHCDL,

id-ignoreMappingRuleIndication,

id-EarlyDataForwardingIndicator,

id-QoSFlowsDRBRemapping,

id-SecurityIndicationModify,

id-DataForwardingSourceIPAddress,

id-M4ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-PDUSession-PairID,

id-SurvivalTime,

id-UDC-Parameters,

id-SecurityIndication,

id-SecurityResult,

id-SDTindicatorSetup,

id-SDTindicatorMod,

id-DiscardTimerExtended,

id-MCForwardingResourceRequest,

id-MCForwardingResourceIndication,

id-MCForwardingResourceResponse,

id-MCForwardingResourceRelease,

id-MCForwardingResourceReleaseIndication,

id-PDCP-COUNT-Reset,

id-MBSSessionAssociatedInfoNonSupportToSupport,

id-VersionID,

id-MBSAreaSessionID,

id-Secondary-PDU-Session-Data-Forwarding-Information,

id-AdditionalF1U-TNL-InfoAtCU-List,

id-AdditionalF1U-TNL-InfoAtDU-List,

id-BroadcastF1UContextReferenceE1,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

maxnoofMRBs,

maxnoofMBSSessionIDs,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofSPLMNs,

maxnoofMDTPLMNs,

maxnoofExtSliceItems,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofExtNRCGI,

maxnoofECGI,

maxnoofSMBRValues,

maxnoofDUs-1

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

-- A

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

AdditionalF1U-TNL-InfoAtDU-List ::= SEQUENCE (SIZE(0..maxnoofDUs-1)) OF AdditionalF1U-TNL-InfoAtDU-Item

AdditionalF1U-TNL-InfoAtDU-Item ::= SEQUENCE {

broadcastF1UContextReferenceE1		BroadcastF1UContextReferenceE1,

bcBearerContextF1U-TNLInfoatDU		BCBearerContextF1U-TNLInfoatDU	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AdditionalF1U-TNL-InfoAtDU-Item-ExtIEs} }	OPTIONAL,

...

}

AdditionalF1U-TNL-InfoAtDU-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalF1U-TNL-InfoAtCU-List ::= SEQUENCE (SIZE(1..maxnoofDUs-1)) OF AdditionalF1U-TNL-InfoAtCU-Item

AdditionalF1U-TNL-InfoAtCU-Item ::= SEQUENCE {

broadcastF1UContextReferenceE1		BroadcastF1UContextReferenceE1,

bcBearerContextF1U-TNLInfoatCU		BCBearerContextF1U-TNLInfoatCU,

iE-Extensions		ProtocolExtensionContainer { {AdditionalF1U-TNL-InfoAtCU-Item-ExtIEs} }	OPTIONAL,

...

}

AdditionalF1U-TNL-InfoAtCU-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

-- B

-- BCBearerContextToSetup

BCBearerContextToSetup ::= SEQUENCE {

snssai									SNSSAI,

bcBearerContextNGU-TNLInfoat5GC			BCBearerContextNGU-TNLInfoat5GC				OPTIONAL,

bcMRBToSetupList						BCMRBSetupConfiguration,

requestedAction							RequestedAction4AvailNGUTermination			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetup-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetup-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCBearerContextNGU-TNLInfoat5GC::= CHOICE {

locationindependent				MBSNGUInformationAt5GC,

locationdependent				LocationDependentMBSNGUInformationAt5GC,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextNGU-TNLInfoat5GC-ExtIEs}}

}

BCBearerContextNGU-TNLInfoat5GC-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBSetupConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBSetupConfiguration-Item

BCMRBSetupConfiguration-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

mbs-pdcp-config					PDCP-Configuration,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBSetupConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBSetupConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-BroadcastF1UContextReferenceE1	CRITICALITY ignore	EXTENSION BroadcastF1UContextReferenceE1	PRESENCE optional},

...

}

-- BCBearerContextToSetupResponse

BCBearerContextToSetupResponse ::= SEQUENCE {

bcBearerContextNGU-TNLInfoatNGRAN			BCBearerContextNGU-TNLInfoatNGRAN			OPTIONAL,

bcMRBSetupResponseList					BCMRBSetupResponseList,

bcMRBFailedList							BCMRBFailedList							OPTIONAL,

availableBCMRBConfig						BCMRBSetupConfiguration					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetupResponse-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetupResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCBearerContextNGU-TNLInfoatNGRAN::= CHOICE {

locationindependent				MBSNGUInformationAtNGRAN,

locationdependent				LocationDependentMBSNGUInformationAtNGRAN,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextNGU-TNLInfoatNGRAN-ExtIEs}}

}

BCBearerContextNGU-TNLInfoatNGRAN-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBSetupResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBSetupResponseList-Item

BCMRBSetupResponseList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

qosflow-setup						QoS-Flow-List,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

bcBearerContextF1U-TNLInfoatCU 		BCBearerContextF1U-TNLInfoatCU,

iE-Extensions		ProtocolExtensionContainer { {BCMRBSetupResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBSetupResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-BroadcastF1UContextReferenceE1		CRITICALITY ignore	EXTENSION BroadcastF1UContextReferenceE1			PRESENCE optional},

...

}

BCBearerContextF1U-TNLInfoatCU ::= CHOICE {

locationindependent				MBSF1UInformationAtCU,

locationdependent				LocationDependentMBSF1UInformationAtCU,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextF1U-TNLInfoatCU-ExtIEs}}

}

BCBearerContextF1U-TNLInfoatCU-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBFailedList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBFailedList-Item

BCMRBFailedList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

cause								Cause,

iE-Extensions		ProtocolExtensionContainer { {BCMRBFailedList-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBFailedList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

-- BCBearerContextToModify

BCBearerContextToModify ::= SEQUENCE {

bcBearerContextNGU-TNLInfoat5GC		BCBearerContextNGU-TNLInfoat5GC	OPTIONAL,

bcMRBToSetupList						BCMRBSetupConfiguration			OPTIONAL,

bcMRBToModifyList					BCMRBModifyConfiguration			OPTIONAL,

bcMRBToRemoveList					BCMRBRemoveConfiguration			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModify-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModify-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCMRBModifyConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBModifyConfiguration-Item

BCMRBModifyConfiguration-Item ::= SEQUENCE {

mrb-ID							MRB-ID,

bcBearerContextF1U-TNLInfoatDU 	BCBearerContextF1U-TNLInfoatDU	OPTIONAL,

mbs-pdcp-config					PDCP-Configuration				OPTIONAL,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List		OPTIONAL,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBModifyConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBModifyConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-BroadcastF1UContextReferenceE1		CRITICALITY ignore	EXTENSION BroadcastF1UContextReferenceE1			PRESENCE optional}|

{ID id-AdditionalF1U-TNL-InfoatDU-List		CRITICALITY ignore	EXTENSION AdditionalF1U-TNL-InfoAtDU-List			PRESENCE optional},

...

}

BCBearerContextF1U-TNLInfoatDU ::= CHOICE {

locationindependent				MBSF1UInformationAtDU,

locationdependent				LocationDependentMBSF1UInformationAtDU,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextF1U-TNLInfoatDU-ExtIEs}}

}

BCBearerContextF1U-TNLInfoatDU-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBRemoveConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MRB-ID

-- BCBearerContextToModifyResponse

BCBearerContextToModifyResponse ::= SEQUENCE {

bcBearerContextNGU-TNLInfoatNGRAN			BCBearerContextNGU-TNLInfoatNGRAN			OPTIONAL,

bcMRBSetupModifyResponseList				BCMRBSetupModifyResponseList,

bcMRBFailedList							BCMRBFailedList							OPTIONAL,

availableBCMRBConfig						BCMRBSetupConfiguration					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModifyResponse-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModifyResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCMRBSetupModifyResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBSetupModifyResponseList-Item

BCMRBSetupModifyResponseList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

qosflow-setup						QoS-Flow-List			OPTIONAL,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

bcBearerContextF1U-TNLInfoatCU 		BCBearerContextF1U-TNLInfoatCU		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBSetupModifyResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBSetupModifyResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-BroadcastF1UContextReferenceE1		CRITICALITY ignore	EXTENSION BroadcastF1UContextReferenceE1			PRESENCE optional}|

{ID id-AdditionalF1U-TNL-InfoAtCU-List		CRITICALITY ignore	EXTENSION AdditionalF1U-TNL-InfoAtCU-List			PRESENCE optional},

...

}

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

BroadcastF1UContextReferenceE1 ::= OCTET STRING (SIZE(4))

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

### 9.4.7	Constant Definitions

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

-- **************************************************************

--

-- Lists

--

-- **************************************************************

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

maxnoofDUs-1								INTEGER ::= 511

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

//////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////

id-AdditionalF1U-TNL-InfoAtCU-List								ProtocolIE-ID ::= 999 – to be assigned

id-AdditionalF1U-TNL-InfoAtDU-List							ProtocolIE-ID ::= 999 – to be assigned

id-BroadcastF1UContextReferenceE1								ProtocolIE-ID ::= 999 – to be assigned

///////////////////////////////////////////////////////////////////////end//////////////////////////////////////////////////////////////////////