3GPP TSG-RAN WG3 Meeting #122	R3-237915

Chicago, USA, 13th – 17th November 2023	was R3-237049

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.470 | CR | 0111 | rev | 7 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BLCR to 38.470) Introduction of NR MBS enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Huawei, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of NR MBS enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support added for enhancement for NR Multicast and Broadcast Services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancements for NR Multicast and Broadcast Services not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2; 6.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR 0281 TS 38.300 CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: integrates TP R3-232165. Rev1: updates the coversheet Rev2: update with the latest spec version Rev3: update the title as: (BL CR to TS 38.470) Introduction of NR MBS enhancements Rev4: update with the latest spec version Rev5: integrates TP R3-235881. Rev6: resumit to RAN3#122 Rev7: update the CR template |  |  |  |  |  |  |  |  |

-----------------Start of the Changes-------------------

5.2.2	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

The gNB-DU is responsible for the encoding of the NR-MIB message. In case broadcast of SIB1 and other SIBs is needed, the gNB-DU is responsible for the encoding of the SIB1 message, SIB10, SIB12, SIB13, SIB14, SIB15, SIB17, SIB18, SIB20 and SIBX, and the gNB-CU is responsible for the encoding of other SIBs. The gNB-DU may re-encode SIB9. The gNB-DU is responsible for the generation of the SystemInformation message.

NOTE:	The SIB19 is generated by the gNB-DU.

Editor’s Note: “SIBX” is a place holder for a new system information block providing configuration information for a multicast MCCH which needs to be specified by RAN2.

The gNB-CU is responsible for receiving the positioning assistance information from LMF, e.g the positioning related SIBs. The gNB-CU transparently sends the positioning assistance information to the gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance information in Positioning SI message(s).

To support Msg3 based on-demand SI and RRC Dedicated SIB Request as described in TS 38.331 [11], the gNB-CU can confirm the received SI request from the UE by including the UE identity, and command the gNB-DU to broadcast the requested SystemInformation messages including the other SI.

To support UE RRC Positioning SI acquisition mechanism, as described in TS 38.331 [11], the gNB-CU can confirm the received positioning SI request from the UE by including the UE identity, and command the gNB-DU to broadcast the requested positioning SI messages.

-----------------Next Change-------------------

6.1.13	NR MBS procedures

The F1 MBS procedures are listed below:

-	Broadcast Context Setup;

Broadcast Context Release (gNB-CU initiated);

-	Broadcast Context Modification;

-	Broadcast Context Release Request (gNB-DU initiated);

-	Multicast Group Paging procedure;

-	Multicast Context Setup;

-	Multicast Context Release (gNB-CU initiated);

-	Multicast Context Modification;

-	Multicast Context Release Request (gNB-DU initiated);

-	Multicast Distribution Setup;

-	Multicast Distribution Release;

-	Multicast Context Notification.

-----------------End of the Change-------------------