3GPP TSG-RAN WG3 Meeting #123-bis	R3-24xxxx

Changsha, China, 15 - 19 April 2024

Agenda Item:	21.2

Source:	ZTE

Title:	(TP to TS 37.340) Support of XR in DC

Document for:	Discussions & Approval

1	Introduction

This TP captures the agreements achieved in RAN3 #123-bis meeting.

2	Text Proposal for BL CR TS 37.340

## 13.x	eXtended Reality Services

13.x.1	Overview

The eXtended Reality Services as described in TS 38.300 [3] is extended to address the NR-DC operation.

### 13.x.2	Awareness

During the SN modification procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling.

The MN may provide XR traffic assistance information to the SN through Xn AP TSC Assistance Information (TSCAI) as specified in clause 5.37.8 of TS 23.501[3] (for both GBR and non-GBR QoS flows).

### 13.x.3	Non-Homogeneous support of PDU set based handling

If the MN supports PDU Set based handling, the MN signals the PDU Set Information over Xn-U if the SN has signaled the support of PDU Set based handling in the SN Addition Request Acknowledge message or the SN Modification Request Acknowledge message.

### 13.x.4	Discard

The PDCP hosting node notifies the corresponding node whether the PSI based SDU discarding is (re)configured/released for uplink discarding.