3GPP TSG-RAN3 Meeting # 123bis	R3-241643

# Changsha, China, 15-04-2024 – 19-04-2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1102 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MT Communication Handling procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, Nokia, Xiaomi, Qualcomm Inc., ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current MT Communication Handling procedure is lacking information to meet SA2 requirements. In fact, SA2 requirements are as follows: - In clause 4.8.1.1a of 23.502, it reads: “For each of the PDU sessions with user plane resources have been activated, the AMF invokes Nsmf_PDUSession_UpdateSMContext Request (PDU Session ID, Cause, Operation type, User Location Information, Age of Location Information, N2 SM Information (Secondary RAT usage data), CN based MT handling indication) towards SMF. The Operation Type is set to a value that indicates to stop user plane DL data transmissions towards the UE and enable data buffering. The SMF starts data buffering for MT data if the data buffering is handled in SMF.” This is missing in the NGAP message, and the AMF has no precise knowledge if the user plane resources for a given PDU session is established or not. - Similarly, during resume, 4.8.2.2 of 23.502 “4b.2 The AMF invokes Nsmf_PDUSession_UpdateSMContext Request towards SMF indicating the Downlink data delivery for each PDU session with active user plane, if the AMF has requested data buffering as described in clause 4.8.1.1a.” The AMF may not have knowledge of the list of PDU sessions that have active user plane connection, and requiring the AMF to remember there was DL data/signaling for a given PDU session from the SMF and generate the Update SM Context request towards that SMF correspondingly adds a new requirement to the AMF, and actually it is not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Since the NG-RAN should be able to tell AMF which PDU sessions have corresponding PDU session resources established in the NG-RAN, the NG-RAN can include the list of those PDU session IDs in the MT COMMUNICATION HANDLING REQUEST message to instruct the AMF that the user plane connections for these PDU sessions are to be “Suspended” / “activated” since the UE enters RRC Inactive with long eDRX, so that the AMF can trigger UpdateSmContext correspondingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF may have no knowledge which PDU session(s) have active user plane resource connection and cannot correctly identify SMF(s) with activated user plane PDU session resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.13.2, 9.2.2.22, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: Resubmission to RAN3#123bis and addition of previously dubitative but now convinced supporters |  |  |  |  |  |  |  |  |

<Start of changes>

### 8.3.13	MT Communication Handling

#### 8.3.13.1	General

The purpose of the MT Communication Handling procedure is to request the AMF to activate or deactivate the CN based MT communication handling for a UE in RRC_INACTIVE state with eDRX beyond 10.24 seconds as specified in TS 23.501 [9]. The procedure uses UE-associated signalling.

#### 8.3.13.2	Successful Operation

Figure 8.3.13.2-1: MT Communication Handling procedure. Successful operation.

The NG-RAN node initiates the procedure by sending the MT COMMUNICATION HANDLING REQUEST message to the AMF.

If the 5GC Action IE is included in the MT COMMUNICATION HANDLING REQUEST message and set to "HLCom Activate", the AMF shall activate MT communication handling for the PDU session(s) indicated in the PDU Session Resource List IE as specified in TS 23.501 [9] and take into account the NR Paging Long eDRX Information for RRC INACTIVE IE when applying MT communication handling as specified in TS 38.304 [12] and TS 23.502 [10].

If the 5GC Action IE is included in the MT COMMUNICATION HANDLING REQUEST message and set to "HLCom Deactivate", the AMF shall deactivate MT communication handling for the PDU session(s) indicated in the PDU Session Resource List IE as specified in TS 23.501 [9].

<Next change>

#### 9.2.2.22	MT COMMUNICATION HANDLING REQUEST

This message is sent by the NG-RAN node to the AMF to request activating or deactivating CN based MT communication handling for UEs in RRC_INACTIVE state with long eDRX beyond 10.24 seconds as specified in TS 23.501 [9].

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| CHOICE 5GC Action | M |  |  |  | YES | reject |
| >HLCom Activate |  |  |  |  |  |  |
| >>NR Paging Long eDRX Information for RRC INACTIVE | M |  | 9.3.3.64 |  | - |  |
| >HLcom Deactivate |  |  |  |  |  |  |
| >>UE Reachability Indication | M |  | ENUMERATED (true, …) |  | - |  |
| PDU Session Resource List |  | 1 |  |  | YES | reject |
| >PDU Session Resource Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

<Next change>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<Unmodified parts omitted>

Partially-Allowed-NSSAI,

PC5QoSParameters,

PDUSessionAggregateMaximumBitRate,

PDUSessionList,

PDUSessionResourceAdmittedList,

<Unmodified parts omitted>

id-Partially-Allowed-NSSAI,

id-PC5QoSParameters,

id-PDUSessionResourceAdmittedList,

id-PDUSessionList,

id-PDUSessionResourceFailedToModifyListModCfm

<Unmodified parts omitted>

-- **************************************************************

--

-- MT Communication Handling Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

-- MT COMMUNICATION HANDLING REQUEST

--

-- **************************************************************

MTCommunicationHandlingRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MTCommunicationHandlingRequestIEs} },

...

}

MTCommunicationHandlingRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-FiveGCAction			CRITICALITY reject	TYPE FiveGCAction		PRESENCE mandatory	}|

{ ID id-PDUSessionList			CRITICALITY reject	TYPE PDUSessionList		PRESENCE mandatory	},

...

}

<Next change>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<Unmodified parts omitted>

PDUSessionResourceItemCxtRelCpl-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionResourceReleaseResponseTransfer	CRITICALITY ignore	EXTENSION OCTET STRING (CONTAINING PDUSessionResourceReleaseResponseTransfer)	PRESENCE optional	},

...

}

PDUSessionResourceListCxtRelReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceItemCxtRelReq

PDUSessionResourceItemCxtRelReq ::= SEQUENCE {

pDUSessionID		PDUSessionID,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceItemCxtRelReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceItemCxtRelReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceListHORqd ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceItemHORqd

PDUSessionResourceItemHORqd ::= SEQUENCE {

pDUSessionID							PDUSessionID,

handoverRequiredTransfer				OCTET STRING (CONTAINING HandoverRequiredTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceItemHORqd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionList	 ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionList-Item

PDUSessionList-Item ::= SEQUENCE {

pDUSessionID							PDUSessionID,

iE-Extensions		ProtocolExtensionContainer { { PDUSessionList-Item-ExtIEs} }	OPTIONAL,

...

}

<Next change>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<Unmodified parts omitted>

id-ERedCapIndication									ProtocolIE-ID ::= 427

id-XrDeviceWith2Rx										ProtocolIE-ID ::= 428

id-UserPlaneErrorIndicator								ProtocolIE-ID ::= 429

id-SLPositioningRangingServiceInfo						ProtocolIE-ID ::= 430

id-PDUSessionList										ProtocolIE-ID ::= xxx

<End of changes>