3GPP TSG-RAN WG3 Meeting #123-bis	R3-241721

# Changsha, China, 15th – 19th April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1256 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement for NR-DC support of XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add changes for support the NR-DC related enhancement for the XR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the support for PDU set based handling in the SN addition and modification procedures. - Add the support for ECN marking or congestion information report in the SN addition and modification procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancement on NR-DC for XR is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.3.3.2, 9.2.1.5, 9.2.1.6, 9.2.1.7, 9.2.1.8, 9.2.1.9, 9.2.1.10, 9.2.1.11, 9.2.1.12, 9.2.3.205, 9.2.3.X (new), 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

------ Unchanged part skipped ------

For each QoS flow which has been successfully established in the S-NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and shall, if supported, perform delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and shall, if supported, use it for RAN part delay reporting. In case such a QoS flow is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring. If the QoS Monitoring Reporting Frequency IE is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it for RAN part delay reporting. For each QoS Flow, if the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

For each DRB configured as MN-terminated split bearer/SCG bearer, if the PDU Set QoS Parameters IE is included in the DRB QoS IE in the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

For each QoS flow which has been successfully established in the S-NG-RAN node, if the ECN Marking or Congestion Information Reporting Request IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it accordingly for the specific QoS flow. If the ECN Marking or Congestion Information Reporting Status IE is included in the PDU Session Resource Setup Response Info – SN terminated IE contained in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to deduce if ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active.

For each DRB configured as MN-terminated split bearer/SCG bearer, if the ECN Marking or Congestion Information Reporting Request IE is included in the PDU Session Resource Setup Info – MN terminated IE contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it accordingly for the specific DRB. If the ECN Marking or Congestion Information Reporting Status IE is included in the PDU Session Resource Setup Response Info – MN terminated IE contained in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to deduce if ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active.

------ Unchanged part skipped ------

NEXT CHANGE

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

------ Unchanged part skipped ------

For each QoS flow which has been successfully added or modified in the S-NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE or the PDU Session Resource Modification Info – SN terminated IE, the S-NG-RAN node shall store this information, and shall, if supported, perform delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE or the PDU Session Resource Modification Info – SN terminated IE, the S-NG-RAN node shall store this information, and shall, if supported, use it for RAN part delay reporting. In case such a QoS flow is included in the DRBs To Be Setup List IE or the DRBs To Be Modified List IE within the PDU Session Resource Setup Response Info – SN terminated IE or the PDU Session Resource Modification Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring. If the QoS Monitoring Reporting Frequency IE is included in the DRBs To Be Setup List IE or the DRBs To Be Modified List IE within the PDU Session Resource Setup Response Info – SN terminated IE or the PDU Session Resource Modification Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it for RAN part delay reporting. For each QoS Flow, if the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resource Setup Info – SN terminated IE or the PDU Session Resource Modification Info – SN terminated IE, the S-NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

For each DRB configured as MN-terminated split bearer/SCG bearer, if the PDU Set QoS Parameters IE is included in the DRB QoS IE in the PDU Session Resource Setup Info – MN terminated IE or the PDU Session Resource Modification Info – MN terminated IE, the S-NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

For each QoS flow which has been successfully added or modified in the S-NG-RAN node, if the ECN Marking or Congestion Information Reporting Request IE is included in the PDU Session Resource Setup Info – SN terminated IE or the PDU Session Resource Modification Info – SN terminated IE contained in the S-NODE MIDIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use it accordingly for the specific QoS flow. If the ECN Marking or Congestion Information Reporting Status IE is included in the PDU Session Resource Setup Response Info – SN terminated IE or the PDU Session Resource Modification Response Info – SN terminated IE contained in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to deduce if ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active.

For each DRB configured as MN-terminated split bearer/SCG bearer, if the ECN Marking or Congestion Information Reporting Request IE is included in the PDU Session Resource Setup Info – MN terminated IE or the PDU Session Resource Modification Info – MN terminated IE contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it accordingly for the specific DRB. If the ECN Marking or Congestion Information Reporting Status IE is included in the PDU Session Resource Setup Response Info – MN terminated IE or the PDU Session Resource Modification Response Info – MN terminated IE contained in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to deduce if ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active.

------ Unchanged part skipped ------

NEXT CHANGE

#### 9.2.1.5	PDU Session Resource Setup Info – SN terminated

This IE contains information for the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UL NG-U UP TNL Information at UPF | M |  | UP Transport Layer Information 9.2.3.30 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | – |  |
| PDU Session Type | M |  | 9.2.3.19 |  | – |  |
| Network Instance | O |  | 9.2.3.85 | This IE shall be ignored if the Common Network Instance IE is present. | – |  |
| QoS Flows To Be Setup List |  | 1 |  |  | – |  |
| >QoS Flow To Be Setup Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.2.3.5 | For GBR QoS flows, this IE contains GBR QoS flow information as received at NG-C | – |  |
| >>Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.3.6 | This IE contains M-Node offered GBR QoS Flow Information. | – |  |
| >>TSC Traffic Characteristics | O |  | 9.2.3.114 | Traffic pattern information associated with the QFI. Details in TS 23.501 [7]. | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.2.3.118 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Request | O |  | 9.2.3.205 |  | YES | ignore |
| Data Forwarding and Offloading Info from source NG-RAN node | O |  | 9.2.1.17 |  | – |  |
| Security Indication | O |  | 9.2.3.52 |  | – |  |
| Security Result | O |  | 9.2.3.67 | Indicates security activation status in MN. | YES | reject |
| Common Network Instance | O |  | 9.2.3.92 |  | YES | ignore |
| Default DRB Allowed | O |  | 9.2.3.93 |  | YES | ignore |
| Split Session Indicator | O |  | 9.2.3.94 |  | YES | reject |
| Non-GBR Resources Offered | O |  | 9.2.3.98 |  | YES | ignore |
| Redundant UL NG-U UP TNL Information at UPF | O |  | UP Transport Layer Information 9.2.3.30 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs for the redundant transmission. | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.2.3.92 |  | YES | ignore |
| Redundant PDU Session Information | O |  | 9.2.3.112 |  | YES | ignore |

NEXT CHANGE

#### 9.2.1.6	PDU Session Resource Setup Response Info – SN terminated

This IE contains the result of the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information at NG-RAN | M |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – |  |
| DRBs To Be Setup List |  | 0..1 |  |  | – |  |
| >DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>SN UL PDCP UP TNL Information | M |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>PDCP SN Length | O |  | 9.2.3.63 | Indicates the PDCP SN length of the DRB. | – |  |
| >>RLC Mode | M |  | 9.2.3.28 | Indicates the RLC mode to be used in the assisting node. | – |  |
| >>secondary SN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | – |  |
| >>Duplication Activation | O |  | 9.2.3.71 | Information on the initial state of UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | – |  |
| >>UL Configuration | O |  | 9.2.3.75 | Information about UL usage in the M-NG-RAN node. This IE is used when the concerned DRB has both MCG resource and SCG resource configured i.e. the concerned DRB is configured as split bearer. | – |  |
| >>QoS Flows Mapped To DRB List |  | 1 |  |  | – |  |
| >>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>MCG requested GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.3.6 | This IE contains GBR QoS Flow Information necessary for the MCG part. | – |  |
| >>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>>>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.2.3.103 |  | YES | ignore |
| >>>>Source DL Forwarding IP Address | O |  | Transport Layer Address 9.2.3.29 | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Status | O |  | 9.2.3.X |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |
| >>RLC Duplication Information | O |  | 9.2.3.111 | . | – |  |
| Data Forwarding Info from target NG-RAN node | O |  | 9.2.1.16 |  | – |  |
| QoS Flows Not Admitted List | O |  | QoS Flow List with Cause 9.2.1.4 |  | – |  |
| Security Result | O |  | 9.2.3.67 |  | – |  |
| DRB IDs taken into use | O |  | DRB List 9.2.1.29 | Indicating the DRB IDs taken into use by the target NG-RAN node, as specified in TS 37.340 [8]. | YES | reject |
| Redundant DL NG-U UP TNL Information at NG-RAN | O |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs for the redundant transmission. | YES | ignore |
| Used RSN Information | O |  | Redundant PDU Session Information 9.2.3.112 |  | YES | ignore |
| Data Forwarding and Offloading Info from source NG-RAN node | O |  | 9.2.1.17 | Contains data forwarding proposal for S-CPAC, to be used later when the S-NG-RAN node is selected for access. | YES | ignore |

NEXT CHANGE

#### 9.2.1.7	PDU Session Resource Setup Info – MN terminated

This IE contains information for the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Type | M |  | 9.2.3.19 |  | – |  |
| DRBs To Be Setup List |  | 1 |  |  | – |  |
| >DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>MN UL PDCP UP TNL Information | M |  | UP Transport Parameters 9.2.3.76 | M-NG-RAN node endpoint(s) of a DRB’s Xn-U transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>RLC Mode | M |  | 9.2.3.28 | Indicates the RLC mode to be used in the assisting node. | – |  |
| >>UL Configuration | O |  | 9.2.3.75 | Information about UL usage in the S-NG-RAN node. This IE is used when the concerned DRB has both MCG resource and SCG resource configured i.e. the concerned DRB is configured as split bearer. | – |  |
| >>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>PDCP SN Length | O |  | 9.2.3.63 | Indicates the PDCP SN length of the DRB. | – |  |
| >>secondary MN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | M-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | – |  |
| >>Duplication Activation | O |  | 9.2.3.71 | Information on the initial state of UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | – |  |
| >>QoS Flows Mapped To DRB List |  | 1 |  |  | – |  |
| >>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>QoS Flow Level QoS Parameters | M |  | 9.2.3.5 |  | – |  |
| >>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>>>TSC Traffic Characteristics | O |  | 9.2.3.114 | Traffic pattern information associated with the QFI. Details in TS 23.501 [7]. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | M-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |
| >>RLC Duplication Information | O |  | 9.2.3.111 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Request | O |  | 9.2.3.205 |  | YES | ignore |

NEXT CHANGE

#### 9.2.1.8	PDU Session Resource Setup Response Info – MN terminated

This IE contains the result of the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRBs Admitted List |  | 1 |  |  | – |  |
| >DRBs Admitted Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>SN DL SCG UP TNL Information | M |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs. | – |  |
| >>secondary SN DL SCG UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs in case of PDCP duplication. | – |  |
| >>LCID | O |  | 9.2.3.70 | LCID for primary path or LCID for split secondary path for fallback to split bearer if PDCP duplication is applied | – |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs in case of additional PDCP duplication. | – |  |
| >>QoS Flows Mapped To DRB List |  | 0..1 |  |  | YES | ignore |
| >>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>Current QoS Parameters Set Index | M |  | Alternative QoS Parameters Set Index 9.2.3.103 |  | – |  |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.2.3.X |  | YES | ignore |
| DRBs Not Admitted To Be Setup or Modified List | O |  | DRB List with Cause 9.2.1.28 |  | YES | ignore |

NEXT CHANGE

#### 9.2.1.9	PDU Session Resource Modification Info – SN terminated

This IE contains information related to a PDU session resource for an M-NG-RAN node initiated request to modify DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UL NG-U UP TNL Information at UPF | O |  | UP Transport Layer Information 9.2.3.30 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | – |  |
| Network Instance | O |  | 9.2.3.85 | This IE shall be ignored if the Common Network Instance IE is present. | – |  |
| QoS Flows To Be Setup List |  | 0..1 |  |  | – |  |
| >QoS Flows To Be Setup Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.2.3.5 | For GBR QoS flows, this IE contains GBR QoS flow information as received at NG-C. | – |  |
| >>Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.3.6 | This IE contains M-Node offered GBR QoS Flow Information. | – |  |
| >>TSC Traffic Characteristics | O |  | 9.2.3.114 | Traffic pattern information associated with the QFI. Details in TS 23.501 [7]. | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.2.3.118 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Request | O |  | 9.2.3.205 |  | YES | ignore |
| Data Forwarding and Offloading Info from source NG-RAN node | O |  | 9.2.1.17 | Applicable for the QoS flows contained in the QoS Flows To Be Setup List IE. | – |  |
| QoS Flows To Be Modified List |  | 0..1 |  |  | – |  |
| >QoS Flows To Be Modified Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>QoS Flow Level QoS Parameters | O |  | 9.2.3.5 | For GBR QoS flows, this IE contains GBR QoS flow information as received at NG-C. | – |  |
| >>Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.3.6 | This IE contains M-Node offered GBR QoS Flow Information. | – |  |
| >>QoS Flow Mapping Indication | O |  | 9.2.3.79 | This IE is not applicable in this version of the specification. | – |  |
| >>TSC Traffic Characteristics | O |  | 9.2.3.114 | Traffic pattern information associated with the QFI. Details in TS 23.501 [7]. | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.2.3.118 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Request | O |  | 9.2.3.205 |  | YES | ignore |
| QoS Flows To Be Released List |  | 0..1 | QoS Flow List with Cause 9.2.1.4 |  | – |  |
| DRBs To Be Modified List |  | 0..1 |  |  | – |  |
| >DRBs to Be Modified Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>MN DL CG UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | M-NG-RAN node GTP-U endpoint(s) of a DRB’s Xn transport bearer at its lower layer CG resource. For delivery of DL PDUs. | – |  |
| >>secondary MN DL CG UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | M-NG-RAN node GTP-U endpoint(s) of a DRB’s Xn transport bearer at its lower layer CG resource. For delivery of DL PDUs in case of PDCP duplication. | – |  |
| >>LCID | O |  | 9.2.3.70 | LCID for primary path or LCID for split secondary path for fallback to split bearer if PDCP duplication is applied | – |  |
| >>RLC Status | O |  | 9.2.3.80 |  | – |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | M-NG-RAN node GTP-U endpoint(s) of a DRB’s Xn transport bearer at its lower layer CG resource. For delivery of DL PDUs in case of additional PDCP duplication. | – |  |
| DRBs To Be Released List | O |  | DRB List with Cause 9.2.1.28 |  | – |  |
| Common Network Instance | O |  | 9.2.3.92 |  | YES | ignore |
| Default DRB Allowed | O |  | 9.2.3.93 |  | YES | ignore |
| Non-GBR Resources Offered | O |  | 9.2.3.98 |  | YES | ignore |
| Redundant UL NG-U UP TNL Information at UPF | O |  | UP Transport Layer Information 9.2.3.30 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs for the redundant transmission | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.2.3.92 |  | YES | ignore |
| Security Indication | O |  | 9.2.3.52 |  | YES | ignore |

NEXT CHANGE

#### 9.2.1.10	PDU Session Resource Modification Response Info – SN terminated

This IE contains the PDU session resource related result of an M-NG-RAN node initiated request to modify DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information at NG-RAN | O |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – |  |
| DRBs To Be Setup List |  | 0..1 |  |  | – |  |
| >DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>SN UL PDCP UP TNL Information | M |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>PDCP SN Length | O |  | 9.2.3.63 | Indicates the PDCP SN length of the DRB. | – |  |
| >>RLC Mode | M |  | 9.2.3.28 | Indicates the RLC mode to be used in the assisting node. | – |  |
| >>secondary SN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | – |  |
| >>Duplication Activation | O |  | 9.2.3.71 | Information on the initial state of UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | – |  |
| >>UL Configuration | O |  | 9.2.3.75 | Information about UL usage in the M-NG-RAN node. This IE is used when the concerned DRB has both MCG resource and SCG resource configured i.e. the concerned DRB is configured as split bearer. | – |  |
| >>QoS Flows Mapped To DRB List |  | 1 |  |  | – |  |
| >>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>MCG requested GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.3.6 | This IE contains GBR QoS Flow Information necessary for the MCG part. | – |  |
| >>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>>>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.2.3.103 |  | YES | ignore |
| >>>>Source DL Forwarding IP Address | O |  | Transport Layer Address 9.2.3.29 | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Status | O |  | 9.2.3.X |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |
| >>RLC Duplication Information | O |  | 9.2.3.111 |  | YES | ignore |
| Data Forwarding Info from target NG-RAN node | O |  | 9.2.1.16 | Applicable for the QoS flows in DRBs to be setup. | – |  |
| DRBs To Be Modified List |  | 0..1 |  |  | – |  |
| >DRBs to Be Modified Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>SN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>DRB QoS | O |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>QoS Flows Mapped to DRB List |  | 0..1 |  | Overwriting the existing QoS Flow List | – |  |
| >>>QoS Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>MCG requested GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.3.6 | This IE contains GBR QoS Flow Information necessary for the MCG part. | – |  |
| >>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>>>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.2.3.103 |  | YES | ignore |
| >>>>Source DL Forwarding IP Address | O |  | Transport Layer Address 9.2.3.29 | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Status | O |  | 9.2.3.X |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |
| >>RLC Duplication Information | O |  | 9.2.3.111 |  | YES | ignore |
| >>secondary SN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | YES | ignore |
| >>PDCP Duplication Configuration | O |  | 9.2.3.86 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.2.3.71 |  | YES | ignore |
| DRBs To Be Released List | O |  | DRB List with Cause 9.2.1.28 |  | – |  |
| Data Forwarding and Offloading Info from source NG-RAN node | O |  | 9.2.1.17 | Contains DL Data Forwarding indications for QoS Flows removed from the SDAP in the SN. | – |  |
| QoS Flows Not Admitted to be Added List | O |  | QoS Flow List with Cause 9.2.1.4 |  | – |  |
| QoS Flows Released List | O |  | QoS Flow List with Cause 9.2.1.4 |  | – |  |
| DRB IDs taken into use | O |  | DRB List 9.2.1.29 | Indicating the DRB IDs taken into use by the target NG-RAN node, as specified in TS 37.340 [8]. | YES | reject |
| Redundant DL NG-U UP TNL Information at NG-RAN | O |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs for the redundant transmission. | YES | ignore |
| Security Result | O |  | 9.2.3.67 |  | YES | ignore |

NEXT CHANGE

#### 9.2.1.11	PDU Session Resource Modification Info – MN terminated

This IE contains information related to PDU session resource for an M-NG-RAN node initiated request to modify DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Type | M |  | 9.2.3.19 |  | – |  |
| DRBs To Be Setup List |  | 0..1 |  |  | – |  |
| >DRBs to Be Setup Item |  | 1 .. <maxnoof DRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>MN UL PDCP UP TNL Information | M |  | UP Transport Parameters 9.2.3.76 | M-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>RLC Mode | M |  | 9.2.3.28 | Indicates the RLC mode to be used in the assisting node. | – |  |
| >>UL Configuration | O |  | 9.2.3.75 | Information about UL usage in the S-NG-RAN node. This IE is used when the concerned DRB has both MCG resource and SCG resource configured i.e. the concerned DRB is configured as split bearer. | – |  |
| >>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>PDCP SN Length | O |  | 9.2.3.63 | Indicates the PDCP SN length of the DRB. | – |  |
| >>secondary MN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | M-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | – |  |
| >>Duplication Activation | O |  | 9.2.3.71 | Information on the initial state of UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | – |  |
| >>QoS Flows Mapped to DRB List |  | 1 |  |  | – |  |
| >>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>QoS Flow Level QoS Parameters | M |  | 9.2.3.5 |  | – |  |
| >>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | M-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |
| >>RLC Duplication Information | O |  | 9.2.3.111 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Request | O |  | 9.2.3.205 |  | YES | ignore |
| DRBs To Be Modified List |  | 0..1 |  |  | – |  |
| >DRBs to Be Modified Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>MN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | M-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs. | – |  |
| >>DRB QoS | O |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>secondary MN UL PDCP UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | M-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of PDCP duplication. | – |  |
| >>UL Configuration | O |  | 9.2.3.75 | Information about UL usage in the S-NG-RAN node. | – |  |
| >>PDCP Duplication Configuration | O |  | 9.2.3.86 |  | – |  |
| >>Duplication Activation | O |  | 9.2.3.71 | Information on the initial state of UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | – |  |
| >>QoS Flows Mapped To DRB List |  | 0..1 |  | Overwriting the existing QoS Flow List | – |  |
| >>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoof QoS Flows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>QoS Flow Level QoS Parameters | M |  | 9.2.3.5 |  | – |  |
| >>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |  | – |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | M-NG-RAN node endpoint(s) of a DRB’s Xn transport bearer at its PDCP resource. For delivery of UL PDUs in case of additional PDCP duplication. | – |  |
| >>RLC Duplication Information | O |  | 9.2.3.111 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Request | O |  | 9.2.3.205 |  | YES | ignore |
| DRBs To Be Released List | O |  | DRB List with Cause 9.2.1.28 |  | – |  |

NEXT CHANGE

#### 9.2.1.12	PDU Session Resource Modification Response Info – MN terminated

This IE contains the PDU session resource related result of an M-NG-RAN node initiated modification of DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRBs Admitted to be Setup or Modified List |  | 1 |  |  | – |  |
| >DRBs Admitted to be Setup or Modified Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>SN DL SCG UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs. | – |  |
| >>secondary SN DL SCG UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs in case of PDCP duplication. | – |  |
| >>LCID | O |  | 9.2.3.70 | LCID for primary path or LCID for split secondary path for fallback to split bearer if PDCP duplication is applied | – |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Item |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | – |  |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs in case of additional PDCP duplication. | – |  |
| >>QoS Flows Mapped To DRB List |  | 0..1 |  |  | YES | ignore |
| >>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.2.3.103 |  | – |  |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.2.3.X |  | YES | ignore |
| DRBs Released List | O |  | DRB List 9.2.1.29 |  | – |  |
| DRBs Not Admitted To Be Setup or Modified List | O |  | DRB List with Cause 9.2.1.28 |  | – |  |

NEXT CHANGE

#### 9.2.3.205	ECN Marking or Congestion Information Reporting Request

This IE indicates to the NG-RAN node to perform ECN marking or to report information for ECN marking or to report congestion information for a QoS flow or a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE ECN Marking or Congestion Information Request | M |  |  |  |
| >ECN Marking at RAN |  |  |  |  |
| >>ECN Marking at RAN Request | M |  | ENUMERATED (ul, dl, both, stop, …) |  |
| >ECN Marking at UPF |  |  |  |  |
| >>ECN Marking at UPF Request | M |  | ENUMERATED (ul, dl, both, stop, …) |  |
| >Congestion Information |  |  |  |  |
| >>Congestion Information Request | M |  | ENUMERATED (ul, dl, both, stop, …) |  |

NEXT CHANGE

#### 9.2.3.X	ECN Marking or Congestion Information Reporting Status

This IE indicates whether ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ECN Marking or Congestion Information Reporting Status | O |  | ENUMERATED (active, not active, …) | Indicates whether ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active. |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

------ Unchanged part skipped ------

id-DirectForwardingPathAvailability,

id-CHO-CPAC-Info,

id-CHO-Maxnoof-CondReconfig,

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-TAISliceUnavailableCellList,

id-MobileIABCell,

id-ECNMarkingorCongestionInformationReportingStatus,

maxEARFCN,

maxnoofAllowedAreas,

------ Unchanged part skipped ------

#### -- E

EarlyMeasurement ::= ENUMERATED {true, ...}

ECNMarkingorCongestionInformationReportingRequest ::= CHOICE {

eCNMarkingAtRANRequest				ECNMarkingAtRANRequest,

eCNMarkingAtUPFRequest				ECNMarkingAtUPFRequest,

congestionInformationRequest		CongestionInformationRequest,

choice-Extensions				ProtocolIE-Single-Container { {ECNMarkingorCongestionInformationReportingRequest-ExtIEs} }

}

ECNMarkingorCongestionInformationReportingRequest-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ECNMarkingAtRANRequest ::= ENUMERATED {ul, dl, both, stop,...}

ECNMarkingAtUPFRequest ::= ENUMERATED {ul, dl, both, stop,...}

CongestionInformationRequest	::= ENUMERATED {ul, dl, both, stop, ...}

ECNMarkingorCongestionInformationReportingStatus ::= ENUMERATED {active, not-active, ...}

------ Unchanged part skipped ------

-- **************************************************************

--

###### -- PDU Session Resource Setup Info - SN terminated

--

-- **************************************************************

PDUSessionResourceSetupInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF				UPTransportLayerInformation,

pduSessionType					PDUSessionType,

pduSessionNetworkInstance		PDUSessionNetworkInstance																	OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List-Setup-SNterminated,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource													OPTIONAL,

securityIndication				SecurityIndication																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-SNterminated-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecurityResult						CRITICALITY reject	EXTENSION SecurityResult							PRESENCE optional}|

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-DefaultDRB-Allowed					CRITICALITY ignore	EXTENSION DefaultDRB-Allowed						PRESENCE optional}|

{ ID id-SplitSessionIndicator				CRITICALITY reject	EXTENSION SplitSessionIndicator					PRESENCE optional}|

{ ID id-NonGBRResources-Offered				CRITICALITY ignore	EXTENSION NonGBRResources-Offered				PRESENCE optional}|

{ ID id-Redundant-UL-NG-U-TNLatUPF			CRITICALITY	ignore	EXTENSION UPTransportLayerInformation			PRESENCE optional}|

{ ID id-RedundantCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-RedundantPDUSessionInformation		CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation		PRESENCE optional},

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Setup-SNterminated-Item

QoSFlowsToBeSetup-List-Setup-SNterminated-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

offeredGBRQoSFlowInfo			GBRQoSFlowInfo																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics								CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional}|

{ ID id-RedundantQoSFlowIndicator								CRITICALITY ignore	EXTENSION RedundantQoSFlowIndicator	PRESENCE optional}|

{ ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Response Info - SN terminated

--

-- **************************************************************

PDUSessionResourceSetupResponseInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation,

dRBsToBeSetup					DRBsToBeSetupList-SetupResponse-SNterminated 	OPTIONAL,

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode			OPTIONAL,

qosFlowsNotAdmittedList			QoSFlows-List-withCause							OPTIONAL,

securityResult					SecurityResult									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse					CRITICALITY reject	EXTENSION DRB-List												PRESENCE optional}|

{ ID id-Redundant-DL-NG-U-TNLatNG-RAN			CRITICALITY ignore	EXTENSION UPTransportLayerInformation						PRESENCE optional}|

{ ID id-UsedRSNInformation						CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation					PRESENCE optional}|

{ ID id-S-CPAC-dataforwardinginfofromSource		CRITICALITY ignore	EXTENSION DataforwardingandOffloadingInfofromSource	PRESENCE optional},

...

}

DRBsToBeSetupList-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-SetupResponse-SNterminated-Item

DRBsToBeSetupList-SetupResponse-SNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

sN-UL-PDCP-UP-TNLInfo									UPTransportParameters,

dRB-QoS													QoSFlowLevelQoSParameters,

pDCP-SNLength											PDCPSNLength						OPTIONAL,

rLC-Mode												RLCMode,

uL-Configuration										ULConfiguration						OPTIONAL,

secondary-SN-UL-PDCP-UP-TNLInfo							UPTransportParameters				OPTIONAL,

duplicationActivation									DuplicationActivation				OPTIONAL,

qoSFlowsMappedtoDRB-SetupResponse-SNterminated			QoSFlowsMappedtoDRB-SetupResponse-SNterminated,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional},

...

}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

mCGRequestedGBRQoSFlowInfo		GBRQoSFlowInfo														OPTIONAL,

qosFlowMappingIndication		QoSFlowMappingIndication											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex									CRITICALITY ignore	EXTENSION QoSParaSetIndex				PRESENCE optional }|

{ ID id-SourceDLForwardingIPAddress								CRITICALITY ignore	EXTENSION TransportLayerAddress		PRESENCE optional}|

{ ID id-ECNMarkingorCongestionInformationReportingStatus		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingStatus		PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Info - MN terminated

--

-- **************************************************************

PDUSessionResourceSetupInfo-MNterminated ::= SEQUENCE {

pduSessionType					PDUSessionType,

dRBsToBeSetup					DRBsToBeSetupList-Setup-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeSetupList-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-Setup-MNterminated-Item

DRBsToBeSetupList-Setup-MNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

mN-UL-PDCP-UP-TNLInfo									UPTransportParameters,

rLC-Mode												RLCMode,

uL-Configuration										ULConfiguration						OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters,

pDCP-SNLength											PDCPSNLength						OPTIONAL,

secondary-MN-UL-PDCP-UP-TNLInfo							UPTransportParameters				OPTIONAL,

duplicationActivation									DuplicationActivation				OPTIONAL,

qoSFlowsMappedtoDRB-Setup-MNterminated		QoSFlowsMappedtoDRB-Setup-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional}|

{ ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional},

...

}

QoSFlowsMappedtoDRB-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsMappedtoDRB-Setup-MNterminated-Item

QoSFlowsMappedtoDRB-Setup-MNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

qosFlowMappingIndication		QoSFlowMappingIndication		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Setup Response Info - MN terminated

--

-- **************************************************************

PDUSessionResourceSetupResponseInfo-MNterminated ::= SEQUENCE {

dRBsAdmittedList				DRBsAdmittedList-SetupResponse-MNterminated,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-DRBsNotAdmittedSetupModifyList	CRITICALITY ignore	EXTENSION DRB-List-withCause		PRESENCE optional},

...

}

DRBsAdmittedList-SetupResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-SetupResponse-MNterminated-Item

DRBsAdmittedList-SetupResponse-MNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

sN-DL-SCG-UP-TNLInfo					UPTransportParameters,

secondary-SN-DL-SCG-UP-TNLInfo			UPTransportParameters				OPTIONAL,

lCID									LCID								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated	CRITICALITY ignore	EXTENSION	QoSFlowsMappedtoDRB-SetupResponse-MNterminated	PRESENCE optional}|

{ ID id-ECNMarkingorCongestionInformationStatus		CRITICALITY ignore	EXTENSION	ECNMarkingorCongestionInformationStatus	PRESENCE optional},

...

}

QoSFlowsMappedtoDRB-SetupResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item

QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item ::= SEQUENCE {

qoSFlowIdentifier				QoSFlowIdentifier,

currentQoSParaSetIndex			QoSParaSetIndex,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModificationInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF				UPTransportLayerInformation						OPTIONAL,

pduSessionNetworkInstance		PDUSessionNetworkInstance						OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List-Setup-SNterminated		OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource		OPTIONAL,

qosFlowsToBeModified-List		QoSFlowsToBeSetup-List-Modified-SNterminated	OPTIONAL,

qoSFlowsToBeReleased-List		QoSFlows-List-withCause							OPTIONAL,

drbsToBeModifiedList			DRBsToBeModified-List-Modified-SNterminated		OPTIONAL,

dRBsToBeReleased				DRB-List-withCause								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ID id-DefaultDRB-Allowed					CRITICALITY ignore	EXTENSION DefaultDRB-Allowed						PRESENCE optional}|

{ID id-NonGBRResources-Offered				CRITICALITY ignore	EXTENSION NonGBRResources-Offered				PRESENCE optional}|

{ID id-Redundant-UL-NG-U-TNLatUPF			CRITICALITY ignore	EXTENSION UPTransportLayerInformation			PRESENCE optional}|

{ID id-RedundantCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ID id-SecurityIndication					CRITICALITY ignore	EXTENSION SecurityIndication						PRESENCE optional},

...

}

QoSFlowsToBeSetup-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Modified-SNterminated-Item

QoSFlowsToBeSetup-List-Modified-SNterminated-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters							OPTIONAL,

offeredGBRQoSFlowInfo			GBRQoSFlowInfo										OPTIONAL,

qosFlowMappingIndication		QoSFlowMappingIndication 							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics					CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional}|

{ ID id-RedundantQoSFlowIndicator					CRITICALITY ignore	EXTENSION RedundantQoSFlowIndicator	PRESENCE optional}|

{ ID id-ECNMarkingorCongestionInformationRequest	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationRequest	PRESENCE optional},

...

}

DRBsToBeModified-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-Modified-SNterminated-Item

DRBsToBeModified-List-Modified-SNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

mN-DL-SCG-UP-TNLInfo					UPTransportParameters		OPTIONAL,

secondary-MN-DL-SCG-UP-TNLInfo			UPTransportParameters		OPTIONAL,

lCID									LCID						OPTIONAL,

rlc-status								RLC-Status					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Response Info - SN terminated

--

-- **************************************************************

PDUSessionResourceModificationResponseInfo-SNterminated ::= SEQUENCE {

dL-NG-U-TNLatNG-RAN				UPTransportLayerInformation								OPTIONAL,

dRBsToBeSetup					DRBsToBeSetupList-SetupResponse-SNterminated			OPTIONAL,

dataforwardinginfoTarget		DataForwardingInfoFromTargetNGRANnode					OPTIONAL,

dRBsToBeModified				DRBsToBeModifiedList-ModificationResponse-SNterminated	OPTIONAL,

dRBsToBeReleased				DRB-List-withCause										OPTIONAL,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource				OPTIONAL,

qosFlowsNotAdmittedTBAdded		QoSFlows-List-withCause									OPTIONAL,

qosFlowsReleased				QoSFlows-List-withCause									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DRB-IDs-takenintouse			CRITICALITY reject	EXTENSION DRB-List								PRESENCE optional}|

{ ID id-Redundant-DL-NG-U-TNLatNG-RAN	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional}|

{ ID id-SecurityResult					CRITICALITY ignore	EXTENSION SecurityResult						PRESENCE optional},

...

}

DRBsToBeModifiedList-ModificationResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

sN-UL-PDCP-UP-TNLInfo									UPTransportParameters									OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters								OPTIONAL,

qoSFlowsMappedtoDRB-SetupResponse-SNterminated			QoSFlowsMappedtoDRB-SetupResponse-SNterminated		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional}|

{ ID id-secondary-SN-UL-PDCP-UP-TNLInfo				CRITICALITY ignore	EXTENSION UPTransportParameters						PRESENCE optional}|

{ ID id-pdcpDuplicationConfiguration				CRITICALITY ignore	EXTENSION PDCPDuplicationConfiguration				PRESENCE optional}|

{ ID id-duplicationActivation						CRITICALITY ignore	EXTENSION DuplicationActivation						PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModificationInfo-MNterminated ::= SEQUENCE {

pduSessionType					PDUSessionType,

dRBsToBeSetup					DRBsToBeSetupList-Setup-MNterminated						OPTIONAL,

dRBsToBeModified				DRBsToBeModifiedList-Modification-MNterminated				OPTIONAL,

dRBsToBeReleased				DRB-List-withCause											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToBeModifiedList-Modification-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF

DRBsToBeModifiedList-Modification-MNterminated-Item

DRBsToBeModifiedList-Modification-MNterminated-Item ::= SEQUENCE {

drb-ID													DRB-ID,

mN-UL-PDCP-UP-TNLInfo									UPTransportParameters						OPTIONAL,

dRB-QoS													QoSFlowLevelQoSParameters					OPTIONAL,

secondary-MN-UL-PDCP-UP-TNLInfo							UPTransportParameters						OPTIONAL,

uL-Configuration										ULConfiguration								OPTIONAL,

pdcpDuplicationConfiguration							PDCPDuplicationConfiguration 				OPTIONAL,

duplicationActivation									DuplicationActivation						OPTIONAL,

qoSFlowsMappedtoDRB-Setup-MNterminated					QoSFlowsMappedtoDRB-Setup-MNterminated	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-RLCDuplicationInformation					CRITICALITY ignore	EXTENSION RLCDuplicationInformation	PRESENCE optional}|

{ ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest	PRESENCE optional},

...

}

-- **************************************************************

--

###### -- PDU Session Resource Modification Response Info - MN terminated

--

-- **************************************************************

PDUSessionResourceModificationResponseInfo-MNterminated ::= SEQUENCE {

dRBsAdmittedList						DRBsAdmittedList-ModificationResponse-MNterminated,

dRBsReleasedList						DRB-List																		OPTIONAL,

dRBsNotAdmittedSetupModifyList			DRB-List-withCause																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs} } 	OPTIONAL,

...

}

PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsAdmittedList-ModificationResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-ModificationResponse-MNterminated-Item

DRBsAdmittedList-ModificationResponse-MNterminated-Item ::= SEQUENCE {

drb-ID									DRB-ID,

sN-DL-SCG-UP-TNLInfo					UPTransportParameters				OPTIONAL,

secondary-SN-DL-SCG-UP-TNLInfo			UPTransportParameters				OPTIONAL,

lCID									LCID								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs} } 	OPTIONAL,

...

}

DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Additional-PDCP-Duplication-TNL-List		CRITICALITY ignore	EXTENSION Additional-PDCP-Duplication-TNL-List	PRESENCE optional}|

{ ID id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated	CRITICALITY ignore	EXTENSION	QoSFlowsMappedtoDRB-SetupResponse-MNterminated	PRESENCE optional}|

{ ID id-ECNMarkingorCongestionInformationReportingStatus	CRITICALITY ignore	EXTENSION	ECNMarkingorCongestionInformationReportingStatus	PRESENCE optional},

...

}

------ Unchanged part skipped ------

NEXT CHANGE

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

------ Unchanged part skipped ------

id-ECNMarkingorCongestionInformationReportingRequest												ProtocolIE-ID ::= 450

id-PDUSetbasedHandlingIndicator																		ProtocolIE-ID ::= 451

id-TAISliceUnavailableCellList																		ProtocolIE-ID ::= 452

id-MobileIAB-AuthorizationStatus																	ProtocolIE-ID ::= 453

id-MIAB-MT-BAP-Address																				ProtocolIE-ID ::= 454

id-MobileIABCell																					ProtocolIE-ID ::= 455

id-sk-Counter																						ProtocolIE-ID ::= 456

id-Source-M-NG-RANnodeID																			ProtocolIE-ID ::= 457

id-S-CPAC-CompleteConfig-Indicator																	ProtocolIE-ID ::= 458

id-SourceSN-to-TargetSN-QMCInfo																		ProtocolIE-ID ::= 459

id-RegistrationRequestForDataCollection																ProtocolIE-ID ::= 460

id-ReportCharacteristicsForDataCollection															ProtocolIE-ID ::= 461

id-ReportingPeriodicityForDataCollection															ProtocolIE-ID ::= 462

id-NodeAssociatedInfoResult																			ProtocolIE-ID ::= 463

id-SLPositioning-Ranging-Services-Info																ProtocolIE-ID ::= 464

id-ECNMarkingorCongestionInformationReportingStatus													ProtocolIE-ID ::= xxx

END

-- ASN1STOP

CHANGES END