3GPP TSG-RAN WG3 Meeting #123bis	R3-241832

Changsha, China, 15 – 19 April 2024

Agenda item:	12.3

Source:	Nokia, TMO US, AT&T, Verizon Wireless, British Telecom, NTT Docomo, KDDI

Title:	[TP for 38.799] Solution for 5G Femto Architecture

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

A RAN architecture solution has been proposed in tdoc [2] to be captured in the TR 38.799.

The corresponding TP for TR 38.799 is available below.

# 2	References

- RP-234041, Study Item on Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-241831, [TP for TR 38.799] Solution for 5G Femto Architecture

# 3	TP for TR 38.799

### 6.2	Architecture

Figure 6.2-1 shows a logical architecture for the 5G Femto that has a set of NG interfaces to connect the 5G Femto to the 5GC.

Figure 6.2-1: NG-RAN 5G femto Logical Architecture

The NG-RAN architecture may deploy a 5G Femto Gateway (5G Femto GW) to allow the NG interface between the 5G Femto and the 5GC to support a large number of 5G Femtos in a scalable manner. The 5G Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface.

The NG interface is defined as the interface:

-	Between the 5G Femto GW and the 5GC;

-	Between the 5G Femto and the 5G Femto GW;

-	Between the 5G Femto and the 5GC;

-	Between the gNB and the 5GC.

The 5G Femto GW appears to the AMF as a gNB. The 5G Femto GW appears to the 5G Femto as an AMF. The NG interface between the 5G Femto and the 5GC is the same regardless of whether the 5G Femto is connected to the 5GC via a 5G Femto GW or not.