3GPP TSG RAN3 meeting #123bis    	         	                  R3-241967

15th Apr – 19 Apr 2024  Changsha

Agenda Item:	10.3.1

Source:	China Unicom

Title:	Discussion on SON/MDT enhancements for NTN mobility

Document for:	Discussion

Introduction

The Work Item on SON/MDT enhancement has been agreed in RAN#102 (RP-234038) with the following objectives:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

This paper discusses the SON/MDT enhancements for NTN mobility.

Discussion

Condition Handover Optimization for NTN network

Time-based CHO Optimization for NTN network

NTN supports the time-based trigger condition upon which UE may execute CHO to a candidate cell. The figure below depicts the basic conditional handover scenario where neither the AMF nor the UPF changes:

The CHO condition is configured from the gNB to UE via the RRCReconfiguration message, and the preparation and execution phase of the conditional handover procedure is performed by the UE without involvement of the 5GC.

For NTN network, the time-based CHO is supported, and the CHO configuration for time-based CHO is configured like below:

condEventT1-r17                  SEQUENCE {

t1-Threshold-r17                 INTEGER (0..549755813887),

duration-r17                     INTEGER (1..6000)

},

The UE should satisfy the time-based CHO condition, the time-based trigger condition is always configured together with one of the measurement-based trigger conditions.

For the time-based CHO, the UE should execute the handover during the t1-Threshold and the  t1-Threshold + duration, if this condition is not configured appropriately, the UE will execute the handover too early or too late, or even handover to the wrong cells because the handover opportunity is not appropriate. In this case, the time-based CHO configuration may need to be optimized. The handover too early or too late, or handover to the wrong cells may be detected by the radio link failure procedure, the UE can record the information about the CHO when the RLF happen, and the UE can report the CHO information to the gNB for CHO configuration optimization.

Proposal 1: Time-based CHO configuration need to be optimized when the RLF happened after the CHO.

Proposal 2: The UE can record and report the time-based CHO related information to the gNB when the radio link failure happen.

The time-based CHO will configure the t1-Threshold and the duration for the handover, and the UE will execute the handover during the configured time duration. Even if the handover is successful, and there is no radio link failure happened after the CHO, the CHO may also need to be optimized. The CHO may happen at the beginning of the t1-Threshold, or nearly at the end of the t1-Threshold + duration, therefore the gNB also needs to get the CHO related information to further optimize the CHO configuration to avoid the potential handover failure. The condition for the UE to report the CHO related information can be configured by the gNB.

Proposal 3: The gNB can configure the CHO successful report condition for the UE to report the time-based CHO related information.

Proposal 4: The UE can record and report the time-based CHO related information to the gNB when the condition for the UE to report the time-based CHO related information if satisfied.

Location-based CHO Optimization for NTN network

For NTN network, the location-based CHO is supported, and the CHO configuration for location-based CHO is configured like below:

condEventD1-r17                  SEQUENCE {

distanceThreshFromReference1-r17 INTEGER(0.. 65525),

distanceThreshFromReference2-r17 INTEGER(0.. 65525),

referenceLocation1-r17           ReferenceLocation-r17,

referenceLocation2-r17           ReferenceLocation-r17,

hysteresisLocation-r17           HysteresisLocation-r17,

timeToTrigger-r17                TimeToTrigger

},

For the location-based CHO, the UE should measure the distance to the reference location of the serving cell and the target cell, and it will execute the handover after the distance threshold for the serving cell and the target cell are satisfied. The UE will execute the handover too early or too late, or even handover to the wrong cells because the handover opportunity is not appropriate. In this case, the location-based CHO configuration may need to be optimized. The handover too early or too late, or handover to the wrong cells may be detected by the radio link failure procedure, the UE can record the information about the CHO when the RLF happen, and the UE can report the CHO information to the gNB for CHO configuration optimization.

Proposal 5: Location-based CHO configuration need to be optimized when the RLF happened after the CHO.

Proposal 6: The UE can record and report the location-based CHO related information to the gNB when the radio link failure happen.

MDT Measurement Enhancement for NTN Network

The UE in NTN network can support mobility in RRC IDLE/INACTIVE states, and can support CHO in RRC CONNECTED state.

For UE in RRC IDLE/INACTIVE states, the network will configure the reference location and the moving reference location to UE via SIB19. UE will execute the cell re-selection according to the reference location and the moving reference location configured by the network.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

movingReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL,       -- Need R

satSwitchWithReSync-r18                  SatSwitchWithReSync-r18                         OPTIONAL        -- Need R

]]

}

If distance Thresh and reference Location are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN quasi-Earth-fixed system and has obtained its location information, the UE will check the distance between UE and the serving cell reference location, if the reference Location is shorter than distance Thresh, the UE may not perform intra-frequency measurements; otherwise, the UE shall perform intra-frequency measurements.

If distance Thresh and moving Reference Location are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving system and has obtained its location information, the UE will check the distance between UE's location and the serving cell reference location determined based on moving Reference Location is shorter than distance Thresh, the UE may not perform intra-frequency measurements; otherwise, the UE shall perform intra-frequency measurements.

The UE will execute the cell re-selection after the measurement, and the reference location configuration is critical for the UE cell re-selection.

For UE in RRC CONNECTED state, the network can configure the location based CHO for UE, and the UE will execute the conditional handover when the condition configured by the network is satisfied.

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

......

eventD1-r17                                 SEQUENCE {

distanceThreshFromReference1-r17            INTEGER(1.. 65525),

distanceThreshFromReference2-r17            INTEGER(1.. 65525),

referenceLocation1-r17                      ReferenceLocation-r17,

referenceLocation2-r17                      ReferenceLocation-r17,

reportOnLeave-r17                           BOOLEAN,

hysteresisLocation-r17                      HysteresisLocation-r17,

timeToTrigger-r17                           TimeToTrigger

}

......

}}

In the location based CHO configuration, the referenceLocation1 is associated to serving cell and referenceLocation2 is associated to candidate target cell. When the distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 or a moving reference location determined by the UE based on referenceLocation1 or referenceLocation2 satisfy the configured threshold, the UE will execute the conditional handover.

Observation 1: The reference point is used to determine the cell re-selection measurement and conditional handover during UE mobility in RRC IDLE/INACTIVE states and RRC CONNECTED state.

The reference location configuration is critical for the UE to execute cell re-selection and handover, and how to configure the reference location and the distance threshold for UE and reference location relies on the network deployment for NTN network. When the reference location and the distance threshold are not configured properly, there will be coverage hole or too much overlap coverage, which will lead to UE handover fail or unnecessary measurements for UE. The reference location and distance threshold configuration need to be optimized according to the measurements based on the reference location.

Proposal 7: The reference location and distance threshold configuration need to be optimized for UE cell re-selection and CHO.

Proposal 8: The reference location related MDT measurement is needed for network optimization.

Conclusion

Proposal 1: Time-based CHO configuration need to be optimized when the RLF happened after the CHO.

Proposal 2: The UE can record and report the time-based CHO related information to the gNB when the radio link failure happen.

Proposal 3: The gNB can configure the CHO successful report condition for the UE to report the time-based CHO related information.

Proposal 4: The UE can record and report the time-based CHO related information to the gNB when the condition for the UE to report the time-based CHO related information if satisfied.

Proposal 5: Location-based CHO configuration need to be optimized when the RLF happened after the CHO.

Proposal 6: The UE can record and report the location-based CHO related information to the gNB when the radio link failure happen.

Observation 1: The reference point is used to determine the cell re-selection measurement and conditional handover during UE mobility in RRC IDLE/INACTIVE states and RRC CONNECTED state.

Proposal 7: The reference location and distance threshold configuration need to be optimized for UE cell re-selection and CHO.

Proposal 8: The reference location related MDT measurement is needed for network optimization.