3GPP TSG-RAN WG3 Meeting #123bis														R3-242145

Changsha, China, from April 15 to April 19 2024

Agenda Item:	11.1

Source:	ZTE

Title:	SoD on CB: # AIRAN1_Slicing

Document for:	Approval

1	Introduction

This SoD is to discuss the following CB:

CB: # AIRAN1_Slicing

- Work on TP

- Discuss the open issues above and relevant ones in the papers

(moderator - ZTE)

Summary of offline disc R3-242145

2	For chairman’s Notes

Proposal 1: Consider the RAN input information for AI/ML model inference in RAN during Rel-19. Input information from CN if available may be studied.

Proposal 2: NG-RAN may need following information as input data for AI/ML-based network slicing:

-	From local node:

-	Legacy predicted UE trajectory

-	From UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

Proposal 3:AI/ML-based network slicing model in NG-RAN node can generate following information as output:

- Resource management within RRM policy (Internal output)

- Slice aware mobility decisions (Internal output)

Proposal 4: Following feedback can be considered to be collected from NG-RAN nodes:

-	Legacy UE performance feedback for those UEs handed over from the source NG-RAN node

Agree R3-242160 TP for TR38.743, which is revision of R3-241868.

Follwing information was initially discussed, but it can not reach consenus:

- Current/Predicted UE traffic

- SLA fulfilment prediction exchanged with OAM/5GC

- Measured/Predicted SLArequirement fulfilment prediction between NG-RAN nodes

Other information can be contribution-driven.

2	Discussion

2.1 Interaction with CN

[1][5][9]shared the views in their contribution the coordination between CN and RAN for AI/ML assisted Network Slicing is needed.

[1] proposed that	predicted AMF capacity can be considered as AI/ML related information provided from CN to NG-RAN to assist AI/ML network slicing.

For legacy network slicing, as specified in TS 38.300, NG-RAN is able to select appropriate AMF for initial attachment by default or by NSAAI provided by UE. This means that the NG-RAN node could select the better AMF to guarantee subsequent service quality if more helpful information is provided.

====================== TS 38.300 ======================

RAN selection of CN entity

-	For initial attach, the UE may provide NSSAI to support the selection of an AMF. If available, NG-RAN uses this information for routing the initial NAS to an AMF. If the NG-RAN is unable to select an AMF using this information or the UE does not provide any such information the NG-RAN sends the NAS signalling to one of the default AMFs.

====================== TS 38.300 ======================

[1][5][9] proposed that predicted slice-level resource status can be exchanged between CN and RAN node.

Summary:

Discuss the benefits of Core Network providing predicted information to NG-RAN nodes.

- Predicted AMF capacity

- Predicted Slice-level resource status

Consider the RAN input information for AI/ML model inference in RAN during Rel-19. Input information from CN if available may be studied.

2.2 Required information

Following is agreed during the online meeting:

Current/predicted slice-level resource status, and current/predicted slice available capacity can be transferred between NG-RAN nodes to assist AI/ML network slicing.

Here, I try to list required information for AI/ML assisted Network Slicing mentioned in all papers.

First, whether following information which was agreed during Rel-18 information can also be applied to support of AI/ML assisted network slicing:

Following may be easily agreed to be captured into TR.

Input information:

To predict the optimized network slicing decisions, NG-RAN may need following information as input data for AI/ML-based network slicing:

From local node:

-	Current/Predicted resource status per slice (agreed online)

-	Current/Predicted slice available capacity (agreed online)

-	Legacy predicted UE trajectory

From neighbouring NG-RAN nodes:

-	Current/Predicted resource status per slice (agreed online)

-	Current/Predicted slice available capacity (agreed online)

From the UE:

-	Legacy UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

Output information:

AI/ML-based network slicing model in NG-RAN node can generate following information as output:

- Predicted resource status per slice (agreed online)

- Predicted slice available capacity (agreed online)

- Resource management within RRM policy (internal output)

- Slice aware mobility decisions (internal output)

Feedback information:

To optimize the performance of AI/ML-based network slicing model, following feedback can be considered to be collected from NG-RAN nodes:

- Resource Status per slice level updates from target NG-RAN node (agreed online)

- Slice Available Capacity updates from target NG-RAN node (agreed online)

- Legacy UE performance feedback for those UEs handed over from the source NG-RAN node

Following may need more time to discuss:

Next, try to discuss something new proposed for AI/ML assisted network slicing:

- Current/Predicted UE traffic

- SLA fulfillment prediction:

- SLA fulfilment prediction exchanged with OAM/5GC

- Measured/Predicted SLA requirement fulfilment prediction between NG-RAN nodes

What’s the use cases? For UE.

- Predicted UE slice level resource status information

- Predicted slice for UE for mobility scenario

- Predicted slice status, e.g., available, unavailable, overloaded

Feedback:

- Measured/Predicted Energy Cost per slice per UE/ per slice per cell

- UE Performance (total per UE, per PDU session, per QoS flow)

- Slice-level UE performance

Summary:

To be continued:

Information may be used for AI/ML based network slicing:

- Current/Predicted UE traffic

- SLA fulfilment prediction exchanged with OAM/5GC

- Measured/Predicted SLArequirement fulfilment prediction between NG-RAN nodes

- …

3	Reference

[1] R3-241867 Discussion on AI/ML assisted Network Slicing (ZTE)	discussion

[2] R3-241868 (TP to TR38.743) AI/ML assisted Network Slicing (ZTE)	other

[3] R3-241589 Considerations on AI/ML based network slicing (NEC)	discussion

[4] R3-241590 TP to TR38.743 for AI/ML-based Network Slicing (NEC)	other

[5] R3-241611 Discussion on NG-RAN AI/ML for Network Slicing (Qualcomm Incorporated)	discussion

[6] R3-241729 (TP to TR 38.743) AI/ML enabled slicing (Samsung)	other

[7] R3-241791 Discussion on AIML based network slicing (Lenovo)	discussion

[8] R3-241792 TP to 38.743 for AIML based network slicing (Lenovo)	other

[9] R3-241822 Discussion on support of AI/ML enabled slicing (CATT)	discussion

[10] R3-241823 (TP for TR 38.743) Support of AI/ML enabled slicing (CATT)	other

[11] R3-241843 (TP for TR 38.743) AI/ML Network Slicing for Rel-19 (Nokia, Telecom Italia)	other

[12] R3-241941 Discussion on AI-based network slice (CMCC)	discussion

[13] R3-241986 Discussion on RAN AI/ML for Network Slicing (Huawei)	discussion

[14] R3-242065 TP on AIML for Network Slicing (Ericsson, Charter, InterDigital, Telecom Italia)	discussion

[15] R3-242066 AIML for Network Slicing (Ericsson, Charter, InterDigital, Telecom Italia)	discussion