3GPP TSG-RAN WG3 Meeting #123bis	R3-242199

Changsha, China, April 15th – 19th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0389 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 LTM correction for LTM with gNB-CU-UP change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Google, Ericsson, Nokia, Huawei,CATT, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 38.401 Section 8.2.1.6 LTM with gNB-CU-UP change shows the procedure used for LTM with the change of gNB-CU-UP within a gNB. The Step 10-11 is used to retrieve the PDCP UL/DL status and to exchange TNL address information to start data forwarding from the source CU-UP. They are shown as if always executed in case of LTM with gNB-CU-UP change, however, they happen only when there is RLC-AM bearers continued in the target CU-UP or data forwarding (optional) happens between them during cell switch. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the Step 10-11 from mandatory to optional in Figure 8.2.1.6-1. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on LTM with gNB-CU-UP change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary Bearer Context Modification procedure may be executed toward the source CU-UP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Updated based on RAN3-123bis discussions with more co-signers. |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.2.1.6	LTM with gNB-CU-UP change

Figure 8.2.1.6-1 shows the procedure used for LTM with the change of gNB-CU-UP within a gNB.

Figure 8.2.1.6-1 LTM with the change of gNB-CU-UP

0.	The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1.	The gNB-CU-CP decides to initiate LTM configuration.

2.	The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the target gNB-CU-UP.

3.	The target gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the target gNB-CU-UP.

4.	LTM configuration procedures are performed between gNB-CU and candidate gNB-DUs, and between gNB-CU and source gNB-DU as specified from step 3 to step 8 in section 8.2.1.5.

5 - 6.	The gNB-CU-CP sends the RRC Reconfiguration message to the UE.

7.	The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a candidate target cell.

8.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU-CP with the selected target cell ID.

9.	The gNB-CU-CP forwards the selected target cell ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

10-11.	The gNB-CU-CP may perform the Bearer Context Modification procedure to retrieve the PDCP UL/DL status and to exchange the TNL address information for data forwarding for the bearers to the source gNB-CU-UP.

12-13.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UL/DL status to the target gNB-CU-UP.

14.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

15.	The target gNB-DU detects the UE in the target cell.

16.	The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

17-19.	Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

20-21.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-CU-UP.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////