3GPP TSG-RAN WG3 Meeting #123	R3-240840

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	9.1.12.1

Source:	Huawei

Title:	Summary of offline discussions on CB: # 13_R18ES

Document for:	Discussion and approval

# Introduction

This contribution discusses the following CB:

CB: # 13_R18ES

- Check details in R3-240839

- Check other corrections

(moderator - HW)

Summary of offline disc R3-240840

# 2	For the Chairman’s Notes

Proposal:

# 3 Discussion

## 3.1 Inter-node beam activation

| R3-240407 | Correction of SSBs activation and deactivation for network energy saving (Huawei, ZTE, CATT, Qualcomm Incorporated) | CR1306r, TS 38.473 v18.0.0, Rel-18, Cat. F NBC CR E///: Support it - Add E///, Nok as co-source - Describe abnormal cases Rev in R3-240839 |
| --- | --- | --- |

Proposal: To be reviewed in the draft folder.

| R3-240130 | Beam deactivation for NES (NEC) | discussion |
| --- | --- | --- |
| R3-240131 | Beam deactivation for NES (NEC) | CR1256r, TS 38.473 v18.0.0, Rel-18, Cat. F |

- Add Beam Deactivation Assistance Information in gNB-CU Configuration Update procedure.

#### 9.3.1.xxx Beam Deactivation Assistance Information This IE provides assistance information for the beam deactivation actions for gNB-DU for network energy saving. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Load threshold; O; INTEGER (0..100); Indicates the traffic load in percent that DU to deactivate some SSB beams.. Time to trigger beam deactivation; O; ENUMERATED (ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120, ...); Indicates the duration during which load threshold should be satisfied in order to trigger deactivation.. Deactivation hysteresis; O; INTEGER (0..30); This parameter is used within the entry and leave condition of an event triggered beam deactivation condition. The actual value is field value * 1 ms.. Deactivation timer; O; INTEGER (0..5); Indicates the duration during which beam to stay in deactivated. The actual value is field value * 1 minutes..

Proposal:

| R3-240672 | Correction on F1AP SSB Activation in proceudal text. (Ericsson) | CR1350r, TS 38.473 v18.0.0, Rel-18, Cat. F |
| --- | --- | --- |

- In case that the SSB beams requested to be activated by gNB-CU are already activated, the gNB-DU consider and include the SSB beams in the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

- If the requested SSB beams in the SSBs within the cell to be Activated List IE are already activated, the gNB-DU includes the SSB beams in the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

Proposal:  over specification? ?

| R3-240742 | Correction on XnAP for NES (ZTE, China Unicom) | CR1224r, TS 38.423 v18.0.0, Rel-18, Cat. F |
| --- | --- | --- |

- To change the description of ‘NR Cells and SSBs IE’ in a more accurate way, e.g., CHOICE Served Cells To Activate IE set to ‘NR Cells and SSBs’.

- If the CHOICE Served Cells To Activate IE is set to ‘NR Cells and SSBs’ in the CELL ACTIVATION REQUEST message

Proposal:  Captured by the XnAP spec rapporteur if needed? ?

| R3-240229 | Correction on beam activation for network energy saving in XnAP (Samsung) | CR1142r, TS 38.423 v18.0.0, Rel-18, Cat. F |
| --- | --- | --- |

- Remove the text “Otherwise, the NG-RAN node2 shall, if supported, activate all the SSB beams in the cell.”

- Change the presence of SSBs to be Activated List IE and SSBs Activated List IE from optional to mandatory.

Proposal:  CR is noted.

| R3-240671 | Correction on ASN.1 code related to Network Energy Saving (Ericsson) | CR1349r, TS 38.473 v18.0.0, Rel-18, Cat. F |
| --- | --- | --- |

Proposal:  CR is merged.

Stage 2 CRs

| R3-240743 | Correction on 38.470 for NES (ZTE, Huawei, China Unicom) | CR0136r, TS 38.470 v18.0.0, Rel-18, Cat. F |
| --- | --- | --- |

- With the gNB-CU Configuration Update function, energy saving with cell activation/deactivation and beam activation can be supported as defined in TS 38.300 [8]. The gNB-CU Configuration Update function also allows to inform the allowed cells for the gNB-DU to deactivate the SSB beams. The gNB-DU Configuration Update function allows to inform the deactivated SSB beams due to network energy saving. The gNB-DU Configuration Update function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

| R3-240228 | Correction on beam activation/deactivation for network energy saving (Samsung, Ericsson) | CR0126r, TS 38.470 v18.0.0, Rel-18, Cat. F Ok |
| --- | --- | --- |

- The gNB-CU Configuration Update function may activate SSB beams of cells. The gNB-CU Configuration Update function may indicate the cells where the gNB-DU is allowed to deactivate the SSB beams.

Proposal:

## 3.2 Cell DTX/DRX

| R3-240184 | Leftover issues on cell DTXDRX for Rel-18 NES (CATT) | discussion |
| --- | --- | --- |
| R3-240185 | Cell DTXDRX activation NR_38.473(Rel-18) (CATT) | CR1270r, TS 38.473 v18.0.0, Rel-18, Cat. F |

The overall procedure of cell DTX/DRX activation or deactivation: Step 1: The gNB-DU determines to use or stop using cell DTX/DRX for a specific cell - The gNB-DU sends the requirement of cell DTX/DRX activation or deactivation to the gNB-CU via GNB-DU CONFIGURATION UPDATE message. The DU further indicates the cell DTX/DRX type (i.e., only DTX, only DRX or both DTX and DRX) required to be used. Step 2: If using cell DTXDRX, the gNB-CU carries out necessary handovers or remove serving cell for UE - If the requirement of cell DTX/DRX activation is received from the gNB-DU, the gNB-CU can carry out the legacy handovers, configure NES CHO or releasing the NES cell to the unsuitable UEs. - Then the gNB-CU sends an indication to the gNB-DU for allowing cell DTXDRX activation via GNB-CU CONFIGURATION UPDATE message. Step 3: The gNB-DU activates or deactivates cell DTX/DRX When the gNB-DU receives the indication from the gNB-CU for allowing activation of cell DTXDRX, it configures/activates cell DTX/DRX to UEs and send L1 signal to trigger NES CHO if necessary.

Proposal:

## 3.3 NES CHO

| R3-240546 | (TPs to 38.473 and 38.401) NES CHO awareness in source gNB (Qualcomm Technologies Int) | Discussion E///: p1 has not been discussed in last meeting, no need to enhance as p2 SS, HW: There is no need to inform the source cell about NES CHO, P2 is not necessary NEC: Has different understanding on p1 ZTE: Agree with majority view |
| --- | --- | --- |

- Proposal 1: Send LS to RAN2 to enhance inter-node RRC signalling so that candidate gNB(s) can inform source gNB whether a UE is configured with NES specific CHO event.

- Proposal 2: Source gNB-CU can indicate to source gNB-DU that the UE is configured with NES-specific CHO event via a NES CHO indicator in F1AP UE CONTEXT MODIFICATION REQUEST.

Discussion:  No consensus??

## 3.4 Others

| Company | Comment |
| --- | --- |

Moderator summary:

# 4	References