3GPP TSG-RAN WG3 Meeting #123	R3-240950

# Athens, Greece, 26th February– 1st March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1054 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for QoE measurement activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R17, RAN3 agrees to include QMC Configuration Information IE in the following messages over NGAP: - INITIAL CONTEXT SETUP REQUEST - UE CONTEXT MODIFICATION REQUEST - HANDOVER REQUEST - HANDOVER REQUIRED The QMC Configuration Information IE is included in INITIAL CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message, and the RAN behaviours are also described that “If the QMC Configuration Information IE is included in the INITIAL CONTEXT SETUP REQUEST/UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, use it for QoE management, as described in TS 38.300” For R18, for HANDOVER REQUEST message and HANDOVER REQUIRED message, the QMC Configuration Information IE is included in Source NG-RAN Node to Target NG-RAN Node Transparent Container. However, there is no NG-RAN behaviour description related to QMC Configuration Information IE in HANDOVER REQUEST message and HANDOVER REQUIRED message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) as it only adds corresponding procedure text for the QMC Configuration Information IE in HANDOVER REQUEST message and HANDOVER REQUIRED message that is optional presence. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the RAN behaviour descriptions if QMC Configuration Information IE is included in Source NG-RAN Node to Target NG-RAN Node Transparent Container in HANDOVER REQUEST message and HANDOVER REQUIRED message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No RAN behaviour descriptions if QMC Configuration Information IE is included in Source NG-RAN Node to Target NG-RAN Node Transparent Container in HANDOVER REQUEST message and HANDOVER REQUIRED message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: R3-240127 Rev 1: The added text description in Handover Required message is removed. |  |  |  |  |  |  |  |  |

Start of change part

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

If the HANDOVER REQUEST message contains within the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE the Time Based Handover Information IE, the target NG-RAN node may use this information to allocate necessary resources for the incoming handover.

If the Candidate Relay UE Information List IE is included in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE within the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to configure the path switch to indirect path as specified in TS 38.300 [8].

Upon reception of the QMC Configuration Information IE, which is included within the Source to Target Transparent Container IE of the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it for QoE management, as described in TS 38.300[8].

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

End of change part