3GPP TSG-RAN WG3 Meeting #123	R3-240469

Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0349 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 update for LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The LTM feature is introduced in Rel-18. However, the following issues and ambiguities still exist in stage 2. - In the figure of intra-DU LTM, the message name after step 12 is not aligned to the one used in inter-DU LTM. - Request PRACH resources for early RACH is optional, however, the existing text in step 3 seems mandatory. - The CSI report Configuration of candidate cells do not need to transfer to the source DU and the other candidate DUs. - In step 5 and step 7 in inter-DU LTM, the description updated CSI resource configuration is missing. Similar issue in intra-DU LTM. - In NOTE2 in intra-DU LTM, the TCI state information, RACH configuration, and the LTM configuration IDs of the candidate cells are not needed to send to the gNB-DU anymore. - In step 7 in inter-DU LTM , there is no need to send the lower layer part of the reference configuration to the candidate gNB-DU(s), as it has been provided in step 3. - Some typos and errors exit in the figure and text in LTM with gNB-CU-UP change section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For section 8.2.1.4 Intra-gNB-DU LTM: - In the figure, after step 12, change “Inter-cell Mobility Execution Decision” to “LTM Cell Switch Decision” to align to the inter-gNB-DU LTM. - In step 3, add “may” in sentence “The gNB-CU requests PRACH resources from the gNB-DU”, and add “The gNB-CU may xxx, or provide the lower layer reference configuration to the candidate gNB-DU” in the end of next sentence. - In step 5, remove “and the gNB-CU may send the CSI report configuration for all the accepted target candidate cells”, and add a new sentence “The gNB-CU may send the updated CSI resource configuration to the source gNB-DU.” - In step 6, add “updated” in between “the” and “CSI report configuration” - In NOTE 2, remove “candidate”, change “the CSI resource configuration” to “the updated CSI resource configuration”, remove “TCI state information, RACH configuration, and the LTM configuration IDs of the candidate cells.” - In step 15, add ”in the target cell” - In step 16, remove “target” For section 8.2.1.5 Inter-gNB-DU LTM: - In step 3, add sentence “The gNB-CU may xxx, or provide the lower layer reference configuration to the candidate gNB-DU.” - In step 5, remove “collected CSI report configuration”, and add “the LTM configuration IDs” and sentence “The gNB-CU may send the update CSI resource configuration to the source gNB-DU.” - In step 6, update “containing the CSI report configuration” to “containing the updated CSI report configuration of the source cell” - In step 7, remove “CSI report configuration”, and “The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s)” - Remove NOTE 2. - In step 8, remove “The candidate gNB-DU may also respond”, and add “(e.g., …)” - In step 13, add “to the target candidate cells” For 8.2.1.6 LTM with gNB-CU-UP change: - In the Figure, update step 8 to terminate the message in target gNB-DU - In step 9-10, add “to the source gNB-CU-UP” - In step 19-20, correct typo “gNB-DU” to “gNB-CU-UP” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors and ambiguities exit in state 2 text for LTM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.4, 8.2.1.5, 8.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/**********************Start of changes****************************************/

8.2.1.4	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.4-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.4-1: Intra-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing measurements of neighbouring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU for each candidate cell, containing one target candidate cell ID, the LTM configuration ID of the candidate cell, and LTM configuration ID mapping list, the CSI resource configuration. The gNB-CU may request PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the candidate gNB-DU.

4.	If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations (e.g., TCI state configuration, RACH configuration, and the CSI report configuration) for the accepted target candidate cell.

NOTE 1:	Steps 3 and 4 may be initiated multiple times for LTM candidate cell preparation of multiple cells including the source cell.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU. The message may include the updated CSI resource configuration.

6.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

NOTE 2:	In case of subsequent LTM, the CU-initiated UE Context Modification procedure may be invoked per each candidate cell to transfer to the gNB-DU the updated CSI  resource configuration.

7.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

8.	The gNB-DU forwards the received RRCReconfiguration message to the UE.

9.	The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

10.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

11.	Early synchronization to the target candidate cell(s) may be performed as specified in TS 38.300 [2].

12.	The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

13.	The gNB-DU sends the Cell Switch Command to the UE.

14.	The gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID.

15.	The gNB-DU detects the UE access in the target cell as specified in TS 38.300 [2].

16.	The gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

17.	The UE sends an RRCReconfigurationComplete message to the gNB-DU.

18.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

19.	The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

20.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

/**********************Next change****************************************/

8.2.1.5	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.5-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.5-1: Inter-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one target candidate cell ID, the LTM configuration ID of the candidate cell, LTM configuration ID mapping list, and the CSI resource configuration. The gNB-CU may indicate the source gNB-DU ID List, and requests PRACH resources from the candidate gNB-DU. The gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the candidate gNB-DU.

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations (e.g., TCI state configuration, RACH configuration, and the CSI report configuration) for the accepted target candidate cell.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the RACH configuration and the TCI state configuration and the LTM configuration IDs for the accepted target candidate cell(s) in other gNB-DU(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells, . The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration(e.g., the updated CSI report configuration).

NOTE 2: Step 7 may be also invoked by the gNB-CU after step 19, or step 22 by implementation.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	Early synchronization to the target candidate cell(s) may be performed as specified in TS 38.300 [2].

14.	The candidate gNB-DU sends the TA value, the associated CFRA resource information, the candidate cell ID and the source gNB-DU ID to the source gNB-DU in the DU-CU TA INFORMATION TRANSFER message to the gNB-CU.

15.	The gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

16.	The UE sends the L1 measurement result to the source gNB-DU.

17.	The source gNB-DU decides to execute LTM to a candidate target cell.

18.	The source gNB-DU sends the Cell Switch command to the UE.

19.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE, for which the message includes the target cell ID and the TCI state ID.

20.	The gNB-CU forwards the target cell ID and the TCI state ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

21.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

22.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

23.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

24.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

25.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

26.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

/**********************Next change****************************************/

8.2.1.6	LTM with gNB-CU-UP change

Figure 8.2.1.6-1 shows the procedure used for LTM with the change of gNB-CU-UP within a gNB.

Figure 8.2.1.6-1 LTM with the change of gNB-CU-UP

0.	The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1.	The gNB-CU-CP decides to initiate LTM configuration.

2.	The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the target gNB-CU-UP.

3.	The target gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the target gNB-CU-UP.

4.	LTM configuration procedures are performed between gNB-CU and candidate gNB-DUs, and between gNB-CU and source gNB-DU as specified from step 3 to step 8 in section 8.2.1.5.

5 - 6.	The gNB-CU-CP sends the RRC Reconfiguration message to the UE.

7.	The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a candidate target cell.

8.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU-CP with the selected target cell ID.

9.	The gNB-CU-CP forwards the selected target cell ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

10-11.	The gNB-CU-CP performs the Bearer Context Modification procedure to retrieve the PDCP UL/DL status and to exchange the TNL address information for data forwarding for the bearers to the source gNB-CU-UP.

12-13.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UL/DL status to the target gNB-CU-UP.

14.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

15.	The target gNB-DU detects the UE in the target cell.

16.	The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

17-19.	Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

20-21.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-CU-UP.

/**********************End of changes****************************************/