3GPP TSG-RAN WG3 Meeting #123	R3-240858

Athens, Greece, 26 February - 1 March, 2024

Agenda item:	9.1.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Summary of discussions on CB: # SONMDT1_NPN

Document for:	Approval

1	Introduction

This paper provides summary of discussions at RAN3#123 on:

CB: # SONMDT1_NPN

- Check whether SNPN Cell Based MDT is needed or not？

- Check RRC support and LS to SA5 if agreeable

(moderator - Nok)

2	For the Chairman’s Notes

[Draft] Reply LS to SA5 on MDT for NPN in R3-241029 agreed

3	Discussion

RRC support:

The moderator observes that the following IEs for SNPN are supported as part of logged MDT configuration in TS 38.331 v18.0.0:

SNPN-ConfigList-r18 ::=          CHOICE {

snpn-ConfigCellIdList-r18        SNPN-ConfigCellIdList-r18,

snpn-ConfigTAIList-r18           SNPN-ConfigTAIList-r18,

snpn-ConfigIDList-r18            SNPN-ConfigIDList-r18

}

SNPN-ConfigCellIdList-r18 ::=    SEQUENCE (SIZE (1..maxSNPN-ConfigCellId-r18)) OF SNPN-ConfigCellId-r18

SNPN-ConfigCellId-r18 ::=        SEQUENCE {

cgi-Identity-r18                 CGI-Info-Logging-r16,

nid-IdentityList-r18             SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16

}

SNPN-ConfigTAIList-r18 ::=       SEQUENCE (SIZE (1..maxSNPN-ConfigTAI-r18)) OF SNPN-ConfigTAI-r18

SNPN-ConfigTAI-r18 ::=           SEQUENCE {

tai-Identity-r18                 TrackingAreaIdentity-r16,

nid-IdentityList-r18             SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16

}

SNPN-ConfigIDList-r18 ::=        SEQUENCE (SIZE (1..maxSNPN-ConfigID-r18)) OF SNPN-ConfigID-r18

SNPN-ConfigID-r18 ::=            SEQUENCE {

plmn-Identity-r18                PLMN-Identity,

nid-IdentityList-r18             SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16

}

This information is referenced in procedural text e.g. as follows:

1>	If the UE is registered in SNPN:

2>	if the LoggedMeasurementConfiguration message includes snpn-ConfigList:

3>	set the snpn-ConfigIDList in VarLogMeasReport to include the current registered SNPN ID as well as SNPN IDs in snpn-ConfigList;

2>	else:

3>	set the snpn-ConfigIDList in VarLogMeasReport to include the current registered SNPN ID;

---

if the UE is in camped normally state on an NR cell and if the registered SNPN is included in snpn-ConfigIDList stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the current serving cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig;

5>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

NGAP currently contains the information needed to configure the UE.

As observed in R3-240365, SA5 in their CRs, e.g. CR to TS 32.422 (S5-238150), attached to their LS [1], the area scope parameter is enhanced only with the MDT SNPN List and not with SNPN Cell ID List for MDT nor with SNPN TAI List. It is therefore proposed to:

Proposal: Send LS to SA5 cc CT4, RAN2 to inform that SNPN Cell ID List for MDT and SNPN TAI List are needed information for configuration of MDT in SNPNs.

Draft LS is provided in annex of R3-240365 and uploaded to the draft folder (R3-24xxxx).

Please provide your company view on the above proposal:

| Company | Comment |
| --- | --- |
| Nokia | Yes, LS to SA5 is needed. |

4	Conclusion, Recommendations [if needed]

If needed

5	References

[1]		S5-238101 – R3-240030, Reply LS on MDT for NPN, SA5#152