3GPP TSG-RAN WG3 Meeting #123	R3-240875

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	9.1.5.1

Source:	Huawei

Title:	Summary offline discusison for SON for NR-U

Document for:	Discussion

Introduction

This document contains the summary of offline discussion for the following CB:

CB: # SONMDT3_NRU

- Discuss the options above and capture the agreements

(moderator - HW)

Summary of offline discR3-240875.

Proposals for Chair notes

Conclusion 1: A request in UE context setup request is needed to request the DU to report the number of DL LBT failure to the CU.

Conclusion 2: Use a new non-UE associated procedure, namely DU-CU Access and Mobility Indication to transfer the number of DL LBT failure from DU to CU.

Conclusion 3: Add a request in the UE context mod request to cover the intra DU mobility case.

R3-240462, stage 2 CR, agreed.

R3-240463, stage 3 CR, rev in R3-24xxxx, agreed.

Discussion

Issue: which procedure is used to transfer the Nbr of DL LBT failure from DU to CU?

Opt1: Reusing Access and Mobility Indication procedure

Opt2: Introduce a new non-UE associated procedure

Opt3: Reusing UE context setup procedure from DU to CU`

Discussion:

- Do we need a request as in Xn?

Conclusion: a request from CU to DU in UE context setup request is needed.

- Procedure to transfer the metric

Option 3 does not work since the metric is not available yet.

38, 401, 8.2.1.1, setup 4 is before RRC reconfiguration.

A new message is much clear.

- Dedicated message

UE associated vs non-UE associated?

Is it collected in a real time manner?

Non UE associated message with UE IDs similar as XN,

Opt1: Reusing Access and Mobility Indication procedure

Opt2: Introduce a new non-UE associated procedure

DU-CU Access and Mobility Indication procedure?

Do we need to update the existing one?

Option 1: DU-CU Measurement Information Transfer?

Option 2: DU-CU Access and Mobility Indication?

Do we need to transfer this info to source DU from source CU?

No, for now.

A request in UE context modification request at the source side?

For intra DU mobility, it is needed to add a request in the UE Context mod request.

In F1AP, examples for different procedures in DL and UL:

- DU-CU TA INFORMATION TRANSFER/CU-DU TA INFORMATION TRANSFER

- DL RRC Message Transfer/UL RRC Message Transfer

- DU-CU Radio Information Transfer/CU-DU Radio Informatino Transfer

- DU-CU Cell Switch Notification/CU-DU Cell Switch Notification

In NGAP,

- Uplink RAN Status Transfer/ Downlink RAN Status Transfer

- Uplink RAN Early Status Transfer/ Downlink RAN Early Status Transfer

3) Downlink NAS Transport/UL NAS Transport

4) Uplink RAN Configuration Transfer/ Downlink RAN Configuration Transfer

5) Uplink RIM Information Transfer/DL RIM Information Transfer

In XnAP,

None.

In X2AP:

Figure 8.7.1.2-1: eNB Initiated EN-DC X2 Setup, successful operation

Conclusion

Reference