3GPP TSG-RAN WG3 Meeting #123	R3-240297

# Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.415 | CR | 0042 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the PDU Set Information in NG-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, ZTE, Nokia, Nokia Shanghai Bell, Qualcomm Inc., Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following PDU set information specified in NR-U is not aligned with the information specified in RTP Header Extension RTP Header Extension for PDU Set Marking in TS 26.522_V0.3.0; PDU Set Information User Plane protocol in TS 38.415_V18.0.0. End of Data Burst [D] (1 bit); EDBI (End Data Burst indicator) (1 bit) +End of Data Burst (EDB) (3 bits). PDU Set Sequence Number [PSSN] (10 bits); PDU Set Sequence Number (PSSN) (16bits). PDU Sequence Number within a PDU Set [PSN] (6 bits); PDU Sequence Number within a PDU Set (PSN) (8 bits). |
| Summary of change |  | - Remove the EDBI - Update the length of EDB to 1 bit - Update the length of PSSN to 10 bits - Update the length of PSN to 6 bits. - Update the description and frame format accordingly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDU set information in GTP-U header extension is not aligned with RTP header extension. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2, 6.5.2.1, 6.5.3, 6.5.3.6, 6.5.3.8, 6.5.3.10, 6.5.3.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.5.2	Frame format for the PDU Set Information user plane protocol

6.5.2.1	DL PDU SET INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN to receive PDU Set Information and indication of End of Data Burst of a QoS flow.

The following shows the respective DL PDU SET INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | EDB |  | EPDU | PSSI | Spare | 1 |
| Spare |  | QoS Flow Identifier (QFI) |  |  |  |  |  |  | 1 |
| PSSN |  |  |  |  |  |  |  |  | 2 |
| PSSN |  | PSN |  |  |  |  |  |  |  |
| Spare |  |  |  |  | PSI |  |  |  | 1 |
| PSSize |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | Spare |  |  | PSSI | EDBI | 1 |
| Spare |  | QoS Flow Identifier (QFI) |  |  |  |  |  |  | 1 |
| PSSN |  |  |  |  |  |  |  |  | 2 |
| PSN |  |  |  |  |  |  |  |  | 1 |
| EPDU | EDB |  |  |  | PSI |  |  |  | 0 or 1 |
| PSSize |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 6.5.2.1-1: DL PDU SET INFORMATION (PDU Type 0) Format

### 6.5.3	Coding of information elements in frames

#### 6.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the PDU Set UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0= DL PDU SET INFORMATION, 1-15=reserved for future PDU type extensions}.

Field length: 4 bits.

#### 6.5.3.2	Spare

Description: The spare field is set to "0" by the sender and should not be interpreted by the receiver. This field is reserved for later versions.

Value range: (0–2n-1).

Field Length: n bits.

#### 6.5.3.3	QoS Flow Identifier (QFI)

Description: When present this parameter indicates the QoS Flow Identifier of the QoS flow to which the transferred packet belongs.

Value range: {0..26-1}.

Field length: 6 bits.

#### 6.5.3.5	PSSI (PDU Set Size indicator)

Description: This parameter indicates the presence of PDU Set Size (PSSize).

Value range: {0= PSSize not present, 1= PSSize present}.

Field length: 1 bit.

#### 6.5.3.6	Void

#### 6.5.3.7	End PDU of the PDU Set (EPDU)

Description: This parameter indicates whether the current PDU is the last PDU of the PDU set.

Value range: {0= all other PDUs of the PDU Set, 1= last PDU of the PDU set}.

Field length: 1 bit.

#### 6.5.3.8	End of Data Burst (EDB)

Description: This parameter indicates the end of a Data Burst.

Value range: {0 =all other PDUs, 1= the last PDU of a data burst}.

Field length: 1 bit.

#### 6.5.3.9	PDU Set Importance (PSI)

Description: This parameter indicates the importance of the current PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance. PDU Set with the highest importance PDU Set is indicated by 0 and the lowest importance PDU Set is indicated by 15.

Value range: {0..24-1}.

Field length: 4 bits.

#### 6.5.3.10	PDU Set Sequence Number (PSSN)

Description: This parameter indicates the sequence number of the PDU Set to which the current PDU belongs acting as an identifier for the PDU Set.

Value range: {0..210-1}.

Field length: 10 bits.

#### 6.5.3.11	PDU Sequence Number within a PDU Set (PSN)

Description: This parameter indicates the sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

Value range: {0..26-1}.

Field length: 6 bits.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>