3GPP TSG-RAN WG3 Meeting #124	R3-243853

Fukuoka, Japan, 20 – 24 May 2024

Agenda item:	12.3

Source:	Nokia

Title:	[TP for TR 38.799] Evaluation of NR Femto Architecture Options

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

1	Introduction

This TP is the result of the evaluation of architecture options related to NG interface.

2	TP for Evaluation of architecture options

## 5.2.x	Evaluation of Architecture options for the NG interface

Option1: direct connection of NR Femto to 5GC

Pros:

- Already supported by current architecture.

- Less CP latency and no processing delay due to absence of a concentration stage.

- Suitable for certain deployments depending on number of NR Femtos to connect and/or virtualization support of the 5GC.

- Local breakout can be supported.

Cons:

- Not suitable for certain deployments with large number of NR Femtos and/or 5GC not virtualized.

- Not suitable for residential deployments with frequent switch on/off of NR Femtos.

Option 2: NR Femto GW

Pros:

- Only one SCTP association from 5GC to NR Femto GW,so it can support large number of femtos and/or no virtualization of 5GC.

- 5GC is shielded from frequent switch on/off of the NR Femtos.

- Enables operators who have already deployed 4G Femtos using HeNB GW to capitalize on operating model and integration process of 5G Femtos.

- Foreseen specification impacts are already well known from 4G.

- Allows to decouple concentration of CP and concentration of UP: concentration of UP is optional i.e. the NR Femto GW can concentrate CP only while the NR Femto connects directly to the UPF.

- Local breakout can be supported.

Cons:

- Some stage3 specification impact.

- Some processing delay for CP message.

Option 3: SCTP concentrator

Pros:

- Only one SCTP association from 5GC to SCTP concentrator due to using multi-streaming.

- Local breakout can be supported.

- Only stage2 specification impact.

Cons:

- The solution requires consistent configuration and handling of SCTP stream identifiers.

- The solution requires consistent SCTP implementation of AMF, SCTP concentrators and NR femtos.

- Some processing delay for CP message.

- Does not provide an evolution path for operators that have already deployed a HeNB GW for E-UTRAN.

Option 4: NR Femto as a gNB-DU

Pros:

- Reuse existing split gNB architecture.

Cons:

- F1-C was not designed to face frequent switch on/off. Usually F1-C is operated by the network operator and statically configured.

- Foreseen additional Interoperability issue of F1 compared to NG.

- F1-C carried over internet backhaul can lead to latency and reliability issue not meeting the stringent requirement for F1-C interface.

- This option forces the concentration of User Plane and not only control plane i.e. concentration of CP only while NR femto UP connects directly to UPF is not possible.

- Does not provide an evolution path for operators that have already deployed a HeNB GW for E-UTRAN.

- Specification impact for F1 needs to be further assessed.

- Local breakout cannot be supported.