3GPP TSG-RAN WG3 Meeting #124                                                                      R3-243334

Fukuoka, Japan, 20th - 24th May 2024

Source:	CATT

Title:	Discussion on support on-demand SSB SCell operation

Agenda Item:	17.2

Document for:	Discussion and Decision

Introduction

For NES on-demand SSB SCell operation topic, the WID[1] content is as follows:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In previous RAN1 meetings, some agreements are achieved on various aspects related to on-demand SSB SCell. In this contribution, we will discuss the on-demand SSB SCell operation topic, our views will be given as well.

Discussion

RAN1#116 [2] and RAN1#116bis [3] meetings have achieved the following agreements about the cases and scenarios of on-demand SSB SCell operation.

RAN1#116:

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: • Case #1: No always-on SSB on the cell • Case #2: Always-on SSB is periodically transmitted on the cell • FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. • Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) • Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) o This does not preclude SCell for which activation is completed o FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement Support on-demand SSB SCell operation triggered by gNB. FFS Details of associated signaling/indication/configuration provided to UE

RAN1#116bis:

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

RAN3 can consider on the triggering of on-demand SSB operation on gNB side based on RAN1’s agreements. Based on the above Case#1 and Case#2 that RAN1#116 meeting has agreed, we assumed the on-demand SSB SCell is configured by gNB-DU and the trigger/stop of on-demand SSB for the SCell is determined by gNB-DU too. So, there is no impact to RAN3 observed.

In RAN1#116bis meeting, RAN1 further concluded different scenarios for Case#1 and Case#2, which capture the specific conditions for gNB triggering the on-demand SSB. Also, RAN1 agreed that timing for on-demand SSB transmission (when the triggered SSB starts and ends) will be separately discussed. However, it can be observed the solutions for gNB triggering on-demand SSB in different scenarios of Case#1 and Case#2 are still under control of gNB-DU, so there is still no impact to RAN3 at present. It is proposed RAN3 continue monitoring progress of other working groups.

Proposal 1: As there is still no impact to RAN3 at present, RAN continue monitoring progress of other working groups.

Conclusion

According to the analysis in section 2, we have following proposal:

Proposal 1: As there is still no impact to RAN3 at present, RAN continue monitoring progress of other working groups.

Reference

- RP-234065, New WID: Enhancements of network energy savings for NR, RAN#103, Ericsson

- Draft Report of 3GPP TSG RAN WG1 #116 v0.3.0, RAN1#116, MCC

- Draft Report of 3GPP TSG RAN WG1 #116bis v0.3.0, RAN1#116bis, MCC