3GPP TSG-RAN WG3 Meeting #125	R3-244686

Maastricht, NL, 19 - 23 August, 2024

Agenda Item:	9.2

Source:	Huawei (moderator)

Title:	SoD of CB: # 12_DLPSI

Document for:	Approval

Introduction

This contribution provides the summary of offline discussion on the following CB:

CB: # 12_DLPSI

-  Try to figure out the solution and whether to solve this issue in R18 or R19?

- Check R3-244583 together

(moderator - HW)

Summary of offline disc R3-244686

For the Chairman’s Notes

DL PSI based discard

RAN3 to adopts option 1: Introducing specific notification in F1AP and suggestion mechanism in F1-U for DL PSI based discard. The change to F1AP and F1-U is from Rel-19. Revise the CR R3-244519 and R3-244520 into R19 CR/TP.

CRs

Agree the revision of R3-244583 in R3-244xxx, to only reflect the following:

- remove “and the value is set as "start" ” from clause 8.3.1 and clause 8.3.4.

Discussion

DL PSI based discard

Issue 1: Which option to be adopted to support DL PSI based discard?

Option 1: Introducing specific notification in F1AP and suggestion mechanism in F1-U for DL PSI based discard

Option 2: Using the Desired buffer size for the data radio bearer and/or the Desired Data Rate contained in the DL DATA DELEVERY STATUS frame

Option 3: Using the DL Congestion Information contained in the ASSISTANCE INFORMATION DATA frame.

Option 4: option 1 but since R19

Summary: Based on further offline discussion, it is suggested RAN3 to adopts option 1. Few companies preferred from Rel-18. But to make progress, Rel-19 is also OK.

Revision for CR R3-244583

Option 1:

- 8.3.1	UE Context Setup

------------------unchanged parts are skipped----------------

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to start performing UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

------------------unchanged parts are skipped----------------

- 8.3.4	UE Context Modification (gNB-CU initiated)

------------------unchanged parts are skipped----------------

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to start performing UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16]. If the PSI based SDU Discard UL IE is included in the DRB To Be Modified List IE and the value is set as "stop", the gNB-DU shall, if supported, take it into account to stop performing UL PSI based SDU discarding activation and deactivation for the indicated DRB.

Option 2:

- 8.3.1	UE Context Setup

------------------unchanged parts are skipped----------------

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

------------------unchanged parts are skipped----------------

- 8.3.4	UE Context Modification (gNB-CU initiated)

------------------unchanged parts are skipped----------------

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE, the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

The tabular below is pasted for information, to show the clear semantics description of the IE.

| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Summary: We go for option 2. CMCC please update the CR accordingly.

References