3GPP TSG-RAN WG3 #125                                                 	R3-244699

Maastricht, NL, 19th – 23th Aug, 2024

Agenda Item:	11.4

Source:	Ericsson (Moderator)

Title:	SoD on CB: # AIRAN3_EC

Document for:	Approval

# 1	Introduction

This SoD is to discuss the following CB:

CB: # AIRAN3_EC

- Get convergence on the options above on finer granularity of EC

- Draw the conclusion

(moderator - E///)

Summary of offline disc R3-244699

# 1	Summary for chairman notes

- RAN3 agrees that enhancements to the node level EC granularity to a scope finer than a whole gNB could be beneficial, assuming that such enhancements enable network level energy saving.

- There is no consensus on how to move forward with a potential solution for the issue

It is proposed to ask online whether, on the basis of the above, RAN3 should treat the topic during normative work.

A TP based on the above will be provided

# 2	Discussion

## 2.1 	Energy Cost Granularity Enhancements

### 2.2.1 Measured Energy Cost

During online discussions the topic of enhanced granularity for the EC was discussed. Legacy Energy Cost represents a measured energy consumption for an entire gNB. The intention in the AI/ML for NG-RAN SI is to explore ways to improve the granularity of the per gNB EC.

In order to guide the discussion and reach progress, a number of questions are formulated below.

- Does RAN3 agree that enhancing the granularity of the Energy Cost to something finer than a whole gNB would benefit the Rel18 AI/ML based energy saving solution?

Discussion outcome:

RAN3 agrees that enhancements to the node level EC granularity to a scope finer than a whole gNB could be beneficial, assuming that such enhancements enable effective energy saving actions.

- Should any new metric introduced over F1/Xn to improve the EC granularity for Rel19 consist of measurements or could it consist also of estimations?

Discussion outcome:

Nokia: there could be benefits to consider estimations of finer granularity over measurements that provide course granularity.

Huawei: Solutions should be based on measurements

BT: Do we need to go into the details of whether the finer granularity EC is measured or estimated? Wouldn´t this only be about providing sufficiently good accuracy?

E///: how to we regulate how accuracy is determined?

DT: training for an AI/ML model should be based on measurements.

CATT: In Rel17 we decided that the EC is per node, not per cell. It is not feasible to measure a per cell level EC.

Lenovo: how can we measure today the amount of energy saved with cell switch off?

DT: We have per node measurements, revealing the delta in energy consumption

TIM: The main point is whether inter vendor estimations are trustable.

Nokia: a good estimation on cell level is better than an irrelevant measurement such as a per node EC

Huawei: How can we derive that the estimation is good?

A finer granularity EC should be based on measurements. Whether an estimation can be considered to be a measurement is FFS.

- At RAN3-125 the following options were proposed to improve the EC granularity:

  - Measured EC per group of Cells

  - EC per Cell

  - EC per one or more HO event

Which of the option above reflects the conclusion of the discussion above and could be proposed for further normative work in Rel19?

Discussion outcome: